3GPP TSG-SA WG2#161	S2-2402793

Athens, Greece, 26 February-1 March 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Key issue #1- Updates to solution #1

Document for:	Discussion/Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_Ph3/Rel.19

Abstract of the contribution: Closes editor’s notes in solution #1.

# 1.	Discussion

Some clarifications are being proposed for this solution:

- Configuration of TAIs and cell-ids on the eNB/gNB on board is using existing methods

- eNB/gNB on board needs to continue to send cell-ids and TAIs where the UE is geographically located as in the current “bent pipe” architecture defined in clause 5.4.11.7 of TS 23.501.

- there is no existing procedure in TS 38.413/ TS 36.413 to release eNB/gNB in CN. It is though possible to  perform NG/S1 RAN configuration update and therefore an existing RAN3 procedure defined in TS 38.413 cl. 8.7.2.2 can be re-used. Eventually the decision whether to define a new procedure or re-use the existing procedure is up to RAN3

- Global RAN Node Id is part of the NG setup procedure and can be re-used. In the view of the authors the Global RAN Node Id does not need to correspond to the geographical area, because the additional information in the ULI will include the cell-id and TAI of where the UE is geographically located

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-029:

# >>>>BEGINNING OF CHANGES<<<<

## 6.1	Solution #1: N2/S1 disconnection for satellites leaving an area served by a CN

### 6.1.1	Description

This is a solution addressing Key Issue #1.

The solution assumes that an eNB/gNB is on-board the satellite, and the EPC/5GC is on ground.

When a LEO satellite with an on-board eNB/gNB leaves an area served by a certain core network, there is no use of the N2/S1 interface anymore. It may even be that the satellite has no longer any ground connectivity towards that specific 5GC. To avoid errors in AMF/MME due to stale N2/S1 connections, e.g. AMF/MME sending paging requests or AMF configuration updates to a unavailable gNB/eNB, a graceful removal or update of the N2/S1 interface connection is proposed in this solution.

The TAC and cell ID broadcasted by the eNBs/gNBs are configured as specified in TS 38.300 [x] and TS 36.300 [13] with mechanisms out of scope of SA2.

The gNB/eNB on board will also provide NR TAI Information in User Location Information, in N2/S1 messages as in existing procedures for the “bent pipe” architecture e.g. as defined in TS 23.501 cl. 5.4.11.7. The information in the ULI representing, if known, also the cell-id and TAI where it is the “served” UE is geographically located e.g. it can be derived from the Coarse Location Info that is signalled in RRC for NR and WB-E-UTRAN. The AMF/MME may consider the NR TAI Information when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions) and when the AMF/MME provides user location information to other NFs (e.g. LMF).

This solution is based on the following basic principles and assumptions:

-	Possible enhancements to the N2 and S1 procedures so that an eNB/gNB on-board a satellite can disconnect or perform RAN configuration update for the N2/S1 interface when the eNB/gNB leaves an area served the core network (CN).

-	If/when the satellite returns to the area served by the CN, a new N2/S1 interface setup or config update is initiated based on the existing procedure. In this procedure as defined in TS 38.413 [12] the Global RAN Node Id of the RAN node is included and does not need to be geographically specific.

NOTE:	The protocol aspects and whether changes are needed in related procedures of the NGAP/S1AP is to be determined by RAN3. The procedures, if agreed for normative work, would be documented in TS 36.413 [14] and TS 38.413 [12]. This solution only shows an example of how it could be done.

### 6.1.2	Procedures

Figure 6.1.2-1: Release or perform Configuration Update of N2/S1 interface

1. 	The gNB/eNB knows, based on its ephemeris and configuration, that it is about to stop serving a certain 5GC/EPC area and it is about to lose the connectivity to the 5GC/EPC via a ground connection.

Before triggering a disconnect or RAN Configuration Update of the N2/S1 interface, it is assumed that the gNB/eNB has e.g. triggered a handover (or a release in case of NB-IoT) to move the connected mode UEs to another gNB/eNB that is serving the area. How this is done is based on pre-rel-19 procedures and not impacted by this solution.

2. 	The gNB/eNB sends a N2/S1 Disconnect Request or RAN Configuration Update for all active N2/S1 connections to the 5GC/EPC.

3. 	The AMF/MME replies with an acknowledgement.

4. 	The gNB/eNB releases any UE context associated to the N2/S1 connections and removes the N2/S1 connection. The eNB/gNB is reconfigured to broadcast a different cell-id/TAC and therefore the UEs in idle mode will perform Registration Req./TAU.

5. 	The AMF/MME releases any associated UE to CM-IDLE state.

Figure 6.1.2-2: Setup of N2/S1 interface

1. 	The gNB/eNB knows, based on its ephemeris and configuration, that it starts serving a certain AMF/MME.

2. 	The gNB/eNB sends a N2/S1 SETUP REQUEST or config update to the AMF/MME, based on existing N2/S1 procedures.

3. 	The AMF/MME provides a reply.

### 6.1.3	Impacts to Services, Entities and Interfaces

NG-RAN and E-UTRAN:

-	Support for detecting when satellite is about to stop serving (and/or lose ground connectivity to) a given AMF/MME and trigger N2/S1 disconnect or use the existing RAN Configuration Update to release or update the N2/S1 connection. Whether a new procedure is needed will be decided by RAN3.

MME/AMF:

-	Support for N2/S1 procedure to release N2/S1 connection and delete corresponding context. Whether it is needed will be decided by RAN3

# >>>>END OF CHANGES<<<<