SA WG2 Meeting #161	S2-240

# Athens, February 26th –March 1st , 2024        	  					       (revision of S2-240751)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Edge Computing for EPC Interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_EC4EPCIWK |  |  |  |  |  | Date: |  |  | 2024-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current §E.4 assumes that traffic offload via UL CL/BP is not supported over EPC, while TEI19_EC4EPCIWK removes this limitation. TEI19_EC4EPCIWK Enables the PGW-C to insert an UL Classifer (UL/CL) and L-PSA close to or co-located with the SGW-U, and to change the UL/CL/ L-PSA when the SGW-U changes (e.g. as a result of UE mobility), such as to be able to offload selective traffic. This includes: - enhancing the S5 GTP-C procedures to support enabling the SGW-C to update the SMF+PGW-C with the SGW-U node name of the (new) SGW-U selected for the PDN connection. - enable the SMF+PGW-C to discover and select an UL/CL (a and L-PSA) co-located with the SGW-U. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indicate that the behavior described in § E.4 applies only when traffic offload via UL CL/BP is not supported over EPC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incoherence between 23.548 and other specifications updated by TEI19_EC4EPCIWK |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.4; E.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# E.4	Session Breakout

When traffic offload via UL CL/BP is not supported over EPC, and a PDN connection is initiated in EPS or a PDU Session is handed-over to EPS, the SMF+PGW-C can send to the EASDF DNS message handling rules requesting the EASDF to transparently forward any DNS traffic. The SMF+PGW-C can send to the EASDF new DNS message handling rules (with actual reporting and actions as defined in clause 6.2.3.2.2) when the PDU Session/PDN connection is handed-over (back) to 5GS.

When traffic offload via UL CL/BP is not supported over EPC, and a PDN connection is initiated in EPS, the SMF+PGW-C can also select a normal DNS Server (i.e. different from an EASDF) to serve the PDN Connection, and then indicate to the UE to use the EASDF as DNS Server when the PDU Session/PDN connection is moved to 5GS.

NEXT CHANGE (2)

# E.1	General

5GC is specified to support interworking with EPC. Edge Computing deployments that use interworking need to consider the aspects outlined in this Annex.

How to support Edge Computing in EPC is specified in TS 23.502 [3] clause 4.11.0a.

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES