3GPP TSG-WG SA2 Meeting #162	S2-2404068

# Changsha, China, April 15th – April 19th, 2024	(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1079 | rev |  | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ML Model Provisioning with Multiple Analytics IDs and Filters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current ML Model Provisioning request, the ML Model consumer can include multiple Analytics IDs in the request, as well as ML model filters and other ML model requirements (e.g., Target of ML Model Reporting). However, it is not clear what is the relationship between the Analytics IDs and the ML model filters or other ML Model requirements. Will the ML Model requirements apply to all the Analytics IDs in the request? Or there should be ML Model requirements for each Analytics ID in the request? Furthermore, is there a requirement that the multiple Analytics IDs shall be different from each other? For example, suppose the ML Model consumer wants to get an ML model of UE communication Analytics ID for UE1, and it also wants to get an ML model of UE communication Analytics ID for UE2, it is not clear how it is supported in the current standard, or it cannot be supported. Based on the argument above, we propose to clarify the parameters in the ML Model Provisioning request. All the parameters about the ML Model filters/requirements are per Analytics ID. If there are multiple Analytics IDs in the request, the analytics IDs do not need to be different from each other. Another correction is ML Model Reporting Information is only applicable to Nnwdaf_MLModelProvision_Subscribe service, but it exists in the Nnwdaf_MLModelInfo service definition. Therefore, ML Model Reporting Information should be removed from the Nnwdaf_MLModelInfo service definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that all the parameters about the ML Model filters/requirements are per Analytics ID. If there are multiple Analytics IDs in the request, the analytics IDs do not need to be different from each other. Adding ML Model filter and Target of ML Model reporting per Analytics ID in the ML Model provisioning notification message. ML Model Reporting Information is removed from the Nnwdaf_MLModelInfo service definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what is the relationship between the Analytics IDs and the ML model filters or other ML Model requirements. ML Model Provisioning may not be implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2A.2, 6.2A.3, 7.5.2, 7.5.4, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e. an NWDAF containing AnLF to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF, to be notified when ML Model Information related  to the ML Model subscription becomes available, using Nnwdaf_MLModelProvision services as defined in clause 7.5. The ML Model Information is used by an NWDAF containing AnLF to derive analytics. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer (i.e. an NWDAF containing AnLF) subscribes to, modifies, or cancels subscription for a (set of) trained ML Model(s) associated with a (set of) Analytics ID(s) and other parameters by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2.

When a ML Model subscription is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the subscription; or

-	determine whether triggering further training for existing trained ML models is needed for the subscription.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML model.

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) trained ML model(s) associated to a (set of) Analytics ID(s) and other parameters, the NWDAF containing MTLF notifies the NWDAF service consumer with the trained ML Model Information (containing a (set of) file address of the trained ML model) and (optionally) other associated parameters by invoking Nnwdaf_MLModelProvision_Notify service operation. The content of trained ML Model Information and other associated parameters that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify an available re-trained ML model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

When step 1 is for a subscription modification (i.e. including Subscription Correlation ID), the NWDAF containing MTLF may provide either a new trained ML model different to the previously provided one, or a re-trained ML model by invoking Nnwdaf_MLModelProvision_Notify service operation.

### 6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an NWDAF containing AnLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

For each Analytics ID, the following parameters may be provided:

-	[OPTIONAL] ML Model Filter Information: enables to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML model is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

The NWDAF containing MTLF provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID:

-	ML Model Information, which includes the ML model file address (e.g. URL or FQDN) for the Analytics ID(s).

-	[OPTIONAL] ML Model Filter Information and Target of ML Model Reporting may be also provided.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

### 6.2A.3	ML Model request

The procedure in Figure 6.2A.3-1 is used by an NWDAF service consumer, i.e. an NWDAF containing AnLF to request and get from another NWDAF, i.e. an NWDAF containing MTLF ML Model Information, using Nnwdaf_MLModelInfo services as defined in clause 7.6. The ML Model is used by an NWDAF containing AnLF to derive analytics. An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.3-1: ML model Request

1.	The NWDAF service consumer (i.e. an NWDAF containing AnLF) requests a (set of) ML Model(s) associated with a (set of) Analytics ID(s) and other parameters by invoking Nnwdaf_MLModelInfo_Request service operation. The parameters that can be provided by the NWDAF Service Consumer are listed in clause 6.2A.2.

When an ML Model Information request is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the request; or

-	determine whether triggering further training for existing trained ML models is needed for the request.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML model.

2.	The NWDAF containing MTLF responds with the ML Model Information (containing a (set of) file address of the trained ML model) and (optionally) other associated parameters to the NWDAF service consumer by invoking Nnwdaf_MLModelInfo_Request response service operation. The content of ML Model Information and other associated parameters that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

# * * * * Second change * * * *

### 7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe.

Description: Subscribes to NWDAF ML model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription), (set of) tuple(s) (Analytics ID, ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), ML Model Target Period), ML Model Reporting Information, Expiry time.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

# * * * * Third change * * * *

### 7.5.4	Nnwdaf_MLModelProvision_Notify service operation

Service operation name: Nnwdaf_MLModelProvision_Notify.

Description: NWDAF notifies the ML model information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Set of the tuple (Analytics ID, address (e.g. URL or FQDN) of Model file, and other ML Model related information as defined in Clause 6.2A.2), Notification Correlation Information.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Fourth change * * * *

### 7.6.2	Nnwdaf_MLModelInfo_Request service operation

Service operation name: Nnwdaf_MLModelInfo_Request

Description: The consumer requests NWDAF ML Model Information.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2.

Inputs, Optional: (Set of) tuple(s) (Analytics ID, ML Model Filter Information to indicate the conditions for which ML model for the analytics is requested and Target of ML Model Reporting to indicate the object(s) for which ML model is requested (e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs)), ML Model Target Period).

Outputs, Required: Set of the tuple (Analytics ID, address (e.g. URL or FQDN) of Model file, and other optional ML Model related information as defined in Clause 6.2A.2).

Outputs, Optional: None.

# * * * * End of changes * * * *