3GPP TSG-WG SA2 Meeting #162	S2-2404100

Changsha, China, April 15th – 19th, 2024

Source:	Huawei, HiSilicon

Title:	KI#1:Evaluation

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2/ Rel-19

Abstract: Evaluation and Conclusion for KI#1: Architectural enhancements for the support of a MWAB.

1. Discussion

This contribution proposes the evaluation for KI#1.

| Key issue | Solutions in TR 23.700-06 V0.2.0 |
| --- | --- |
| #1:Architectural enhancements for the support of a MWAB | Solution #1: Architecture enhancements to support MWAB operations Solution #2: MWAB architecture and procedures Solution #3: N3 backhaul PDU session management |

Depending on the different usages of PDU sessions of the MWAB-UE, 4 categories are identified among these 3 solutions:

- Category#1: Connection with OAM server over BH PDU session

- Category#2: N2 over BH PDU session

- Category#3: N3 over BH PDU session

- Category#4: Xn over BH PDU session

Considering whether or not to support Xn connection over BH PDU Session is still under discussion, this pCR focuses on the evaluation on the first three categories.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-06.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.X  Evaluation for KI#1

7.X.1 Evaluation for architectural

Solution #1 describes different architecture options for the MWAB operation including non-roaming and roaming of MWAB-UE as listed in following:

- In case of non-roaming, the Serving PLMN of MWAB-UE (e.g. PLMN 1) can be same or different from the PLMN served by the MWAB-gNB (e.g. PLMN 1 or PLMN 2). The HPLMN of the MWAB-UE is the PLMN 1.

- In case of roaming, the Serving PLMN of MWAB-UE is the PLMN 1. The PLMN served by the MWAB-gNB is PLMN 2. The HPLMN of the MWAB-UE is the PLMN 3.

  - The BH PDU Session can be Local Breakout or Home Routed.

- The UE served by the MWAB-gNB is not aware of the Serving PLMN of MWAB-UE.

7.X.2 Evaluation for BH PDU session usage

From all 3 solutions proposed for KI#1, three categories are identified for architectural enhancements for the support of a MWAB as described below.

- Category#1: Connection with OAM server over BH PDU session.

| Solutions | High-level Descriptions | How to trigger the PDU session to connect with the OAM server? |
| --- | --- | --- |
| Solution #1 | The MWAB-gNB establishes the connection to the OAM system and obtains the corresponding configurations to operate as a gNB. | The details are not clear. |
| Solution #2 | the MWAB-UE establishes a dedicated PDU session for the OAM traffic. The MWAB-UE is configured dedicated DNN/S-NSSAI for the PDU session for backhaul link to the OAM server, or the AMF provides it when the MWAB-UE attempts to establish the PDU session in the slice where the OAM service is (e.g a default DNN/S-NSSAI can be used). | By configuration |

- Category#2: N2 over BH PDU session.

| Solutions | High-level Descriptions | How to trigger the BH PDU session to connect with the AMF over N2? | How to discover and determine the AMF for the MWAB-gNB to connect to |
| --- | --- | --- | --- |
| Solution #1 | The MWAB-gNB may instructed by the OAM system to establish N2 interface using the NG setup procedure defined in TS 38.413 with some AMFs over the PDU session provided by MWAB-UE | Triggered by the OAM | Based on the configuration by OAM |
| Solution #2 | The BH PDU session’s PSA routes the N2 message between the MWAB-gNB and AMF based on the IP address. The MWAB-UE is configured with dedicated DNN/S-NSSAI for the PDU session for backhaul link to be used by the MWAB-gNB. When MWAB-UE establishes the PDU session to access the OAM server, the address of the AMF(s) for the MWAB-gNB to connect to can be configured by the OAM based on MWAB’s location. | Triggered by the OAM | Based on the configuration by OAM |
| Solution #3 | MWAB provides wireless connection to the 5GC through an IP connectivity provided by a PDU session; hence, both N2 interface and N3 interface are carried over the PDU session(s) between NWAB-UE and 5GC. At least one PDU session is established between NWAB-UE and 5GC for both N2 interface and N3 interface depending on the configuration of the MWAB: Single BH PDU session for both N2 and N3. Multiple BH PDU session(s) for N2 and PDU session(s) for N3. | Not clear | Not clear |

Mobility aspect for N2 over BH PDU session will be evaluated in Key issue#4.

- Category#3: N3 over BH PDU session.

| Solutions | High-level Descriptions | How to trigger the BH PDU session to connect with the UPF over N3? |
| --- | --- | --- |
| Solution #1 | When the UE establishes a PDU session, the UE SMF selects a proper UE UPF. The MWAB-gNB may establish the N3 interface with the UPF over the PDU session of the MWAB-UE, if it is not yet established | Triggered by the PDU session establishment of UE when MWAB-gNB receives the UPF address of N3 |
| Solution #2 | The MWAB stores the association between N3 tunnel information of UE’s PDU session and PDU Session ID of BH PDU Session. The IP address for the BH PDU Session allocated for MWAB-UE is included in the N3 AN tunnel corresponding to the UE’s PDU Session and provided by the MWAB-gNB to the UE’s SMF. The BH PDU Session Modification for the existing N3 connection is supported. | Triggered by the PDU session establishment of UE when MWAB-gNB receives the CN tunnel information of N3 UPF. |
| Solution #3 | The MWAB binds the UE QoS flow to BH PDU session based on the UE requested PDU session and the QoS flow characteristics. The MWAB-gNB triggers MWAB-UE to establish/modify the BH PDU session | The BH PDU session can be established or modified triggered by the MWAB-gNB. |

# * * * * End of changes * * * *