3GPP TSG-WG SA2 Meeting #162	S2-2404149

Changsha, China, April 15th – 19th, 2024	(revision of S2-231xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2 and KI#4: Update on Solution #9: PDU Set information identification for encrypted traffic

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2/ Rel-19

Abstract: It proposes update on solution#9 to remove ENs.

1. Introduction/Discussion

In solution#9, the following ENs are added:

Editor's note: It is FFS if all tracks are mapped into a single QoS Flow, or tracks may be mapped to separate QoS Flows. In the former case, it is FFS how the QoS requirements can be differentiated per track in N3 and in NG-RAN.

Editor's Note: It is FFS if and how the current UPF discovery and selection in SMF is impacted.

Editor's Note: It is FFS how the MoQ ANNOUNCE messages from the app servers are handled in the UPF.

For the first EN, track corresponds to a sub-stream of traffic, e.g. a video track corresponds to a video stream and an audio track corresponds to an audio track, it proposes the mapping of track to QoS flow following the existing QoS flow mapping principle, i.e. tracks with the same QoS requirement are mapped into the same QoS flow, and tracks with different QoS requirements are mapped into different QoS flow.

For the second EN, it assumes SMF knows which UPF supports MoQ functionality, and there could be different ways for SMF to select the UPF, e.g. based on (DNN, S-NSSAI) or other service related information (e.g. in case EASDF-based EAS discovery, as defined in TS23.548, is deployed, the FQDN in DNS query can be used for UPF discovery).

For the third EN, MoQ ANNOUNCE message are handled by MoQ relay at UPF following MoQ Transport protocol, it proposes remove it directly.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70-030.

# * * * * First change * * * *

6.9	Solution #9: PDU Set information identification for encrypted traffic

6.9.1	Key Issue mapping

This solution applies to Key Issue #2: Support PDU Set information identification for end-to-end encrypted XRM traffic and Key Issue #4: Traffic detection and QoS flow mapping for multiplexed data flows.

6.9.2	Description

Media over QUIC (MoQ) is a simple low-latency media delivery solution for interactive media, live media and hybrid interactive and live media, the media will be mapped onto underlying QUIC mechanisms (QUIC streams and/or QUIC datagrams) and can be used over raw QUIC or WebTransport.

The basic data element of Media over QUIC Transport [9] is an object. An object is an addressable unit whose payload is a sequence of bytes. Objects are comprised of two parts: metadata and a payload. The metadata is hop-by-hop encrypted and is always visible to relays. And because the QUIC connection to the relay is encrypted, thus the metadata is not visible to routers, the base station, etc. The payload portion can be end-to-end encrypted, in which case it is only visible to the producer and consumer. The definition of OBJECT message [9] is as follows:

OBJECT Message {

Track ID (i),

Group Sequence (i),

Object Sequence (i),

Object Send Order (i),

[Object Payload Length (i),]

Object Payload (b),

}

* Track ID: The track identifier which identifies a sub-stream of an application service.

* Group Sequence: The object is a member of the indicated group within the track.

* Object Sequence: The order of the object within the group.

* Object Send Order: An integer indicating the object send order or priority value.

* Object Payload Length: The length of the following Object Payload. If this field is absent, the object payload continues to the end of the stream.

* Object Payload: An opaque payload intended for the consumer which can be end-to-end encrypted and SHOULD NOT be processed by a relay.

For Media over QUIC, metadata is provided to relay node, and the relay node can make the forwarding decision based on metadata.

This solution proposes that the PSA UPF act as MoQ Relay to identify PDU Set Information and demultiplex sub-flows based on metadata provided as part of Object of MoQ.

6.9.3	Procedures

For the XR traffic between UE and Application Server, the UPF of the PDU session acts as MoQ Relay and identifies PDU Set Information and demultiplex sub-flows based on metadata of MoQ. There will be a MoQ Transport connection between UE and MoQ Relay, also a MoQ Transport connection between the MoQ Relay and Application server. During PDU session Establishment procedure, SMF selects UPF supporting MoQ relay functionality based on UPF capability and other information (e.g. specific (DNN, S-NSSAI) or in case of EASDF-based EAS discovery as defined in TS23.548[x] is deployed, SMF may select UPF supporting MoQ relay functionality for specific FQDN of XR service received from EASDF.

For XR traffic, each PDU Set (e.g. frame) could be mapped to an Object of MoQ, and the PDU Set Information could be determined as following:

-	PDU Set Sequence Number: can be mapped from Object Sequence.

-	Indication of End PDU of the PDU Set: can be determined based on Object Sequence and Object Payload Length or determined based on the end of stream indication in case each Object is mapped into a QUIC stream.

-	PDU Sequence Number within a PDU Set: can be determined based on packets it receives for an Object.

-	PDU Set Size in bytes: can be determined based on Object Payload Length.

-	PDU Set Importance: can be determined based on Object Send Order.

The Track ID in the metadata of MoQT (MoQ Transport) is an identifier of the sub media flow, which could be used to differentiate the sub-flows within one transport connection. By acting as MoQ relay, the UPF identifies sub-flow based on Track id and maps the identified sub-flow into QoS flow. The mapping of track to QoS flow following the existing QoS flow mapping principle, i.e. only tracks with the same QoS requirement are mapped into the same QoS flow, and tracks with different QoS requirements are mapped into different QoS flow.

The Track ID may be provided by AF as part of Flow description information of Nnef_AFsessionWithQoS Create/Update as described in procedures defined in clause 4.15.6.6 and clause 4.15.6.6a TS 23.502 [3] or the mapping between Track ID and QoS requirements can be configured within 5GC (e.g. PCF).

The MoQ Relay functionality can be discovered by UE via application layer methods, e.g. in DNS procedure an anycast address related with PSA UPF can be returned to UE or UE can be redirected by app server to MoQ relay via migration mechanism defined in MoQT[9].

The MoQ Relay could discover the App server based on the URL related with the track name requested by UE, which is defined in MoQT [9].

In case SSC mode2 or SSC mode 3, when the PSA UPF is reselected, it assumes the MoQ Transport connection between UE and MoQ relay on UPF and the connection between the MoQ relay and App server will be re-established.

6.9.4	Impacts on services, entities and interfaces

UPF:

-	Implements the MoQ Relay functionality. Identifies PDU Set Information and demultiplex sub-flows based on received metadata of MoQ.

SMF:

-	Updates UPF with N4 rule, which includes the Track id in the PDR.

PCF:

-	Includes the Track id as part of Service data flow detection of PCC rule.

AF:

-	Provides traffic description and corresponding QoS requirements for MoQ track in step 1 of the procedure defined in clause 4.15.6.6 of TS 23.502 [3].

UE:

-	Support MoQ transport protocol in user space.

# * * * * Second change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[12]	TR 26.926 v18.1.0: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[13]	TR 26.925 v18.1.0: "Typical traffic characteristics of media services on 3GPP networks".

[14]	IETF RFC 9330: "Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture.

[15]	IETF RFC 9331: "The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[16]	IETF draft-ietf-tsvwg-ecn-encap-guidelines-22: "Guidelines for Adding Congestion Notification to Protocols that Encapsulate IP".

[17]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[18]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[19]	IETF RFC 9332: " Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[20]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[21]	IETF draft-ietf-tsvwg-udp-options: "Transport options for UDP".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	IETF RFC 6364: "Session Description Protocol Elements for the Forward Error Correction (FEC) Framework".

[24]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME".

[25]	IETF RFC 6682: "RTP Payload Format for Raptor Forward Error Correction (FEC) ".

[26]	IETF RFC 6695: "Methods to Convey Forward Error Correction (FEC) Framework Configuration Information".

[27]	IETF RFC 6816: "Simple Low-Density Parity Check (LDPC) Staircase Forward Error Correction (FEC) Scheme for FECFRAME".

[28]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME".

[29]	IETF RFC 8680: "Forward Error Correction (FEC) Framework Extension to Sliding Window Codes".

[30]	IETF RFC 8681: "Sliding Window Random Linear Code (RLC) Forward Erasure Correction (FEC) Schemes for FECFRAME".

[31]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[32]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[33]	IETF RFC 7656: "A Taxonomy of Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources".

[34]	S2-2210181: "Reply LS on further details on XR traffic".

[35]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[36]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[37]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[38]	IETF RFC 9298: "Proxying UDP in HTTP".

[39]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[40]	IETF draft-ietf-masque-quic-proxy-01: "QUIC-Aware Proxying Using HTTP".

[41]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[42]	IETF RFC 5764:" Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[43]	IETF RFC 7983: "Multiplexing Scheme Updates for Secure Real-time Transport Protocol (SRTP) Extension for Datagram Transport Layer Security (DTLS)".

[44]	IETF RFC 8872: "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams".

[45]	IETF RFC 9443: "Multiplexing Scheme Updates for QUIC".

[46]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[x]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

# * * * * End of changes * * * *