SA WG2 Meeting #162	S2-2404741

# 15 April – 19 April 2024, Changsha, China

Source:	Lenovo

Title:	New solution for KI#1: USS influencing USS change via NEF service

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution:

# 1		Discussion

This solution addresses the objectives of Key Issue 1 and in particular the scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

The procedure takes into account the following assumptions:

-	A USS is aware of multiple USS provider serving a geographical area where each USS serving area is mapped to a specific DNAI. Such assumption has been also captured in 3GPP TS 23.255 clause 7.6.

The USS initiates an AF Traffic Influence request including a (list of) DNAIs and associated Routing Profile ID(s). When the SMF determines that DNAI change needs to take plae the SMF notifies the USS of the pending UP path change that triggers the USS to move the session to the target USS.

Further details for the solution are provided below:

# 2		Proposal

The following key issue is proposed.

******************************** First change  *******************************

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> |
| #1 | x |  |  |
| #2 | x |  |  |
| #3 | x |  |  |
| #4 |  | x |  |
| #5 |  | x |  |
| #6 |  | x |  |
| #7 |  |  | x |
| #8 |  |  | x |
| #9 |  |  | x |
| #x | x |  |  |

******************************** Second change (all new text)  *******************************

## 6.X	Solution #X: USS influencing USS change via NEF service

### 6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

This solution addresses the objectives of Key Issue 1 and in particular the scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

### 6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

.

The procedure takes into account the following assumptions:

-	A USS is aware of multiple USS provider serving a geographical area where each USS serving area is mapped to a specific DNAI. Such assumption has been also captured in 3GPP TS 23.255 clause 7.6.

The USS initiates an AF Traffic Influence request including a (list of) DNAIs and associated Routing Profile ID(s). When the SMF determines that DNAI change needs to take place the SMF notifies the USS of the pending UP path change that triggers the USS to move the session to the target USS.

.

### 6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

The procedure is described below:

Figure 6.x.3-1 : Multi-USS support during UAV flight path

1.-	USS initiates an AF Traffic Influence request which may include a list of DNAI(s) and associated Routing Profile ID(s)

2.	If DNAI(s) are not included in step 1, the NEF/UAS NF based on the knowledge of AF Service Identifier to DNAI may add DNAI information in the request.

3.	When the SMF determines a UP path change is needed (based on DNAIs) the SMF informs the USS of the pending UP path change as per 3GPP TS 23.502 [x] clause 4.3.6.3.

4.	The the session is relocated to USS#2. USS#2 may trigger UAV re-authorisation as per 3GPP TS 23.256

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing and/or new 3GPP nodes and functional elements.

No impacts

******************************** End of change *******************************