SA WG2 Meeting #162	S2-2404752

15 – 19 April, 2024, Changsha, China                               (revision of S2-24xxxxx)

Source: 	vivo

Title: 	KI#4: New Solution – SBA signalling storm mitigation and prevention based on NWDAF detection and prediction

Document for: 	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes a solution for KI#4 to support Signalling storm into core network mitigation and prevention based on NWDAF detection and prediction.

1	Discussion

Network function(s) may crash due to e.g. massive UE access and unexpected misconfiguration or failure of IP routers, which may consequently generate a significant number of error and redundant reports, leading to a signalling storm.

Thus, this contribution proposes a solution to detect, predict, mitigate and prevent the SBA signalling storm.

2. Proposal

It is proposed to add the following contents to TR 23.700-84.

Start of Change

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  |  |  | Use cases (optional) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <use case #3> | <use case #6> |
| 6.X |  |  |  | X | X | X |

Next Change (all new text)

6.X	Solution #X: SBA signalling storm mitigation and prevention based on NWDAF detection and prediction

6.X.1	Description

This solution is for Key Issue#4: NWDAF enhancements to support network abnormal behaviours (i.e. Signalling storm) mitigation and prevention.

Network function(s) may crash due to e.g. massive UE access and unexpected misconfiguration or failure of IP routers, which may consequently generate a significant number of error and redundant reports, leading to a signalling storm.

6.X.1.1	Input Data

To detect and predict SBA signalling storms, 2 distinct types of input data are required, namely SBA signalling feature data, and NF context data.

The SBA signalling feature data can indicate the presence of a SBA signalling storm, enabling the NWDAF to identify such storms. Additionally, by training a ML model to recognize typical SBA signalling patterns, the NWDAF can predict the occurrence of SBA signalling storm when there is a trend of abnormal SBA signalling.

The SBA signalling storm feature is characterized by a significant increase in redundant SBA signalling directed towards the other core network functions over a period of time, leading to abnormal consumption of core network functions resources (e.g. over 85% CPU or memory usage). This can overwhelm the core network function's capacity to handle the SBA signalling, resulting in a high volume of SBA rejection or unhandled SBA requests. So, the following input data is needed in table 6.X.1.1-1.

Table 6.X.1.1-1: Input data: SBA signalling feature data

| Information | Source | Description |
| --- | --- | --- |
| S-NSSAI | NF | Identifies the Network Slice for which analytics information is provided. |
| DNN | NF | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Source NF ID | NF | NF instance ID of the NF who sends the SBA signalling. |
| SBA unhandled request message amount | NF | Amount of SBA unhandled request message, e.g. HTTP GET, POST message |
| SBA reject message amount | NF | Amount of SBA Reject message, e.g. HTTP response with cause value is 40x or 50x. |
| SBA redundant message amount | NF | Amount of SBA redundant message. |
| SBA message amount | NF | Amount of all the SBA message. |
| Start/end time | NF | Start and end time of detection. |

The NF context data contains information about the current UE context, or network context and NF’s load, enabling the NWDAF to determine the cause of a signalling storm, such as abnormal redundant request.

Table 6.X.1.1-2: Input data: NF context data

| Information | Source | Description |
| --- | --- | --- |
| Number of UEs | NF | Total number of UEs that is served by the NF. |
| Number of network context | NF | Total number of network context (e.g. interface context) that is maintained by the NF. |
| NF load | NRF | The load of specific NF instance(s) in their NF profile |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk). |

6.X.1.2	Output Analytics

The output analytics of signalling storm detection of NWDAF is defined in Table 6.X.1.2-1.

Table 6.X.1.2-1: signalling storm detection

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm |
| > Target NF ID | The target NF of signalling storm detected by NWDAF |
| > Reason | The reason of signalling storm (abnormal redundant request, etc.) |
| > Source NF ID | The source NF which cause the signalling storm |

The output analytics of signalling storm prediction of NWDAF is defined in Table 6.X.1.2-2.

Table 6.X.1.2-2: signalling storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm |
| > Target NF ID | The target NF of signalling storm detected by NWDAF |
| > Reason | The reason of signalling storm (abnormal redundant request, etc.) |
| > Source NF ID | The source NF which cause the signalling storm |
| > Confidence | Confidence of this prediction |

6.X.2	Procedures

For this new analytics, current procedures to support network data analytics as defined in clause 6 of TS 23.288 [5] can be reused. The NF consumer may be NRF.

Besides, the following mechanism can be used to mitigate and prevent the signalling storm.

Table 6.X.2-1: Mechanisms to mitigate and prevent the signalling storm

| Reason | NF consumer | Actions of NFs |
| --- | --- | --- |
| Abnormal redundant request | NRF | NRF configures the local policy to prevent the source NF from being discovered or discovering others. |

6.X.3	Impacts on services, entities and interfaces

NWDAF:

-	New service to provide NWDAF capability for signalling storm detection and prediction.

-	Collects training data for signalling storm detection and prediction.

-	Trains ML model for signalling storm detection.

NRF:

-	Set local policy to isolate the source NF.

End of Changes