3GPP TSG-WG SA2 Meeting #162	S2-2404819

# Changsha, China, April 15- April 19, 2024

Source: 	ZTE

Title:	Evaluation for KI#2

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Propose Evaluation for KI#2.

1. Discussion

This proposal proposes the evaluation and conclusion for KI#1 as baseline for normative work.

2. Proposal

It is proposed to capture the following changes in TR 23.700-63.

FIRST CHANGE

7	Overall Evaluation

7.x	Key Issue #2: Enhancements on UPF information exposure

There are 2[to be updated] solutions (Sol#5, Sol#7) mapping to Key Issue #2.

Solution 5 proposes that the TSN AF or TSCTSF gets UMIC for the address of user plane node ID through PDU session modification procedure via PCF and SMF, and then directly subscribe the UPF event exposure service to receive the updated TSC management information (i.e. UMIC and/or PMIC).

Solution 7 proposes that the NF/AF can request UPF via SMF to provide the UE’s NATED Public IP address and TCP/UDP port number from UPF, by using UE private IP address or SUPI as input parameter.

# * * * * End of changes * * * *