SA WG2 Meeting #162	S2-2405084

# April 15 – 19, 2024, Changsha, China	          (revision of S2-240396)

Source: 	Samsung

Title: 	Updates to Terminology

Document for: 	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This pCR proposes updates to Terminology.

# 1	Discussion

This pCR proposes updates to Terminology.

# 2. Proposal

It is proposed to adopt the following text in TR 23.700-84.

*** Start of 1st change ***

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

Editor's note:	Whether different feature space required for VFL is FFS.

Label: A label is the property of interest that is to be learned in supervised machine learning.

VFL Server: An NF that discovers and selects VFL clients, and coordinates the VFL process. The VFL server may also participate in the VFL tasks such as training and inference.

VFL Client: An NF with access to a local dataset that performs VFL training and inference coordinated by a VFL server.

VFL Active Participant: An NF with access to input data and related labels for a VFL training task.

NOTE 1:	A VFL server may have the role of VFL Active Participant in the VFL process.

VFL Passive Participant: A VFL client with access to the required input data without the required labels for a VFL training task. There can be multiple passive participants in VFL.

Editor's note:	Terms may be added, deleted or modified during conclusion discussions.

Editor's note:	During the conclusions discussion, it will be decided whether both the VFL server / VFL client and the VFL active participant / passive participant terminology is required.

*** End of 1st change ***