SA WG2 Meeting #S2-166-Ad-Hoc-e	S2-2500802

20 - 24 January, 2025, Electronic meeting	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6004 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the Qos monitoring capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_QME |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In this meeting, the CT3 send the LS (S2-2500037) on Clarification of the scope of QME. CT3 would like to ask: May all the QoS Monitoring mechanism (e.g. RT delay over two QoS flow, Packet Delay Variation, congestion, data rate) to be supported for the QoS monitoring capability report or not? There are two Qos monitoring capability in the RAN. i.e. Packet delay monitoring and Congestion information monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the capabilities in Qos monitoring feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The issue raised by CT3 is not resolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.45	QoS Monitoring

### 5.45.1	General

QoS monitoring comprises of measurements of QoS monitoring parameters and reports of the measurement result for a QoS Flow and can be enabled based on 3rd party application requests and/or operator policies configured in the PCF. Event Reporting from PCF is specified in clause 6.1.3.18 of TS 23.503 [45] and User Plane QoS Flow related QoS monitoring and reporting in clause 5.8.2.18.

The AF may request measurements for one or more of the following QoS monitoring parameters, which may trigger QoS monitoring for service data flow(s):

-	UL packet delay, DL packet delay, round trip packet delay for a service data flow, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

-	Packet Delay Variation, see clause 5.37.7.

-	Round trip packet delay considering UL on a service data flow and DL of another service data flow, see clause 5.37.4.

The following AF requested QoS requirements may trigger QoS monitoring for service data flow(s):

-	Round-trip latency requirement, see clause 5.37.6.

The PCF may generate the authorized QoS Monitoring policy for a service data flow based on the QoS Monitoring request received from the AF (as described in clause 6.1.3.21 of TS 23.503 [45]) or based on PCF local policy or configuration reasons, such as PCF awareness of dynamic satellite backhaul connection. The PCF includes the authorized QoS Monitoring policy in the PCC rule and provides it to the SMF.

The QoS monitoring parameter(s) that can be measured by means of QoS monitoring are listed below. The QoS monitoring policy in PCC rule (described in clause 6.3.1 of TS 23.503 [45]) may include the following:

-	UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

The SMF configures the UPF to perform QoS monitoring for the QoS Flow and to report the monitoring results as described in clause 5.8.2.18 with parameters determined by the SMF based on the authorized QoS Monitoring policy received from the PCF or local configuration or both.

The SMF may also configure NG RAN to measure the QoS monitoring parameters by sending QoS monitoring request based on the authorized QoS Monitoring policy received from the PCF and/or local configuration. The QoS monitoring request to the NG RAN for different parameters is as defined in clause 5.45.2 and 5.45.3.

Based on configuration, the AMF may know whether the QoS monitoring feature (i.e. each of Packet delay monitoring and/or of Congestion information monitoring) is supported by all NG RAN nodes in an area. The AMF shall, when configured, inform the SMF about the support of the QoS monitoring feature by sending the parameter NG RAN QoS monitoring capability during PDU Session establishment, PDU Session modification, Service request and UE mobility procedures. The SMF determines whether QoS monitoring is possible or not for the PDU Session based on whether the NG RAN QoS monitoring capability is received (or not) from the AMF and the QoS monitoring capability of the UPF. QoS monitoring is possible if the SMF has received the NG RAN QoS monitoring capability in the last message received from AMF (i.e. Nsmf_PDUSession_CreateSMContext or Nsmf_PDUSession_UpdateSMContext service operation)and the UPF also supports the corresponding QoS monitoring capability. Otherwise, QoS monitoring is not possible for the PDU Session. The SMF may need to notify the PCF when the support for QoS monitoring has changed as described in clause 6.1.3.5 of TS 23.503 [45].

NOTE 1:	The AMF is not meant to create dedicated signalling to warn SMF about a change of support of a  QoS monitoring feature (by all NG RAN nodes in an area). .

The following clauses describe the QoS monitoring parameters which can be measured and any specific actions or constraints for their measurement.

NOTE 2:	The QoS monitoring parameter which can be measured are parameters which describe the QoS experienced in the 5GS by the application, i.e. they are not restricted to the 5G QoS Parameters defined in clause 5.7.2.

END OF CHANGES