3GPP TSG-WG2 Meeting #166-AH-E	S2-2500229r02

# 20th – 24th January 2025, Elbonia

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0643 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to AI/ML-based positioning procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-14-01 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes a number of changes addressing the ENs below and completing/clarifying some aspects in the procedures for AIML-based positioning, related to the exception sheet approved in SP-241513. Editor's note: Whether the LMF will subscribe at NWDAF for Network Performance Analytics to decide when to trigger data collection is FFS. Editor's note: Whether and how to limit the number of UEs is FFS. Editor's note: The input data used for AI/ML based positioning will be decided by RAN WGs. Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS. Editor's note: The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs. Editor's note: Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS. Editor´s Note: Whether the LMF provides Inference Input Data information to NWDAF in the Nnwdaf_MLModelProvision service is FFS. In addition, other miscellaneous changes are proposed to clarify the specification as described below. |  |  |  |  |  |  |  |  |
| Summary of changes: |  | Following changes are proposed for 6.22.2: - Editorial correction to clarify meaning of first paragraph - 1st EN is removed, and A NOTE is added indicating that LMF can separately subscribe to analytics, but it does not need to be captured in this spec (it’d be enough indicating in TS 23.288 that LMF can be a consumer of the analytics). - For the 2nd EN, it is proposed to re-use a NOTE from TS 23.288 indicating that load can be alleviated via event filters - Clarifications on user consent are proposed indicating LMF behaviour when user consent is not provided or revoked - 3rd EN is removed, and A NOTE is added indicating the input data for AIML-based positioning is not in the scope of this specification. Following changes are proposed for 6.22.3: - Miscellaneous corrections to match figure and procedure description and clarify user consent revocation. - 4th EN is removed, proposal not to take RAN load into consideration, as LMF would only request NG-RAN to provide measurements, not trigger any measurement as such Following changes are proposed for 6.22.4: - NOTE 1 is proposed to clarify NWDAF does not decide where LMF collects the darta from - 5th EN is removed, and A NOTE is added indicating the input data for AIML-based positioning is not in the scope of this specification. - 6th EN is removed, indicating that PRU information can be included in the request. In the response, the data source type can be indicated - User consent revocation is clarified Following changes are proposed for 6.22.5: - Miscellaneous editorial clarifications are proposed - 7th EN is removed, proposal to indicate that other ML model provisioning parameters in TS 23.288 are not prevented here but do not need to be captured in this procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification of AI/ML based positioning won’t be completed on time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.2, 6.22.3, 6.22.4, 6.22.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.273 CR 0642 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 23.273 CR 0644 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 6.22.2	Data Collection at LMF for the LMF-based AI/ML positioning based on UE measurements

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training, performance monitoring and UE positioning calculation. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.22.2-1: Data collection at LMF for the LMF-based AI/ML positioning based on UE measurements

1.	The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF). The LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) that served for the area of interest.

Editor's note:	Whether the LMF will subscribe at NWDAF for Network Performance Analytics to decide when to trigger data collection is FFS.

2.	The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and "UE Positioning Capability".

Editor's note:	Whether and how to limit the number of UEs is FFS.

3.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If "UE Positioning Capability" is also requested in step 1, AMF includes UE Positing Capability, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF.

4.	The LMF checks the user consent status from UDM for the UEs with SUPIs received in step 3 and discards UEs for which the user has not provided consent for the corresponding purpose. 

The LMF further determines the UEs from the list of SUPIs that received from AMF in step 2 for data collection based on UE Positioning Capability, UE User Plane Positioning Capabilities, the PRU information available in the LMF and operator's policy. The User consent check is not needed for data collection from PRU for model performance monitoring.

Editor's note:	Further details on user consent for data collection for model training and/or model performance monitoring for LMF-based AI/ML Model positioning will be aligned with SA WG3.

5.	For each UE where the user provides consent to data collection for the corresponding purpose and the UE can support data collection, the LMF initiates a request for input data. The UE may reject the data collection request from the LMF (e.g. considering UE status, user's input). If the UE accepts data collection request, the UE may cancel the data collection later as defined in clause 6.3.4.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

At the time the LMF becomes aware that input data for a UE can no longer be collected, e.g. the user consent for data collection for model training for AI/ML positioning is revoked, the LMF stops collecting the input data from the UE.

* * * Next Change * * *

### 6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN for LMF-based AI/ML positioning is used to e.g. train the AI/ML Model.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

1.	The LMF determines that data collection is required e.g. to train an AI/ML Model for UE positioning for a number of UEs or to monitor the AI/ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

2.	The LMF may know the SUPI of these UEs, e.g. when training a AI/ML model using a PRU, which are associated to this LMF that executes data collection, the LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF in order to identify the LMF(s) which has PRU associated to a list of TAIs. The LMF may send an Nlmf_Location_MeasurementData Request service operation to one or more of the other PRU serving LMFs in order to get the data collection and the known PRU location in this case step 6 follows.

If the LMF does not know the SUPIs of those UEs, the LMF try to get the list of SUPIs from the AMF, before that the LMF finds the AMFs via NRF using Nnrf_NFDiscoveryRequest.

3.	For each AMF serving a specific area. the LMF subscribes to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest".

4.	The AMF send Namf_EventExposure_Subscriber_Response or Namf_EventExposure_Notify ("list of SUPIs in/out the area of interest").

For each SUPI provided by the AMF that is in the area of interest then steps 5 to step 6 are performed, subject to the user consent check described in step 5.

5.	The LMF checks whether the SUPI provided user consent for data collection for the purpose of training for AI/ML positioning with UDM. If user consent is provided, then step 6 follows. If user consent is not provided, the LMF does not perform step 6 for this UE, that will not be part of the data collection, and its data is not used for the LMF-based AI/ML Model positioning.

Editor's note:	Further details on user consent for data collection for model training or for performance monitoring for LMF-based AI/ML positioning will be aligned with SA WG3.

6.	The LMF requests and receives input data for the target UE from NG-RAN.

7.	The LMF requests and receives ground truth data from the target UE.

The measurements and ground truth data are used for training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for performance monitoring.

At the time the LMF becomes aware that input data for a UE can no longer be collected, e.g. the user consent for data collection for model training for AI/ML positioning is revoked, the LMF stops collecting the input data from the UE.

The LMF may initiate data collection for multiple UEs simultaneously, as such step 8 and 9 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

* * * Next Change * * *

### 6.22.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement related data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Editor's note:	It is FFS how to adapt this procedure to ML model performance monitoring.

Figure 6.22.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note:	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

NOTE 2:	The specific input data collected by LMF from UE and NG-RAN for LMF-based AI/ML Positioning are not in the scope of this specification.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF after having previously subscribed by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number for data samples and time window for data collection.

Editor's note:	Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. Whether the NWDAF needs to indicate Data source type (UE or RAN measurements) and/or data type(s) and/or a target ground truth label precision is also FFS.

4.	The LMF performs steps 1 to 4 in clause 6.22.2, or steps 1 to 7 in clause 6.22.3.

5.	The LMF sends the collected data samples to the NWDAF by invoking Nlmf_DataExposure_Notify service operation, then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples.

At the time the LMF becomes aware that input data for a UE can no longer be collected, e.g. the user consent for data collection for model training for AI/ML positioning is revoked, the LMF stops collecting and sharing with NWDAF the input data from the UE.

* * * End of Changes * * *