3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500485r04

# Electronic Meeting, 20-24 January 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5963 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of non-3GPP device connection information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, InterDigital, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It has been agreed in clause 4.3.3.2 of TS 23.502 that a UE may conatin non-3GPP device connection information in the request of PDU session modification for QoS differentiation of connected non-3GPP device traffic. However, it’s not clear what parameters are included in the non-3GPP device connection information. The scenerio where a non-3GPP device stops being connected to the UE and therefore no longer requires differentiated QoS has not been addresed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the definition of non-3GPP device connection information and conatined parameters. The parameter to indicate adding or deleting QoS differentiation for non-3GPP device traffic is included in non-3GPP device connection information. A note is added to clarify the selection of PDU session for non-3GPP device traffic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The paramters included in non-3GPP device connection information are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

### 5.52.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS, the non-3GPP Device Connection Information may be signaled by the UE as defined in TS 24.501 [x]. When a non-3GPP device is connected to the UE, the UE may include the non-3GPP Device Connection Information in PDU Session Modification Request to SMF. The SMF forwards the non-3GPP Device Connection Information to the PCF for policy control.

NOTE: It is up to UE implementation to determine when to initiate PDU Session Modification procedure for updating the non-3GPP device connection information.

For an Ethernet PDU Session, the non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier,

-	MAC address used in PDU session,

-	Optionally, VLAN tag ID that is associated with the non-3GPP device used in PDU session,

-	Requested operation (add, delete).

For an IPv4 or IPv4v6 PDU Session, the non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier,

-	IP Address associated with the non-3GPP device used in PDU session,

-	Optionally, port ranges associated with the non-3GPP device used in PDU session,

-	Requested operation (add, delete).

For an IPv6 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

- Non-3GPP Device Identifier,

- IPv6 Address/prefix(sub) associated with the non-3GPP device used in PDU session,

- Optionally, port ranges associated with the non-3GPP device used in PDU session,

- Requested operation (add, delete).

Editor’s Note: The detailed encoding of Non-3GPP Device Connection Information will be defined in stage-3.

# >>>>END OF CHANGES<<<<