3GPP TSG-SA2 Meeting #166AH-e	S2-2500527r03

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5969 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on IP version used for MPQUIC-E |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When a PDU Session is established, a UE does not provide capability on IP version the UE supports. Instead, the UE explictly indicates which IP version should be used in the PDU Session Type (e.g. IPv4, IPv6, IPv4v6). In case of Etherent PDU Session, PDU Sessio Type is set to "Etherent". For MPQUIC-E, the SMF need to provide "MPQUIC link-specific multipath" addresses/prefixes to the UE. However, network does not know which IP version is supported by the UE. If network provides IP address not supported by the UE, MPQUIC-E does not work. In order to solve this issue, two alternatives are possible. Alternative 1: Providing UE's IP version capability to the network. In this case, when a UE requests MA PDU Session, the UE shall include capability on supported IP versions in the PDU Session Establishment Request message. Based on the information, SMF can indicate the capability to the UPF so that UPF assigns correct MPQUIC link-specific multipath addresses/prefixex and MPQUIC proxy information. Alternative 2: Mandating the UE to support all IP versions so that the network can use any IP version. In this case, based on operator's configuration, the UPF can use any IP versions when the UPF allocates MPQUIC link-specific multipath addresses/prefixex and MPQUIC proxy information. Considering overall impact, CR is written based on Alternative 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify how the network determines correct IP version of MPQUIC link-specific multipath addresses/prefixex and MPQUIC proxy information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification and MPQUIC-E may not work if wrong IP version of MPQUIC is allocated to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

##### 5.32.6.2.2	MPQUIC-based Functionalities

MPQUIC-based steering functionalities are supported as follows:

-	The MPQUIC-based functionalities enable steering, switching and splitting of data traffic between the UE and UPF, in accordance with the ATSSS policy created by the network.

-	The operation of the MPQUIC-UDP functionality is based on IETF RFC 9298 [170] "proxying UDP in HTTP", which specifies how UDP traffic can be transferred between a client (UE) and a proxy (UPF) using the IETF RFC 9114 [171] HTTP/3 protocol.

The MPQUIC-UDP functionality may be enabled for an MA PDU Session with type IPv4, IPv6 or IPv4v6, when both the UE and the network support this functionality. The MPQUIC-UDP functionality shall not be enabled when the type of the MA PDU Session is Ethernet.

-	The operation of the MPQUIC-IP functionality is based on RFC 9484 [214] "Proxying IP in HTTP", which specifies how IP traffic can be transferred between a client (UE) and a proxy (UPF) using the IETF RFC 9114 [171] HTTP/3 protocol.

The MPQUIC-IP functionality may be enabled for a MA PDU Session with type IPv4, IPv6 or IPv4v6, when both the UE and the network support this functionality. The MPQUIC-IP functionality shall not be enabled when the type of the MA PDU Session is Ethernet.

-	The operation of the MPQUIC-E functionality is based on IETF draft-ietf-masque-connect-ethernet [215] "Proxying Ethernet in HTTP", which specifies how Ethernet traffic can be transferred between a client (UE) and a proxy (UPF) using the IETF RFC 9114 [171] HTTP/3 protocol.

The MPQUIC-E functionality may be enabled when the type of the MA PDU Session is Ethernet. In this case, the UPF allocates the same IP type of "MPQUIC link-specific multipath" addresses/prefixes and MPQUIC proxy information based on local configuration. When the MPQUIC-E functionality is enabled for an Ethernet type PDU Session, based on the allocated IP type (IPv4, IPv6, IPv4v6) of "MPQUIC link-specific multipath" addresses/prefixes and MPQUIC proxy information, the SMF indicates PDU Session type as IPv4 or IPv6 or IPv4v6 in the N2 information to NG-RAN, as described in TS 23.502 [3].

NOTE:	In case of MPQUIC-E, it is assumed that the UE supports all IP types (IPv4, IPv6, IPv4v6) so that the UPF can select any version of IP addresses based on local configuration.

The HTTP/3 protocol operates on top of the QUIC protocol (IETF RFC 9000 [166], IETF RFC 9001 [167] , IETF RFC 9002 [168]), which supports simultaneous communication over multiple paths, as defined in draft-ietf-quic-multipath [174].

The MPQUIC functionality(ies) in the UE communicates with the associated MPQUIC Proxy functionality(ies) in the UPF (see Figure 4.2.10-1) using the user plane of the 3GPP access, or the non-3GPP access, or both.

The MPQUIC-UDP, MPQUIC-IP and MPQUIC-E functionality(ies) are composed of three components:

1)	QoS flow selection & Steering mode selection: This component in the UE initiates the establishment of one or more multipath QUIC connections, after the establishment of the MA PDU Session and, for each uplink UDP flow for MPQUIC-UDP functionality), IP flow (for MPQUIC-IP functionality) and Ethernet flow (for MPQUIC-E functionality), it selects a QoS flow (based on the QoS rules), a steering mode and a transport mode (based on the ATSSS rules). This component in the UPF selects, for each downlink traffic flow of respective type, a QoS flow (based on the N4 rules), a steering mode and a transport mode (based on the N4 rules). The supported transport modes are defined below.

In the UE, this component is only used in the uplink direction, while, in the UPF, this component is only used in the downlink direction.

2)	HTTP/3 layer: Supports the HTTP/3 protocol defined in RFC 9114 [171] and the extensions defined in:

-	IETF RFC 9298 [170] for supporting UDP proxying over HTTP;

-	IETF RFC 9484 [214] for supporting IP proxying over HTTP;

-	IETF draft-ietf-masque-connect-ethernet: [215] for supporting Ethernet proxying over HTTP;

-	IETF RFC 9297 [172] for supporting HTTP datagrams; and

-	IETF RFC 9220 [173] for supporting Extended CONNECT.

The HTTP/3 layer selects a multipath QUIC connection to be used for each traffic flow and allocates a new QUIC stream on this connection that is associated with the traffic flow. It also configures this QUIC stream to apply a specific steering mode.

In the UE, the HTTP/3 layer implements an HTTP/3 client, while, in the UPF, it implements an HTTP/3 proxy.

3)	QUIC layer: Supports the QUIC protocol as defined in the applicable IETF specifications (IETF RFC 9000 [166], IETF RFC 9001 [167], IETF RFC 9002 [168]) and the extensions defined in:

-	IETF RFC 9221 [169] for supporting unreliable datagram transport with QUIC; and

-	draft-ietf-quic-multipath [174] for supporting QUIC connections using multiple paths simultaneously.

When any one of the MPQUIC based steering functionalities is applied, the protocol stack of the user plane is depicted in figure below.

Figure 5.32.6.2.2-1: UP protocol stack when an MPQUIC-based functionality is applied

If the UE indicates that it is capable of supporting any one of the MPQUIC-UDP, MPQUIC-IP or MPQUIC-E functionalities, as described in clause 5.32.2 and the network agrees to enable the corresponding MPQUIC functionality(ies) for the MA PDU Session then:

i)	An associated MPQUIC Proxy functionality is enabled in the UPF for the MA PDU Session.

ii)	The network allocates two IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses/prefixes; one associated with 3GPP access and another associated with the non-3GPP access (in the case of IP based PDU Session Types, the MPQUIC link-specific multipath addresses/prefixes are allocated in addition to the UE IP address/prefix for the MA PDU Session). In the UE, these two link-specific multipath IP addresses/prefixes are used only by the associated MPQUIC functionality. Each "MPQUIC link-specific multipath" address/prefix assigned to UE may not be routable via N6. The MPQUIC functionality in the UE and the associated MPQUIC Proxy functionality in the UPF shall use the "MPQUIC link-specific multipath" addresses/prefixes for transmitting traffic flows over non-3GPP access and over 3GPP access. The MPQUIC Proxy functionality shall use the IP address/prefix of the MA PDU session for the communication with the final destination. In Figure 5.32.6.1-1, the IP@3 corresponds to the IP address of the MA PDU Session and the IP@4 and IP@5 correspond to the "MPQUIC link-specific multipath" addresses. The following UE IP address management applies:

-	The MA PDU IP address/prefix shall be provided to the UE via mechanisms defined in clause 5.8.2.2.

-	The "MPQUIC link-specific multipath" IP addresses/prefixes shall be allocated by the UPF and shall be provided to the UE via SM NAS signalling.

NOTE 1:	After the MA PDU Session is released, the same UE IP addresses/prefixes are not allocated to another UE for MA PDU Session in a short time.

NOTE 2:	When both MPQUIC-UDP and MPQUIC-IP steering functionalities are enabled for a single MA PDU Session, the network can allocate a common single pair of "MPQUIC link-specific multipath" addresses/prefixes to be used for both MPQUIC-UDP and MPQUIC-IP functionalities.

iii)	The network shall send MPQUIC proxy information to UE, i.e. one IP address of UPF, one UDP port number and the proxy type (e.g. "connect-udp"). This information is used by the UE for establishing multipath QUIC connections with the UPF, which implements the MPQUIC Proxy functionality.

NOTE 3:	When both MPQUIC-UDP and MPQUIC-IP steering functionalities are enabled for a single MA PDU Session, the network can send a single MPQUIC proxy information (i.e. one IP address of UPF and one UDP port number) to the UE, to be used for both MPQUIC-UDP and MPQUIC-IP functionalities.

iv)	After the MA PDU Session is established, the UE determines the number of multipath QUIC connections to be established with the UPF. The UE determines to establish at least as many multipath QUIC connections as the number of QoS flows of the MA PDU Session, i.e. one multipath QUIC connection per QoS flow. Each multipath QUIC connection carries the traffic mapped to a single QoS flow.

Security aspects are defined in TS 33.501 [29].

For the downlink traffic to which the MPQUIC functionality is to be applied, the QoS rules provided to UE include downlink QoS information and the UE applies the downlink QoS information to establish multipath QUIC connections for the QoS flows used for the downlink traffic only.

v)	During a QUIC connection establishment, the UE and UPF negotiate QUIC transport parameters and indicate (a) support of QUIC Datagram frames and (b) support of multipath. They indicate support of QUIC Datagram frames by providing the "max_datagram_frame_size" transport parameter with a non-zero value (see RFC 9221 [169]) and they indicate support of multipath by providing the "enable_multipath" transport parameter (see draft-ietf-quic-multipath [174]).

In addition, during a QUIC connection establishment the QoS flow associated with this connection is determined. The UE sends all traffic of a QUIC connection over the QoS flow associated with this QUIC connection. This enables the UPF to determine the QoS flow associated with a QUIC connection and to select a QUIC connection for sending the downlink traffic of a QoS flow.

vi)	After a QUIC connection establishment, the HTTP/3 client in the UE and the HTTP/3 proxy in the UPF negotiate HTTP settings and indicate support of HTTP Datagrams (see RFC 9297 [172]) and support of Extended CONNECT (see RFC 9220 [173]). To use MPQUIC-UDP for a UDP traffic flow, the UE then sends a HTTP/3 CONNECT request (see RFC 9298 [170]) to the HTTP/3 proxy in the UPF. To use MPQUIC-IP for an IP traffic flow, the UE then sends a HTTP/3 CONNECT-IP request (see IETF RFC 9484 [214]) to the HTTP/3 proxy in the UPF. To use MPQUIC-E for an Ethernet traffic flow, the UE then sends a HTTP/3 CONNECT-Ethernet request (see IETF draft-ietf-masque-connect-ethernet [215]) to the HTTP/3 proxy in the UPF.

vii) The network may indicate to UE the list of applications for which the MPQUIC-UDP, MPQUIC-IP or MPQUIC-E functionality should be applied. This is achieved by using the Steering Functionality component of an ATSSS rule (see clause 5.32.8).

* * * * End of Changes * * * *