3GPP TSG-WG SA2 Meeting #167	S2-2501850

# Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6122 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on shifting N3 traffic to the new BH PDU sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If a new BH PDU Session needs to be established for supporting a UE's traffic for a S-NSSAI in the MWAB-Broadcasted PLMN/SNPN, after the completion of the PDU session establishment procedure, it should be MWAB-gNB not MWAB-UE to shift the N3 traffic for the UE to the new BH PDU sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction is added that it is MWAB-gNB not MWAB-UE to shift the N3 traffic for the UE to the new BH PDU sessions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.49.1.4	Support of network slicing for UEs served by a MWAB

A MWAB-gNB can be configured to associate S-NSSAI(s) in the MWAB-Broadcasted PLMN/SNPN to the traffic descriptor for selection of the BH PDU Sessions parameters e.g. use dedicated S-NSSAI in BH PLMN. If no such association exists for a specific S-NSSAIs of the MWAB Broadcasted PLMN/SNPN, the MWAB-gNB associates this S-NSSAI to default traffic descriptors for the selection of BH PDU Session(s) for the related N2 and/or N3 backhauling. The MWAB-gNB requests the BH PDU sessions as specified in clause 5.49.1.2.

When the MWAB-gNB obtains the S-NSSAI of a UE PDU Sessions from the UE PDU session context during PDU session establishment, it checks whether a configured association exists for the S-NSSAI of the PDU session of the UE and, if so, it uses the related BH PDU session for the UE's PDU Session.

If a new BH PDU Session needs to be established for supporting a UE's traffic for a S-NSSAI in the MWAB-Broadcasted PLMN/SNPN, after the completion of the PDU session establishment procedure, the MWAB-gNB may trigger a PDU session resource modification procedure to shift the N3 traffic for the UE to the new BH PDU sessions.

NOTE:	Due to the limitation of the maximum number of PDU sessions supported by a UE, it is expected that there can only be a small number of dedicated BH PDU sessions for specific S-NSSAI(s) in the MWAB-broadcasted PLMN.

# * * * * End of changes * * * *