SA WG2 Meeting #S2-167	S2-2502054

17 - 21 February, 2025, Athens, Greece	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6143 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the S-NSSAI for BH PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the last meeting, the RAN3 sends the LS (S2-2500052), it said: RAN3 has agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO: For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling. In the exsiting 23.501, the MWAB-UE is authorized based on its subscription information which includes dedicated S-NSSAI(s) and DNN for MWAB operation. If multi-hop prevention is configured by the operator, the dedicated S-NSSAI used for BH PDU session, should not be used by the normal UEs. Otherwise this UE cannot be handed over to MWAB-gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on the S-NSSAI for BH PDU session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If multi-hop prevention is configured by the operator,the UE can not be handed over to MWAB-gNB if it has established a PDU session with S-NSSAI used for BH PDU session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.1.4, 5.49.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.49.1.4	Support of network slicing for UEs served by a MWAB

A MWAB-gNB can be configured to associate S-NSSAI(s) in the MWAB-Broadcasted PLMN/SNPN to the traffic descriptor for selection of the BH PDU Sessions parameters e.g. use dedicated S-NSSAI in BH PLMN. If no such association exists for a specific S-NSSAIs of the MWAB Broadcasted PLMN/SNPN, the MWAB-gNB associates this S-NSSAI to default traffic descriptors for the selection of BH PDU Session(s) for the related N2 and/or N3 backhauling. The MWAB-gNB requests the BH PDU sessions as specified in clause 5.49.1.2.

When the MWAB-gNB obtains the S-NSSAI of a UE PDU Sessions from the UE PDU session context during PDU session establishment, it checks whether a configured association exists for the S-NSSAI of the PDU session of the UE and, if so, it uses the related BH PDU session for the UE's PDU Session.

If a new BH PDU Session needs to be established for supporting a UE's traffic for a S-NSSAI in the MWAB-Broadcasted PLMN/SNPN, after the completion of the PDU session establishment procedure, the MWAB-UE may trigger a PDU session resource modification procedure to shift the N3 traffic for the UE to the new BH PDU sessions.

NOTE 1:	Due to the limitation of the maximum number of PDU sessions supported by a UE, it is expected that there can only be a small number of dedicated BH PDU sessions for specific S-NSSAI(s) in the MWAB-broadcasted PLMN.

The multi-hop prevention is configured by the operator, the dedicated S-NSSAI used for BH PDU session, should not be used by the normal UEs.

NOTE 2:	If the UE has established a PDU session with S-NSSAI used for BH PDU session, it can not be handed over to MWAB-gNB as specified in the clause 5.49.5.

Next CHANGE

#### 5.49.3.2	MWAB-UE authorization

The MWAB-UE is authorized based on its subscription information which includes dedicated S-NSSAI(s) and DNN for MWAB operation. Such dedicated S-NSSAI(s) and DNN(s) shall be part of the subscription information for the MWAB-UE as specified in clause 5.2.3.3.1 of TS 23.502 [3].

NOTE X:	The S-NSSAI in dedicated S-NSSAI(s) and DNN for MWAB operation, cannot be used by normal UE. Otherwise, the UE cannot be handed over to MWAB-gNB.

The authorization of the MWAB-UE to establish connectivity in a BH PLMN/SNPN is based on whether dedicated S-NSSAI(s) for MWAB operations are received as part of Allowed NSSAI in BH PLMN/SNPN following existing procedures as described in clause 5.15 and whether it can establish BH PDU session(s) in the BH PLMN/SNPN using the dedicated S-NSSAI(s) and DNN(s).

The MWAB-UE authorization supports time-based or location-based control.

-	For time-based control, existing time-based network slice subscription control can be reused (including the enhancements in clause 5.15.16).

-	For location-based control, existing mechanism, e.g. service area restriction, LADN based control (see e.g. clauses 5.6.5 and 5.6.5a) can be reused.

NOTE 1:	When the MWAB-UE can no longer use S-NSSAI(s) dedicated to MWAB operation,, e.g. the dedicated S-NSSAIs and DNNs for MWAB operation are removed from the subscription (i.e. the MWAB becomes unauthorized) and a UE subscription update (including a UE configuration update) step happens, or the MWAB-UE moves to an area where the S-NSSAI is not available, if the AMF is configured to do so for the dedicated S-NSSAI and DNN for MWAB operation, the AMF delays the corresponding PDU session release based on an operator configuration (e.g. a local configured timer to allow time for the MWAB-gNB to handover the UE(s) it serves to other cells).

NOTE 2:	If the S-NSSAI(s) used by BH PLMN for MWAB operation is not dedicated for MWAB, the behaviours of AMF as in NOTE 1 cannot be limited to MWAB-UEs only. Additionally, it may also not be possible to configure specific treatment for MWAB-UEs (different from other UEs) in the NG-RAN.

The MWAB-UE may be connected to the BH PLMN/SNPN for other services, if allowed by the subscription data, even if the S-NSSAIs and DNNs for Backhaul PDU sessions are no longer authorized.

END OF CHANGES