3GPP TSG SA WG2 Meeting #167	S2-2502441

# 17 - 21 February, 2025, Athens, Greece	    (Revision of S2-2502200, 1733, 1096, 0624)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1552 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to support multiplexing multiple DC applications over single SCTP connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-12-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are two editor’s notes related to how to handle the case when the standalone IMS DC session is rejected by 488 due to not supporting data channel multiplexing. This contribution aims to propose a solution to resolve the editor’s notes: The IMS AS reports the session establishment event to DCSF. If DCSF determines to retry the session establishment with de-multiplexed data channels, it instructs IMS AS accordingly. IMS AS sends INVITE request with de-multiplexed SDP offer again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor’s notes are removed and texts are added to describe the solution described above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.10.2.2, AC.7.10.3.1, AC.7.10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In revision 2, three changes are added: - Move the last bullet of AC.7.10.2.1 to AC.7.10.2.2 because this bullet is about capability negotiation between IMS networks; Add an editor’s note of whether to include Feature-Caps header in SIP response message. - Generalize the new texts in AC.7.10.3.1 to apply to both 488 and 18X. - Update the procedure of handling 18X in AC.7.10.4.2.2 to determine whether the media should be de-multiplexed before re-sending INVITE request. In revision 3: - Remove the first EN; - Add a NOTE in AC.7.10.2.2 to clarify why 488 is restricted to standalone DC in this context; - Void the original AC.7.10.4.2.2 and add AC.7.10.4.2.2a with the title of the clause changed; |  |  |  |  |  |  |  |  |

**** First Change****

AC.7.10.2.1	Capability negotiation between UE and IMS network

The UE and IMS network may support DC multiplexing capability negotiation as follows:

-	The UE and the IMS network mutually negotiate their capabilities of data channel multiplexing during registration procedure.

-	If the UE supports data channel multiplexing, when the UE registers on the IMS network, the UE shall include a media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing in the initial and any subsequent REGISTER request.

-	If the UE supports data channel multiplexing, when the UE initiates a IMS session using data channel multiplexing, the UE shall include the media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing in the INVITE request.

-	If the P-CSCF supports data channel multiplexing, when the P-CSCF receives the REGISTER request from the UE, the P-CSCF shall include a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the initial and any subsequent REGISTER request.

NOTE:	The P-CSCF and IMS-AGW need to understand the multiplexed SDP to avoid gating the multiplexed media streams.

-	If the home IMS network supports this feature, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the 200 OK response to the initial and any subsequent REGISTER request.

-	If the IMS network supports this feature, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting data channel multiplexing in the SIP request messages to the remote network.

**** Second Change****

AC.7.10.2.2	Capability negotiation between originating and terminating network

The DCSF in originating IMS network shall determine whether the terminating IMS network supports data channel multiplexing by local configuration or based on the SDP answer received from remote network setting port of multiplexed DC media to zero.

NOTE 1:	The DCSF can be configured to only allow multiplexing data channels between the UE and the IMS network, i.e. on UNI interface.

If the DCSF in originating IMS network receives session establishment failure 488 response to INVITE request from IMS AS, and if the IMS session is a standalone IMS DC session, based on operator policy, the DCSF may assume the session failure is due to terminating network not supporting data channel multiplexing and instruct the IMS AS to de-multiplex all data channel media and resend INVITE request with de-multiplexed SDP offer as described in clause AC.7.10.3.1.

NOTE 2:	If the IMS session rejected by 488 includes other media components, e.g. voice, it means the session is not rejected because of not supporting data channel multiplexing. Thus re-sending INVITE request with de-multiplexed SDP is not needed.

**** Third Change****

### AC.7.10.3	Determination and handling of multiplexing and de-multiplexing

#### AC.7.10.3.1	Determination of multiplexing and de-multiplexing

When the IMS AS receives the INVITE request with data channel multiplexing, if data channel multiplexing is supported, it reports the event to the DCSF for data channel management. The DCSF determines whether the data channel multiplexing is allowed and whether serving network needs to de-multiplex the data channel traffic based on subscription data, binding information, the endpoints of data channels and capability of remote network.

When the IMS AS receives a response that rejects the multiplexed data channel media, e.g. a 18X with SDP answer setting port of multiplexed data channel media to zero, or a 488 response to the INVITE request of a  standalone IMS DC session with multiplexed SDP offer, the IMS AS shall firstly report the session establishment failure event to the DCSF and follows the instructions from DCSF. The DCSF determines whether to release the session or retry the session establishment with normal SDP offer as specified in clause AC.7.10.2.2. If the DCSF determines to retry the session establishment, it instructs the IMS AS to modify the media resource of multiplexed data channels in the MF and further re-send INVITE request with de-multiplexing SDP offer after the media resource modification is successful.

#### AC.7.10.3.2	Handling multiplexing and de-multiplexing at the originating network

The IMS AS and DCSF in the originating network follows the following principles to handle data channel multiplexing and de-multiplexing:

-	If the data channels are kept multiplexed, the DCSF instructs originating IMS AS accordingly. The originating IMS AS further instructs the MF to reserve the same media termination for multiplexed data channels. This attempt may succeed and IMS session establishment may be successful.

-	If the data channels are kept multiplexed, the DCSF instructs originating IMS AS accordingly. The originating IMS AS further instructs the MF to reserve the same media termination for multiplexed data channels. If the terminating network rejects the SDP offer because the terminating network does not support multiplexing, then originating IMS AS and originating DCSF needs to revert to a new demultiplexed SDP offer towards the target network as described in next bullet.

-	If the originating network determines to de-multiplex the data channels, the DCSF in the originating network instructs originating IMS AS accordingly. The originating IMS AS further instructs the MF to reserve separate media terminations for the de-multiplexed streams. When the originating IMS AS sends the SDP offer to terminating IMS network, the data channels are not multiplexed in the SDP offer. IMS session establishment continues.

#### AC.7.10.3.3	Handling at the terminating network.

The IMS AS and DCSF in the terminating network follows the following principles to handle data channel multiplexing and de-multiplexing:

-	If the data channels are kept multiplexed, the terminating DCSF instructs terminating IMS AS accordingly. The terminating IMS AS further instructs the MF to reserve the same media termination for multiplexed data channels;

-	If the data channels is determined to be de-multiplexed, the terminating DCSF instructs terminating IMS AS accordingly. The terminating IMS AS further instructs the MF to reserve separate media terminations for the de-multiplexed streams. When the terminating IMS AS sends the SDP offer to terminating UE, the data channels are not multiplexed in the SDP offer.

**** Fourth Change****

AC.7.10.4.2.2	Void

**** Fifth Change****

##### AC.7.10.4.2.2a	Application data channel multiplexing when terminating network rejects multiplexed data channel media

Figure AC.7.10.4.2.2a-1 shows the procedure of multiplexing application data channels when terminating network does not support data channel multiplexing and rejects the multiplexed data channel media.

In this example procedure, the originating UE initiates a IMS session with audio and DC media components. The UE multiplexes 3 applications all with different endpoints. The originating network de-multiplexes app#1 because app#1 is terminated in originating network. The originating network further de-multiplexes app#2 and app#3 because the terminating network does not support data channel multiplexing and rejects the multiplexed SDP offer, by updating the session towards terminating network with normal SDP offer.

Figure AC.7.10.4.2.2a-1: Application data channel multiplexing when terminating network does not support data channel multiplexing

1.	When the UE#1 wants to multiplex its application data channels for app#1, #2 and #3, the UE includes multiplexed SDP media description when sending reINVITE request. The endpoints of the applications are different, that of app#1 is DC AS in originating network, that of app#2 is DC AS in terminating network and that of app#3 is terminating UE. The media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing is included in reINVITE request.

2.	The IMS AS report event to DCSF.

3.	The DCSF determines that data channel for app#1 needs to be de-multiplexed because its target endpoint is local DC AS. The DCSF also determine that the multiplexing for app#2 and #3 is kept unchanged towards their target terminating network as described in clause AC.7.10.3.

NOTE:	The DCSF can determines that terminating network does not support data channel multiplexing based on local configuration, In this case, the DCSF will determine to de-multiplex app#2 and app#3 and instruct IMS AS to send normal SDP offer to terminating network without the following steps.

4.	The DCSF generates data channel media information for originating side and terminating side. In this example, media stream of data channel for app#1 reuses the originating media ID of multiplexed media in the SDP, media stream of data channels for app#2 and #3

5.	The DCSF instructs the IMS AS to reserve resources on MF.

6.	The IMS AS instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF.

7-8.	The session continues after the media resource is reserved successfully.

9-10.	The IMS AS generates reINVITE request to terminating network. The reINVITE request includes capability of supporting data channel multiplexing and the SDP offer for multiplexed data channels of app#2 and #3.

11-12.	The terminating network does not support data channel multiplexing. The terminating network replies 18X with the port of media component for multiplexed data channels set to zero in SDP answer and no capability indication of supporting data channel multiplexing is included.

13.	The IMS AS reports the event of terminating network not supporting data channel multiplexing to the DCSF along with the media information of SDP answer.

14.	The DCSF instructs the IMS AS and the IMS AS further requests the MF to modify media resources for streams of app#2 and #3.

15-16.	The IMS AS determines that the terminating network does not support data channel multiplexing. It sends UPDATE request with normal SDP offer for app#2 and #3 to re-negotiate SDP with terminating network.

17-18.	The terminating network replies 200 OK (UPDATE) with SDP answer for app#2 and #3.

19.	The IMS AS replies 18X response with SDP answer for multiplexed data channels for app#1, #2 and #3 to UE#1.

20.	The session is established successfully. The streams of app#1, #2 and #3 are transported in the same SCTP connection between UE#1 and originating MF. The stream of app#1 is routed to DC AS and streams of app#2 and #3 are transported to terminating network and are routed to DC AS and UE#2 accordingly.

##### **** Sixth Change****

##### AC.7.10.4.2.3	Application data channel multiplexing when terminating network rejects standalone IMS DC session

Figure AC.7.10.4.2.3-1 shows the procedure of multiplexing application data channels when terminating network rejects the session.

In this example procedure, the originating UE initiates a standalone IMS DC session. The UE multiplexes 2 applications. The originating network sends INVITE request with only multiplexed SDP offer towards terminating network. The terminating network returns 488 response due to not supporting data channel multiplexing. The IMS AS in originating network reports the session establishment failure event to the DCSF in originating network. The DCSF decides to retry the session establishment with normal SDP offer since the session is a standalone IMS DC session and instructs the IMS AS to modify the media resource of multiplexed data channels in the MF and further send INVITE request with de-multiplexing SDP offer after the media resource modification is successful. The session is established successfully after the retry in this example procedure.

Figure AC.7.10.4.2.3-1: Application data channel multiplexing when terminating network rejects standalone IMS DC session

1.	When the UE#1 initiates a standalone IMS DC session which multiplex the data channels for application #1 and #2, the UE includes multiplexed SDP media description when sending the INVITE request. The endpoints of the applications are terminating network. The media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing is included in INVITE request.

2.	The IMS AS report event to DCSF.

3.	The DCSF determines that data channels for app#1 and app#2 are kept multiplexed towards terminating network.

4.	The DCSF generates data channel media information for originating side and terminating side.

5.	The DCSF instructs the IMS AS to reserve resources on MF.

6.	The IMS AS instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF.

7-8.	The session continues after the media resource is reserved successfully.

9-10.	The IMS AS generates an INVITE request to terminating network. The INVITE request includes capability of supporting data channel multiplexing and the SDP offer for multiplexed data channels of app#1 and #2.

11-12.	The terminating network does not support data channel multiplexing. Since the multiplexed SDP offer is not supported, the terminating network rejects the session with 488 response and no capability indication of supporting data channel multiplexing is included.

13.	The IMS AS determines that the terminating network does not support data channel multiplexing and the session is a standalone IMS DC session. It reports the session establishment failure event to the DCSF with the indication that the terminating network does not support data channel multiplexing.

14. The DCSF determines to retry session establishment to use SDP offer with separate data channels for app#1 and #2 since the session is a standalone IMS DC session.

15-17.	The DCSF instructs the IMS AS and the IMS AS further requests the MF to modify media resources of streams of app#2 and #3 from multiplexed data channels to separate data channels.

18.	The DCSF responds the session establishment failure event notification and indicates to the IMS AS to retry the session establishment.

19-20.	The IMS AS sends INVITE request to terminating network again with updated SDP offer with separate data channels for app#1 and #2.

21-26.	The session is established successfully. The streams of app#1 and #2 are transported in same SCTP connection between UE#1 and originating MF and in separate SCTP connections between originating MF and terminating network.

**** End of Changes ****