3GPP TSG-SA2 Meeting #168	S2-2503214

Goteborg, Sweden, April 7 – 11, 2025

Source:	Ericsson

Title:	Extension for XRM on-path signalling Connect-UDP Forwarded Mode.

Document for:	Discussion

Agenda Item:	30.6

Work Item / Release:

Abstract: The E2E performance can be further enhanced if the Forwarded Mode of Connect-UDP method used for on-path N6 signalling is used together with communication between Application client (AC) and UPF.

# 1. Introduction

The scope proposed in NWM for XRM Phase 3 includes WT-1 “Enhancements to UDP and QUIC-aware proxying using HTTP”, which then includes:

a) WT-1.1: Study whether and how to allow the proxy functionality to be located optionally also in the UPF in addition to the AS side, enabling the Application Client (AC) to connect to and establish a secure channel with the UPF for bi-directional UE-NW information exchange.

When Connect-UDP method is used for on-path N6 signalling, if the traffic is carried over QUIC, UPF and AS proxy may agree on using the Forwarded Mode which allows for forwarding of packets without requiring full re-encapsulation and re-encryption. Discussion follows on how WT-1.1 could further improve the E2E performance in this case.

# 2. Discussion

As part of XRM Phase 2, several on-path N6 signalling methods have been specified that can be used to transfer media related information (MRI) over N6 together with the encrypted payload allowing that this information is received by 5GS synchronously with the media packet (see TS 23.501 clause 5.37.9). MRI is provided by expanding through encapsulation, transformation, or protocol extension mechanisms of the encrypted transport IP packet transferred between Application Server (AS) and Application Client (AC). These methods have some known shortcomings that could be addressed to enhance the performance of the application end-to-end.

When Connect-UDP method is used for on-path N6 signalling, if the traffic is carried over QUIC, UPF and AS proxy may agree on using the Forwarded Mode which allows for forwarding of packets without requiring full re-encapsulation and re-encryption (TS 23.501 clause 5.37.9.3).

In their liaison statement (LS) response (see S2-2412356), the IETF Masque WG noted for Connect-UDP solution with Forwarded Mode several aspects that could not be taken in Rel-19 due to tight deadlines. Addressing these aspects opens an opportunity for improvement:

“When processing QUIC packets without acting as a QUIC endpoint, standards-compliant processing is limited to the invariants documented in RFC 8999. Nothing in that RFC promises that Connection ID lengths will be fixed for the duration of a connection.”

“In the proposed design, the UPF decides whether each packet is tunneled or forwarded, and the UE is not aware of which path was taken. This prevents the UE from correctly implementing Path MTU detection, as these different paths are likely to have distinct MTUs.”

The first aspect from the LS response is about reliable detection of QUIC packets and correctly identifying end-to-end QUIC connections. The IP/UDP header is not expected to change even if the QUIC connection ID for the E2E packet changes. Thus, the flow is maintained in the same UPF to AS proxy context. When the E2E CID changes, the more efficient QUIC aware forwarding has to fall back to encapsulation for as long as UPF determines the new CID from the encapsulated E2E packets and then re-establishes new QUIC aware forwarding rules for the detected CID. Detection of the new CID depends on the type of change and it may take longer if CID length changes e.g. if it gets smaller. This change may in addition impact the effective MTU that the E2E flow sees during the flow live. The overhead of tunnel encapsulation is different (higher) compared to when the QUIC aware forwarding transform for XRM is applied to secure and carry the MRI and authentication information.

Observation 1: E2E CID changes and especially length changes for the CID lead to UPF leaving the preferred Forwarded Mode to fallback to tunneled mode, which is heavier and may have impacts on the effective MTU.

Observation 2: When Forwarded Mode is used, CID changes should be avoided unless the UPF can be prepared in advance with the knowledge of what CIDs will be used in the end-to-end QUIC connection and UPF and AS proxy can stay in Forwarded Mode.

The second aspect from the LS response is specific to potentially varying MTU seen end-to-end. In cases where the end-to-end QUIC connection implements Datagram Packetization Layer Path MTU discovery (DPLPMTUD) there is some risk that the mechanism itself combined with fluctuations of the effective MTU can cause some packet loss when packets over the current effective MTU will be dropped. Also varying MTU can cause the DPLPMTUD to use conservative measurement reducing efficiency.

Observation 3: The methods to transfer the XRM MRI between AS and UPF impact the end-to-end MTU and may cause data transmission inefficiencies.

Observation 4: Awareness of the overhead can empower the Application client (AC) to effectively determine how that impacts MTU.

By introducing direct signaling between the Application client and the UPF, it is possible to address the observations described above as follows:

-  First, by informing the application (UE side) of the E2E QUIC connection that changing CID length should not be performed

-   Secondly providing the UPF with the E2E QUIC connections destination CIDs such that QUIC aware forwarding rules can be in place already before any CID changes

-   Third, providing the amount of packet expansion currently being applied, as well as the maximum expected to ensure that the E2E QUIC connection can avoid, in critical phases like connection establishment, sending oversized packets. In addition, this information can be used to optimize any DPLPMTUD.

Observation 5: it has been described how signalling between AC and UPF can be used to to improve further the E2E performace when Connect-UDP method with Forwarded Mode is used for on-path N6 signalling.

In the XRM Phase 3 scope being discussed, WT-1 (“Enhancements to UDP and QUIC-aware proxying using HTTP”) proposes in WT-1.1 to study whether and how to allow the proxy functionality to be located optionally also in the UPF in addition to the AS side, enabling the Application Client (AC) to connect to and establish a secure channel with the UPF for bi-directional UE-NW information exchange.

The signalling described in Observation 5 can be enabled by a proxy functionality placed in UPF so that the Application Client (AC) can connect and establish a secure channel with the UPF for bi-directional transfer of transport related information. Since the traffic is carried over QUIC (a prerequisite for the Forwarded Mode) also Connect-UDP Forwarded Mode can be used between UE and UPF, and so neither full reencapsulation nor reencrytion are required.

Conclusion: The E2E performance can be further enhanced if the Connect-UDP Forwarded Mode for on-Path N6 signalling is complemented with signalling between Application client (AC) and UPF (see observation 5) enabled by a proxy functionality in the UPF (as proposed in WT-1.1) that also supports Connect-UDP method with Forwarded mode.

# 3. Proposal

Based on the Conclusion above the proposal is to manage WT-1.1 as part of Rel-20.