3GPP TSG-WG SA2 Meeting #168	S2-2503607

# Goteborg, SE, 7th April – 11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1551 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarificaiton on supporting multiple non-3GPP device identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the C1-250822, it is not clear that: - whether the multiple non-3GPP device identifiers are allowed in a single PDU session modification procedure - whether the SMF rejects the Non-3GPP Device Identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message - whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message when more than one non-3GPP device identifier are removed from the subscription. And only a single non-3GPP Device Identifier is allowed in one message may lead to increase signalling overhead within the system if the UE/5G-RG needs to request QoS differentiation for multiple devices in a short time. CT1 also checked that it is allowed the SMF can provide multiple non-3GPP device identifiers to the PCF in one message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the clarification on supporting multiple non-3GPP Device Identifiers in PDU SESSION MODIFICATION message. - Clarify that the unavailable non-3GPP Device Identifiers are permitted in PDU Session Modification Command message when there is unavailable non-3GPP device identifiers in the PDU Session modification request or PCF deletes the PCC rules related to the non-3GPP device identifiers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no consensus in CT1 based on the existing description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3.31	Policy control for non-3GPP devices connecting behind a UE

The support of QoS differentiation for non-3GPP devices connecting behind a UE using a Non-3GPP Device Identifier is specified in clause 5.52 of TS 23.501 [2].

The SMF may receive from the UE the Non-3GPP Device Identifier and the corresponding user plane address if the UE initiated the PDU session modification procedure, as specified in clause 4.3.3.2 of TS 23.502 [3]. The SMF sends the Non-3GPP Device Identifier and the user plane address of the non-3GPP device to the PCF. At reception of the Non-3GPP Device Identifier and user plane address, the PCF creates PCC rule(s) considering the Non-3GPP Device Identifier. The PCF determines QoS parameters in the PCC rule(s) based on the Non-3GPP Device Identifier Information retrieved from the UDR and/or based on operator policy. The PCF installs the PCC rules at the SMF to enable QoS handling of the traffic to/from the non-3GPP device behind the UE.

If the SMF receives from the UE multiple non-3GPP devices in one PDU Session Modification Request, the SMF sends multiple Non-3GPP Device Identifier(s) and their corresponding user plane address(es) to the PCF.

If there is Non-3GPP Device Identifier(s) which is not present within the Non-3GPP Device Identifier Information as specified in clause 5.2.12.2.1 of TS 23.502 [3] for the UE, then the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE. In case of PDU Session Modification (i.e. SMF contacted PCF via SM policy association modification), the PCF shall reject SM Policy Association Modification and based on the rejection received, SMF shall reject the whole PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier is not available for the UE and the unavailable Non-3GPP Device Identifier(s).

If the Non-3GPP Device Identifier is removed from the UE subscription, the UDR notifies to the PCF via Nudr_DM_Notify service of removal of N3GPP device (as defined in clauses 4.15.6.7.2 and 4.15.6.15 of TS 23.502 [3]). If there are PCC rules associated to the traffic associated to the removed Non-3GPP Device Identifier in a PDU session, the PCF shall remove the PCC rules and start a PCF initiated PDU session modification including a cause indicating that the Non-3GPP Device Identifier is not available and the unavailable Non-3GPP Device Identifier. Multiple unavailable Non-3GPP Device Identifiers are included for the scenario where PCC rules need to be removed because the removal of multiple non-3GPP Device Identifiers.

# * * * * End of changes * * * *