3GPP SA WG2# 168	S2-2503715

Goteborg, SE, April 7 – 11, 2025	(revision of xx-yyxxxx)

{Guidance text shown in curly brackets, in italics. All guidance text is to be deleted before WID submission.

Note that this form is to be used for all types of Work Item, covering both normative work and/or studies. A Work Item covering only studies is also known as "Study Item", as stated in TR 21.900.}

# Source:	NEC

# Title:	New SID on Study on Indirect Network Sharing phase2

{"Revised" to be used only for WID already approved at plenary. For Revised WIDs, two versions have to be provided in a zip file: a clean one and one with revision marks showing the differences with the previously plenary-approved WID.}

# Document for:	Agreement

# Agenda Item:	30.8

Abstract: This is SI-1 as per moderator report in S2-2501576, based on initial WTs according to SA plenary outcome. The document needs to be updated based on any agreements reached during the meeting.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Indirect Network Sharing phase2

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	FS_NetShare_ph2-ARC

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

{ Replace XX by the intended Release, e.g. Rel-19.  Note that this field indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan}

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X |  | X |  | X |
| Don't know |  | X |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

{Tick one or more box(es). The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI19_NetShare | SA2 | 1030011 | Indirect Network Sharing |
| NetShare_Ph2 | SA1 | 1050106 | Indirect Network Sharing Phase 2 |
| INS_SAT | SA1 | 1050105 | Indirect Network Sharing on Satellite Access Network |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

# 3	Justification

WT#1: R19 leftover

When the operator deploys the Indirect Network Sharing currently, if the shared RAN broadcasts the HPLMN ID or EHPLMN ID of UE of participating operator, the serving PLMN ID in the message which is sent to the NF of the participating operator network is set to the HPLMN ID or EHPLMN ID. Therefore, the NF of the participating operator network may not identify the INS case and the hosting operator network. It will cause that some PLMN related subscription data (e.g. mobility restriction parameter) may not be retrieved by the hosting operator network accurately and the PLMN specific policy (e.g. session management policy, charging policy) may not be performed correctly. The above issue will also happen if more than one hosting operator correspond to one participating operator and the same PLMN ID broadcast by the shared RAN of these hosting operators for the participating operator.

WT#2: INS for disaster condition

With the development and deployment of 5G network, the operator has paid more and more attention to the robustness of network, e.g. how to minimize the service interruption when the network meets some unexpected disaster condition.

The existing mechanism to solve the network failure for disaster condition is disaster roaming as specified in MINT feature in R17. This mechanism needs the tight coordination between the network and UE, the UE is required to support the disaster roaming (e.g. UE needs to support lots of indicators sent from core network and supports to bypass the list of forbidden PLMNs). However, most of the existing UEs do not support these functionalities based on the factual situation from the operator network, it causes the MINT mechanism cannot deploy quickly. And the MINT limits the NG-RAN nodes with the disaster condition only. However, the case that the 5GC broke down may happen as well. How to resolve this case needs to be defined considering this case also needs the interconnection between the operators.

For these reasons, the Indirect Network Sharing mechanism for disaster condition considering the factual situation of legacy UEs needs to be studied. As shown in the figure below, if the OP1’s RAN or OP1’s core network or both broke down, the UE of OP1 can access the network via OP2’s network using the mechanism of Indirect Network Sharing for disaster condition.

Editor's note:	Based on the moderated discussion, the further discussion is necessary to make the INS for disaster condition clearer.

WT#3: Satellite access network sharing

The satellite network and the terrestrial network can effectively complement each other in scenarios where they are inaccessible in their respective areas. As the Indirect Network Sharing case, the satellite access network can be shared by the participating terrestrial network operator without direct interface between the shared satellite access network and participating terrestrial operator’s core network.

Editor's note:	Based on the moderated discussion, it needs to clarify the SA2 gaps on the satellite access network sharing via INS.

WT#4: Accessing service over non-3GPP access

In rel-19, there is a limitation that the UE cannot get a service over non-3GPP access when the UE is registering or registered over 3GPP access to the participating network. In case the user is accessing the services over non-3GPP access, and the UE starts registering to participating network over 3GPP access then the service over non-3GPP access will not be continued. This will seriously affect the user experience.

# 4	Objective

Draft objectives taking into account different views (not consensus):

WT#1: Investigate the enhancement on how to enable the NFs of the participating operator’s network to detect the Indirect Network Sharing case and identify the hosting operator’s network.

WT#2: Investigate the enhancement on how to utilize Indirect Network Sharing mechanism for supporting a variety of disaster scenarios.

WT#2.1: Investigation on how to prevent the high network load condition when the UEs of participating PLMN with the disaster condition attempt to access the hosting PLMN(s).

WT#2.2: Investigation on whether and how the existing procedure (e.g. registration procedure, PDU session related procedures) could be enhanced to support the Indirect Network Sharing for disaster condition.

WT#2.3: Investigation on how to handle when a disaster condition is no longer applicable.

Editor's note:	The further clarification or input are necessary to make the WT-2 clearer.

Editor's note:	It needs to clarify the SA2 gaps on the satellite access network sharing via INS (WT#3).

WT#4 How to support simultaneous services over 3GPP access and non-3GPP access in indirect network sharing case.

Discuss the content of the WTs and justification and further revise the objective and then determine the TU budget.

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text, e.g. "This TS covers Stage 2" or "This TS covers Stage 3" or "This TS covers both stages 2 and 3"} |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

# 7	Work item leadership

SA WG2

# 8	Aspects that involve other WGs

{This information is provided as best effort assumption, at the time of submission of the WID to TSG approval. It can be later changed without a need to revise the WID.

The “aspects” can be provided by topic (e.g. “security”, “multimedia”) and/or by specifying the WG(s) e.g.: "SA2, SA3, SA5, SA6. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet"}

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: {possible values: "Not applicable", " unknown", "CT WGs", etc}

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name