3GPP TSG-SA2 Meeting #168	S2-2503799

Goteborg, Apr 7th – 11th 2025

Source:	Huawei, HiSilicon

Title:	Architecture assumptions of Sensing study

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposed the architecture for the study.

# 1.	Introduction

The study on architecture related aspects includes the network functions and the functionalities for sensing, and the interfaces introduced/enhanced for sensing purpose.

Prioritized discussion by SA2 in Q2 includes the followings:

-	5GC NF involved for the sensing. For example, whether a dedicated Sensing Function in 5GC should be introduced, and whether the separation between the control plane and user plane should be considered.

-	Interfaces between 5GC NF (sensing entities) and the AF (service consumer), and the possible involved NFs.

The document proposes to address the above-mentioned issue as follows:

-	A Dedicated 5GC NF, i.e., Sensing Function is introduced, to integrate the necessarily separated functionalities in 5GC. Possible functionalities of SF including sensing result calculation, downstream node selection, authorizing the parameters for the sensing service etc. This is due to the reason that 1) having a dedicated SF is future-proof, and 2) sensing is the service that not related to any existing services (e.g., LCS) supported by 5GC.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[x2]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[x3]	3GPP TS 22.137: " Service requirements for Integrated Sensing and Communication; Stage 1".

# * * * * Second change * * * *

# 4	Architectural assumptions and principles

## 4.1	Architectural assumptions

The study should be based on the following architecture assumptions:

-	Solutions shall be built on the 5G System architectural principles as in TS 23.501 [x1], including new functionalities.

-	Application Function can invoke the sensing services provided by 5GC directly or via NEF. The application client hosted in the UE can request and receive Sensing Result to the Application Function. The interaction between the application client and the Application Function is outside the scope of this study.

-	Only the 3GPP Sensing Data is in the scope of this study.

-	The sensing architecture and procedures shall be able to work independently by location architecture and procedures defined in TS 23.273 [x2].

Editor's Note: The Sensing modes to be supported should be determined at TSG#108.

## 4.2	Architecture requirements

Solutions of the study should provide architecture and system level enhancements to the 5G system to support sensing to satisfy the normative requirements specified in TS 22.137 [x3]. Specifically:

-	Configuration parameters/policy provisioning for the support of sensing services;

-	Authorization and revocation sensing service requests from the Sensing Service Consumer;

-	Discovering and selection of entities involved in sensing;

-	Collection and transfer of 3GPP Sensing Data for Sensing Result calculation;

-	Provision of information associated to sensing and Sensing Result(s) exposure, for one time, periodic or event-based reporting;

## 4.3	Architecture baseline

### 4.3.1	General

Figure 4.3-1 provides the reference architecture baseline for sensing.

NOTE 1:	Whether some extra NFs in the figure are needed, it is dependent on solution and captured in related solutions.

Figure 4.3.1-1: Reference Architecture.

Editor's note: The interface(s) between Sensing Tx/Rx and 5GC is FFS.

### 4.3.2	Functional entities

NOTE:	The enhancements of network functions can be refined based on the conclusion.

#### 4.3.2.1	Sensing Function

The Sensing Function performs the following functions to support Sensing:

For the functionalities of sensing service control:

-	Interaction with NEF for exposure of Sensing Results and reception of sensing requests.

-	Interaction with downstream sensing entities for requesting to perform sensing measurement.

For the functionalities of sensing data collection and processing:

-	Reception of 3GPP Sensing Data;

-	Determine the Sensing Result based on the 3GPP Sensing data.

4.3.2.2	NEF

The NEF performs the following functions to support Sensing:

-	Selection of Sensing Function.

-	Interaction with AF and Sensing Function for sensing authorization, invocation and revocation.

-	Provision the Sensing Result retrieved from Sensing Function to AF.

#### 4.3.2.3	AF

The AF requests sensing service, and AF does so by:

-	Interaction with 5GC for requesting sensing service to obtain the Sensing Result(s).

-	Acquisition Sensing Result(s) from 5GC.

# * * * * End of change * * * *