3GPP TSG-WG SA2 Meeting #168	S2-2503843

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on RAN Procedure Interactions

Document for:	Discussion

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Outline of the need and uses for keeping the CN and RAN aligned for when a procedure ends and the ability to stop the procedure.

1. Introduction

When an AIOTF performs an operation, either an inventory operation, or an inventory+command operation, the NG-RAN would need the ability to inform each other when an operation is completed and for the AIOTF to request that the NG-RAN completes the operation.

This discussion paper provides background on the different operations and procedures needed for each inventory or inventory+command case and the interactions for them as described above.

2. Discussion

2.1 AIOTF – NG-RAN Interactions Overview

Several cases are examined between an AIOTF and one or more NG-RAN nodes:

-	Procedure Completion/Ending: In this case procedure, whether an inventory or command+inventory case, the AIOTF informs the NG-RAN node that the procedure is complete, and no further interactions between the AIOTF and NG-RAN node will be performed. The termination may occur at any point.

-	NG-RAN Completing Inventory Procedure: In this case the inventory procedure is completed by the gNB and it notifies the AIOTF it is complete. The inventory may be done for the inventory only case or as part of the command+inventory case.

-	Stopping Inventory as part of command+inventory procedure:

The ability to complete/end a procedure allows the NG-RAN node and AIOTF to be synchronised about their states and expectations of each other, which is required. When a procedure is ended “early” while it is ongoing the NG-RAN can save resources (e.g. airtime) and start any other procedures it has been requested to perform.

Each of the above cases are expected to use a single Correlation ID, i.e. the interactions are related to a single overall request an AIOTF has received.

With the AIOTFs ability to selecting multiple NG-RAN nodes for a request it receives and the ability to known when a procedure has completed within a single NG-RAN node then the ability to terminate an ongoing procedure becomes more important. While this does not mean that we need to update the TS to describe the interactions with multiple NG-RAN nodes, it is important to consider them nevertheless, therefore, in addition to the single NG-RAN cases. An overall case involving 2 NG-RAN nodes is also shown to provide a full context.

In all the example high-level flows shown, the Naiotf requests, responses and notifications are not shown, but it can easily be determined when the occur.

It is also therefore easy to determine that similar information needs to be provided in notification events (e.g., end of inventory results etc), so the caller NF can determine not to expect any further results. Examples of this have not been illustrated for brevity.

2.2	Procedure Completion/Ending

When the AIOTF determines that a procedure has completed for a NG-RAN node it can inform that NG-RAN node it is complete and, for example, the NG-RAN node can then release any resources and discard any state that it may hold related to the procedure. The procedure is always identified by the AIOTF assigned correlation ID, as described in the TS, and therefore once a procedure is terminated the AIOTF will not send any more commands to the NG-RAN node related to the correlation ID, and the NG-RAN node will not expect any further commands.

The same end procedure can be used whether the NG-RAN node is performing inventory or inventory+command, as the actions from the NG-RAN node are the same.

This procedure is expected to be used when an NG-RAN node indicates the completion of inventory in an inventory only procedure. The AIOTF may also use it when it has all the inventory results it needs from an NG-RAN node or to signal the end of a command+inventory procedure to the NG-RAN node.

The signalling the completion of the command+inventory procedure is essential, as the NG-RAN node cannot know whether the AIOTF will send commands for the reported AIoT Devices.

This becomes especially useful when the AIOTF has selected multiple NG-RAN nodes and depending on the operation has determined, for example, any further inventory results from a specific NG-RAN node are no longer needed (e.g. all the required inventoried AIoT Devices have been reported by other NG-RAN nodes) or it has determined no further commands will be sent to devices reported by that NG-RAN node (i.e. not all the reported devices will be sent commands).

Figure 2.2-1: Inventory End

In the high-level flow in Figure 2.2-1, the AIOTF requests an inventory procedure (steps 1 & 2), the NG-RAN sends zero or more inventory reports (step 3) depending upon the AIoT Devices that reply to the paging. The AIOTF performs the end procedure (step 4) and the NG-RAN stops performing inventory. If the inventory has not completed in RAN on the air interface, NG-RAN stops the procedure, and if the procedure has already completed in the NG-RAN node on the air interface, sometime may have passed before the AIOTF send the Service End and the resources are released.

Figure 2.2-2: Command Completion

In the high-level flow in Figure 2.2-2, the AIOTF requests an inventory procedure for the inventory+command procedure (steps 1 & 2), the NG-RAN sends zero or more inventory reports (step 3) depending upon the AIoT Devices that reply to the paging. In steps 4&5 the AIOTF sends commands to zero or more of the AIoT Devices reported in step 3. When the commands have completed, the AIOTF performs the Service End procedure (step 6). If the Service End is performed before the command or inventory has completed on the air interface then NG-RAN stops the procedure on the air interface as well.

Figure 2.2-3: Command Completion, no devices found.

In the high-level flow in Figure 2.2-3, the AIOTF requests an inventory procedure for the inventory+command procedure (steps 1 & 2), the NG-RAN sends zero inventory reports and therefore the AIOTF does not send any commands. The AIOTF performs the Service End procedure (step 3). If the end is performed before the inventory part has completed on the air interface, then NG-RAN can stop the procedure on the air interface as well.

As already mentioned, the AIOTF may determine to end the procedure, when for example, it has located all the AIoT Devices it wishes to on other NG-RAN nodes.

2.3	NG-RAN Completing Inventory Procedure

When the inventory procedure completes in an NG-RAN node it needs to inform the ATIOF that the procedure has completed. This enables the AIOTF to know that no further results are expected from that NG-RAN node, and therefore the procedure can be completed, including the cases when the AIOTF selected multiple NG-RAN nodes.

To enable this a message from the NG-RAN node is needed. Then based on this, following existing multiple other examples for different types of CN/RAN interaction, the AIOTF can instruct that NG-RAN node that that the procedure has ended and it should, for example, clean-up/discard state/information related to the requested operation.

Figure 2.3-1: Example Inventory Complete in Inventory procedure

In the high-level flow in Figure 2.3-1, the AIOTF requests and inventory procedure (steps 1 & 2), the NG-RAN node sends zero or more inventory reports (step 3) depending upon the AIoT Devices that reply to the paging. In step 4 the NG-RAN node indicates that the inventory procedure is complete, and therefore the AIOTF knows when to perform the end procedure (step 5) as per clause 2.2, and in this case the elapsed time after RAN has completed the procedure and the Service End is minimised compared to clause 2.2.

Figure 2.3-2: Example Command Complete in inventory+command procedure

In the high-level flow in Figure 2.3-2, the AIOTF requests an inventory procedure for the inventory+command procedure (steps 1 & 2), the NG-RAN node sends zero or more inventory reports (step 3) depending upon the AIoT Devices that reply to the paging. In step 4 the NG-RAN node indicates that the inventory procedure is complete, and therefore the AIOTF knows not to expect any further results from NG-RAN. In steps 5&6 the AIOTF sends commands to zero or more of the devices reported in step 3. When the commands have completed, the AIOTF performs the Service End procedure (step 7) as per clause 2.2.

Figure 2.3-3: Example Command Complete in inventory+command with no Inventory Reports

In the high-level flow in Figure 2.3-3, the AIOTF requests an inventory procedure for the inventory+command procedure (steps 1 & 2), and in step 3 the NG-RAN indicates that the inventory procedure is complete (no inventory reports have been sent), and therefore the AIOTF knows not to expect any results from NG-RAN. The AIOTF performs the Service End procedure in step 4 as per clause 2.2. In this case with the inventory complete event the AIOTF knows the NG-RAN has completed the inventory phase, and therefore there will be no inventory reports.

2.4	Stopping Inventory during inventory+command procedure

When performing the inventory “phase” of the inventory+command procedure, the AIOTF may determine that no further inventory results are required from a specific NG-RAN Node and send commands to the AIoT Devices reported by the NG-RAN node (i.e., move onto the command “phase”).

To support this an interaction is required between the AIOTF and an NG-RAN node to stop the inventory “phase” or part of the procedure and move onto the command “phase” or part of the procedure.

Figure 2.4-1: Stopping inventory part of inventory+command procedure

In the high-level flow in Figure 2.4-1, the AIOTF requests an inventory procedure for the inventory+command procedure (steps 1 & 2), the NG-RAN node sends one or more inventory reports (step 3) depending upon the AIoT Devices that reply to the paging.

In step 4 the AIOTF stops the inventory part of the inventory+command procedure. This could be based on the devices found by this NG-RAN node or others, with the final result that all the devices of interest to send commands to have been reported. To allow continuity with the procedure as a whole, it may be beneficial for the NG-RAN to report inventory complete (step 5).

Similar to the previous flows, in steps 6&7 the AIOTF sends commands to zero or more of the devices reported in step 3. When the commands have completed, the AIOTF performs the Service End procedure (step 8) as per clause 2.2.

2.5 Multiple NG-RAN nodes example

In this example, many of the above interactions are used to show how they can and will be used in certain cases.

Figure 2.5-1: Example over 2 NG-RAN nodes

The example in figure 2.5-1, start inventory towards NG RAN node 1 and NG-RAN node 2 (steps 1, 2, 4, 5) and the NG- RAN Nodes report results in steps 3 & 6. In this example, let’s say, step 6 report allows that AIOTF to determine that no further inventory reports are expect or wanted from any of the NG-RAN nodes and therefore triggers the stop inventory procedure with NG-RAN 2 (steps 8 &), while at a similar time NG-RAN 1 completes the inventory naturally (step 10). Commands are sent to the responding devices (steps 11, 12, 13, 14) and the procedure ended (steps 15 & 16) with both NG-RAN nodes.

Although the example shows a very “linear” flow, in reality, as the NG-RAN nodes are not coordinated, the events they generate can occur at any time, relative to the other NG-RAN node (only the sequence per RAN node needs to be fulfilled).

3. Conclusion and proposal(s)

It can be seen that there is a need to ensure that an NG-RAN node and the AIOTF know then the end of a procedure is reached.