3GPP TSG-WG SA2 Meeting #168	S2-2504119

# Goteborg, Sweden, April 07 – 11, 2025		(revision of S2-2503141)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0548 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Multi-hop U2N and U2U Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In clause 6.3.2.5.3 step3, "…drop the received Solicitation message if the hop count… has reached the Hop-Limit of the received Solicitation message or the (pre)configured maximum number of hops associated with the RSC". As the Hop-limit is optional, it is needed to clarify the conditions before and after the "or". |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the condition for Relay dropping the Solicitation is determined by whether Hop-Limit is included in the Solicitation. - Some textual improvements in discovery procedures and related communication procedures to reduce ambiguity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment and not clear enough for multi-hop U2N/U2U discovery procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.3, 6.3.2.6.3.2, 6.4.3.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.3.2.5.3	Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B

Figure 6.3.2.5.3-1: General Procedures for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE determines the Hop-Limit for discovery based on policy configuration (i.e. a mapping between maximum number of hops and RSC) or QoS requirements.

If the Hop-Limit is determined based on configuration associated with the RSC, the 5G ProSe Remote UE does not include the Hop-Limit in the Solicitation message. Otherwise, the 5G ProSe Remote UE includes the Hop-Limit in the Solicitation message.

2a.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Relay Discovery Solicitation message additionally contains following IEs compared with that in clause 6.3.2.3.3: an indication that multi-hop relay is supported, Hop-Count and Hop-Limit (optional).

The Target Info may contain the User Info ID of the UE-to-Network Relay and Intermediate UE-to-Network Relay(s).

3a.	If an indication that multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe Intermediate UE-to-Network Relay (if any), the 5G ProSe Intermediate UE-to-Network Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message .

The 5G ProSe Intermediate UE-to-Network Relay shall drop the received Solicitation message when its own User Info ID is contained in the Path information of received Solicitation message, or if the hop count (corresponding to the number of Relays included in the message) has reached the Hop-Limit (if included in  the received Solicitation message) or the (pre)configured maximum number of hops associated with the RSC (if no Hop-Limit is included in the received Solicitation message).

The 5G ProSe Intermediate UE-to-Network Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), e.g. based on the unicast link profiles, without sending Solicitation message. i.e. steps 4a-7a are skipped and step 8a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate UE-to-Network Relay(s), optionally hop count and optionally Accumulated QoS for PC5 link, as described in clause 5.8.3.1.

If the same information on User Info IDs of Remote UE and UE-to-Network Relay is received from different ProSe UEs, the 5G ProSe Intermediate UE-to-Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.).

4a.	A 5G ProSe Intermediate UE-to-Network Relay sends the Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID and Layer-2 ID of Relays in the path that has relayed the Solicitation message. The hop count is increased by 1. The solicitation message may include Accumulated QoS for PC5 link (e.g. sum of delay of each hop).

The Accumulated QoS for PC5 link, if present in the received solicitation message, will be updated to include the QoS of the PC5 link between the 5G ProSe Intermediate Relay and its child UE (Intermediate Relay or Remote UE).

2b.-5b.	The Solicitation message from the same Remote UE goes through a different ordered list of 5G ProSe Intermediate UE-to-Network Relays.

6-7. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of the 5G ProSe UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe UE-to-Network Relay (if any), then the 5G ProSe UE-to-Network Relay responds to the 5G ProSe Intermediate UE-to-Network Relay with a 5G ProSe UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to- Network Relay Discovery Response message additionally contains the path information compared with that in clause 6.3.2.3.3. The response message may include the received Accumulated QoS for PC5 link.

The 5G ProSe UE-to-Network Relay may choose the path based on e.g. the PC5 signal strength of each message received, hops to the Remote UE, the path information, Accumulated QoS for PC5 link, etc.

NOTE 1:	It is up to implementation how the 5G ProSe UE-to-Network Relay decides when to send the Relay Discovery Response message.

8-9.	When a 5G ProSe UE-to-Network Relay Discovery Response message is received, if the User Info ID of the receiving 5G ProSe Intermediate UE-to-Network Relay is included in the path information, the 5G ProSe Intermediate UE-to-Network Relay forwards the 5G ProSe UE-to-Network Relay Discovery Response message. The Response message additionally contains the path information which is a list of User Info IDs of 5G ProSe Intermediate UE-to-Network Relay(s) and the remote UE in the order (starting with the remote UE and ending with the last 5G ProSe Intermediate UE-to-Network Relay) of position in the path. Based on the path information, the Relay Discovery Response message is forwarded along the path, where each 5G ProSe Intermediate UE-to-Network Relay determines the next hop information (e.g. User Info ID and Layer-2 ID) to forward the Relay Discovery Response message. The response message may include the received Accumulated QoS for PC5 link.

10.	If the Remote UE receives multiple Discovery Response messages within a pre-configured time window, it may perform relay path selection based on e.g. path information, the PC5 signal strength between Remote UE and its neighbour Intermediate UE-to-Network Relay, Accumuled QoS for PC5 link and the number of hops to the 5G ProSe UE-to-Network Relay.

NOTE 2:	If the 5G ProSe Remote UE does not receive any response after a pre-configured time, based on application requirement, it may increase the Hop-Limit and send the discovery message again.

# * * * * Second change * * * *

6.3.2.6.3.2	Procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B

Depicted in Figure 6.3.2.6.3.2-1 is the procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B.

Figure 6.3.2.6.3.2-1: Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B

1.	The Source End UE determines the Hop-Limit for discovery based on policy configuration (i.e. a mapping between Hop-Limit and RSC) or QoS parameters.

If the Hop-Limit is determined based on RSC, the Source End UE may not include the Hop-Limit in the Solicitation message. Otherwise, the Source End UE includes the Hop-Limit in the Solicitation message.

2a.	The Source End UE sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the information as described in clause 5.8.6.3.

3a.	If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s) of a 5G ProSe UE-to-UE Relay, the 5G ProSe UE-to-UE Relay may decide to send a 5G ProSe UE-to-UE Relay Discovery Solicitation message .

The 5G ProSe UE-to-UE Relay should drop the received Solicitation message when its own User Info ID is contained in the Path information of received Solicitation message, or if the hop count (i.e. the number of Relays included in the message) has reached the Hop-Limit of the received Solicitation message. The Hop-Limit may be obtained from the Solicitation message or may be decided based on RSC if it is not obtained from the message.

If the same Direct Discovery Set is received from different 5G ProSe-enabled UEs, the 5G ProSe UE-to-UE Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.).

NOTE 1:	If the Source End UE does not receive any response after a timeout, based on application requirement, it may increase the Hop-Limit and send the discovery message again.

NOTE 2:	Based on implementation, Relay UEs can decide to send the discovery message with the same direct discovery set at most once in order to control the total number of discovery messages transferred among Relays. To determine the identical direct discovery set, the Relay UE can compare the direct discovery set bit by bit if it is encrypted, or check the source End UE and target End UE User info if the direct discovery set is not encrypted.

4a.	A 5G ProSe UE-to-UE Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID of Relays in the path that has sent the Solicitation message. The hop count is increased by 1.

2b.-5b.	(Optional) Another transmission path of the Solicitation message(s).

6-7a. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s) of the Target 5G ProSe End UE and the Target 5G ProSe End UE matches the user info (i.e. Application Layer ID) of the discoveree 5G ProSe End UE contained in the solicitation message, then the Target 5G ProSe End UE responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the information as described in clause 5.8.6.3. The 5G ProSe UE-to-UE Relay decides the information (e.g. User Info ID, Layer-2 ID) to unicast the Relay Discovery Response message based on its own User Info ID and the path information contained in the received message.

The Target 5G ProSe End UE may choose a path and sends Response message to 5G ProSe UE-to-UE Relay(s) corresponding to the chosen path based on e.g. the PC5 signal strength of each solicitation message received, the number of hops to the Source End UE, the path information, order in which solicitation message were received, etc.

8a.	A 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message to the Source End UE. A 5G ProSe UE-to-UE Relay may associate the User Info ID and Layer-2 ID of neighbour UE-to-UE Relay(s) according to the Response message. The association may be used in the subsequent Link Management procedures.

7b.-9b.	(Optional) Transmission of another Response message corresponding to another path.

10.	The Source End UE may perform the UE-to-UE Relay(s)/path selection for subsequent Link Management procedures based on e.g. the PC5 signal strength of the received Response messages (from one or multiple Relays), the number of hops to the Target 5G ProSe End UE.

# * * * * Third change * * * *

##### 6.4.3.10.2	Layer-2 link establishement and management over PC5 reference point for non-IP PDU type

For the 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.7.1a, the description in clause 6.4.3.7.1 applies with following differences and clarifications:

-	The Direct Communication Request message over the PC5 reference point between source End UE and the first hop UE-to-UE Relay additionally includes:

-	Path Information: an (ordered) list of User Info ID of 5G ProSe UE-to-UE Relays selected by the source 5G ProSe End UE based on the path information provided by the first hop UE-to-UE Relay to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure.

-	QoS Info: indicates the End to End QoS Info from the source End UE to the target End UE.

The Direct Communication Request message over the PC5 reference point between two UE-to-UE Relays includes:

-	User Info of source 5G ProSe End UE.

-	User Info of target 5G ProSe End UE.

-	Path information.

-	ProSe Service Info: the information about the ProSe identifier(s).

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the PC5 link establishment between two UE-to-UE Relays (see TS 33.503 [29]).

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Direct Communication Request message over the PC5 reference point between between the target End UE and UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Direct Communication Accept message over the PC5 reference point between target End UE and UE-to-UE Relay additionally includes:

-	QoS Info which is the accepted QoS Info of the hop between target End UE and the last UE-to-UE Relay.

The Direct Communication Accept message over the PC5 reference point between two UE-to-UE Relays includes:

-	User Info of target 5G ProSe End UE.

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between two UE-to-UE Relays and the accumulated QoS Info of hops from the receiver UE-to-UE Relay to the target End UE.

The Direct Communication Accept message over the PC5 reference point between the source End UE and UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between two UE-to-UE Relays and the accumulated QoS Info of hops from the source End UE to the target End UE.

The Link Modification Request message over the PC5 reference point between source End UE and the first hop UE-to-UE Relay additionally includes:

-	Path Information: an (ordered) list of User Info ID of 5G ProSe UE-to-UE Relays provided by the first hop UE-to-UE Relay to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure.

-	QoS Info: indicates the End-to-End QoS Info from the source End UE to the target End UE.

The Link Modification Request message over the PC5 reference point between two UE-to-UE Relays includes:

-	User Info of source 5G ProSe End UE.

-	Path Information.

-	User Info of target 5G ProSe End UE.

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Link Modification Request message over the PC5 reference point between the target End UE and UE-to-UE Relay additionally includes:

-	Path Information.

-	QoS Info: indicates the rest QoS Info from the UE-to-UE Relay to the target End UE.

The Link Modification Accept message over the PC5 reference point between the target End UE and UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between target End UE and UE-to-UE Relay.

The Link Modification Accept message over the PC5 reference point between two UE-to-UE Relays includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between two UE-to-UE Relays and the accumulated QoS Info of hops from the UE-to-UE Relay to the target End UE.

The Link Modification Accept message over the PC5 reference point between source End UE and the first UE-to-UE Relay additionally includes:

-	Path information.

-	QoS Info which is the accepted QoS Info of the hop between source End UE and the first UE-to-UE Relay and the accumulated QoS Info of hops from the UE-to-UE Relay to the target End UE.

NOTE:	The path information in DCR/LMR/DCA/LMA message is used for multi-hop 5G ProSe UE-to-UE Relay Discovery only. A 5G ProSe UE-to-UE Relay Discovery that only supports single-hop UE-to-UE Relay will ignore the messages that contains path information.

In the Security Procedure of the PC5 reference point between source End UE and UE-to-UE Relay, the source 5G ProSe Layer-3 End UE provides the QoS Info of the end-to-end QoS to the 5G ProSe Layer-3 UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the source 5G ProSe Layer-3 End UE provides its Ethernet MAC address. The 5G ProSe Layer-3 UE-to-UE Relay may provide the Ethernet MAC address of the target 5G ProSe Layer-3 End UE in the DCA/LMA message.

In the Security Procedure of the PC5 reference point between two UE-to-UE Relays, a 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info from itself to the target 5G ProSe Layer-3 UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE. The 5G ProSe Layer-3 UE-to-UE Relay may provide the Ethernet MAC address of the target 5G ProSe Layer-3 End UE in the DCA/LMA message

In the Security Procedure of the PC5 reference point between target End UE and a UE-to-UE Relay, the 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info of the last hop QoS to the target 5G ProSe End UE. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE. The target 5G End UE may provide its Ethernet MAC address in the DCA/LMA message

The 5G ProSe Layer-3 UE-to-UE Relay decides the QoS Info of the next hop to source End UE with considering the accumulated QoS Info of the hop(s) to the target End UE which is received in the Direct Communication Accept message.

For adding new end-to-end QoS flow or modifying existing end-to-end QoS flow, the Link Modification Accept message over each PC5 reference point additionally includes QoS Info of that hop. If the PC5 link is used for transferring Ethernet traffic, the target 5G ProSe Layer-3 End UE may provide its Ethernet MAC address. For a path between a specific pair of End UEs, if at least one of the PC5 link is released, the 5G ProSe Layer-3 UE-to-UE Relay(s) in the path may release other PC5 link(s) in the path.

# * * * * End of changes * * * *