3GPP TSG-SA WG2 Meeting #168	S2-2504309

Gothenburg, Sweden, 7th – 11th April 2025

Agenda item:	20.4.1

Source:	Nokia

Title:	KI#1: new solution: P-CR on exposure of UE scoring data

WI/Release:	EnergySys_Ph2 /Rel-20

Document for:	Approval

# 1	Discussion

The calculation of the energy consumption at EIF ultimately result in rel-19 specification in the exposure of absolute values of energy consumption. This is useful information, but it is also useful for the network and its customers to identify whether a UE is estimated to consume more or lass than a given percentile of the UEs that the EIF collects statistics of energy consumption about. The customer can benefit from this information to perhaps perform some root cause analysis of the energy consumption. Note that by receiving absolute values the consumer of the estimates from EIF has no means to evaluate the relative consumption of a UE vs the rest of the UE population, hence this has to be provided by the EIF by means of some sort of a scoring.

For this reason, we propose the EIF provides additional information which can be:

- Percentile of the consumption of the UE at the granularity of the estimate; or

- In which consumption band a UE is positioned (e.g. in the band of 30% to 40% of UEs)

It may also be beneficial to provide the data volume consumed which is at the basis of the estimate or a consumption per unit of data volume (so as to evaluate the efficiency in the reporting period).

We propose a Solution is added to the study to cover the aspects above.

# 2. Conclusion

It is proposed that the following text is added to TR 23.700-67

Proposed text

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| X | x |  |  |

Proposed text

# * * * * additional change * * * *

## 6.x	Solution#x:	Enhancement of Exposure API to include UE categorization data.

### 6.x.0	High-level solution principles

EIF to expose more information a consumer may request, for the same granularity in the same time window or periodically as the energy consumption data, to categorize the energy consumption compared to other UEs served by EIF and gather information on energy consumption per bit for the requested energy consumption estimates granularity.

The purpose can be information collection by the consumer for statistics or analysis purposes, or, if the energy consumption estimates are robust and accurate enough, to take e.g. service adjustment action based on operator policy on these exposed values.

NOTE: 	The value of this information increases with the increase in the robustness and accuracy of the energy consumption calculation estimation method used, which is the basis for the calculation for categorization and energy per bit estimations. Whether to provide categorization and energy consumption per bit information and for which purpose is for a PLMN to decide based on its internal or customer needs.

### 6.x.1	Description

This solution addresses Key Issue #1.

This solution proposes that a consumer may request in addition to energy consumption data in a time window or periodically, the EIF can provide also UE categorization data at the desired granularity including e.g.

- Percentile of the consumption of the UE at the granularity of the estimate; or

- In which consumption band a UE at the granularity of the estimate is positioned (e.g. in the band/category of 30% to 40% of UEs). The possible bands reported are defined based on PLMN policy.

It may also be beneficial to provide the data volume consumed which is at the basis of the estimate or a consumption per unit of data volume (so as to evaluate the efficiency). In this solution we propose to provide the energy consumption per bit at the specified granularity of energy consumption requested.

The consumer should be able to request to be reported this information alongside the energy consumption estimates.

It is proposed this is achieved by adding additional Event IDs to the Neif_EventExposure exposure Service.

NOTE: 	If there are more than 1 EIF in a PLMN, it is possible to consider the option to have an aggregation of the categorization information by some specialized EIF frontend. Otherwise the scope of the categorization is per EIF.

### 6.x.3	Procedures

There is no new procedure proposed. The Neif_EventExposure service can be modified so that the following new Event IDs are added:

| Event ID for EIF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Energy consumption percentile (or band/category) | <Parameter Type = UE level, Value = SUPI/GPSI> <Parameter Type = S-NSSAI per UE level, Value = SUPI/GPSI and S-NSSAI > <Parameter Type = PDU session level, Value = SUPI/GPSI and DNN/S-NSSAI > <Parameter Type = Application Identifier, Value = SUPI/GPSI, DNN/S-NSSAI and, Application identifier> <Parameter Type = Flow Info, Value = SUPI/GPSI, DNN/S-NSSAI and Flow description(s)> |
| Energy consumption per bit | <Parameter Type = UE level, Value = SUPI/GPSI> <Parameter Type = S-NSSAI per UE level, Value = SUPI/GPSI and S-NSSAI > <Parameter Type = PDU session level, Value = SUPI/GPSI and DNN/S-NSSAI > <Parameter Type = Application Identifier, Value = SUPI/GPSI, DNN/S-NSSAI and, Application identifier> <Parameter Type = Flow Info, Value = SUPI/GPSI, DNN/S-NSSAI and Flow description(s)> |

The Event IDs above are used for reporting of the related information at the granularity requested within the requested time window or periodically.

### 6.x.4	Impacts on existing services, entities and interfaces

EIF:

-	new categorization information to be provided upon request by consumers

-	support of the necessary analysis of consumption data to support providing new categorization information

Service consumer NF or AF:

-	ability to request new categorization information from EIF.

-	ability to receive new categorization information from EIF

# * * * * end of changes * * * *