3GPP TSG-SA WG2 Meeting #168	S2-2504344

Goteborg, Sweden, April 07 – 11, 2025

Source:	MediaTek Inc., vivo

Title:	Solution for WT#2: Support of QoS adjustments

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes solution to the key issue for WT#2 on Support of service adjustments for the UE.

* * * Start of Change * * * *

6.X	Key Issue #X: Support of QoS adjustments

6.X.0	High-level solution principles

The proposed solution is

-	to update the value of QoS rules, QoS profile and Alternative QoS profile by re-using the existing parameters. No new parameters are added based on the energy related information.

-	to enhance the SMF to be aware of the energy status of NG-RAN (e.g., whether the NG-RAN is using renewable energy or not) and then decide whether to update Alternative QoS profile or not

-	to allow SMF to update the QoS configuration based on the PCC rules from PCF which has interacted with EIF

NOTE 1:	The energy-related information at UE or NF level used in KI#2 and KI#3 should be based on robust and accurate enough estimates to be fit for purpose.

NOTE 2:	The usage of QoS rules to UE and QoS profiles to NG-RAN are the same as specified in TS 23.501 [y1], i.e., UE and NG-RAN are not required to take the energy criteria/attributes/characteristics into account to determine how to use QoS rules and QoS profile.

6.X.1	Description

This solution is to resolve the issue for the QoS adjustments for the UE based on the energy attributes and energy related information from OAM and EIF.

The QoS adjustments can be to update or configure QoS rules to the UE and QoS profile to the RAN by adjusting the value of the existing parameters to achieve the purpose of energy efficiency and/or network energy saving for the UE.

The QoS adjustments may also occur after the PCF interacts with EIF and may update the PCC rules related to QoS adjustments to the SMF and then the SMF updates the QoS rules and profile to the UE and/or to the RAN.

NOTE 1: How PCF interacts with EIF to update policy is not addressed in this solution. The PCC rules made by PCF are based on the operators policy or local configuration.

NOTE 2: The adjustment to QoS rules or to QoS profile may mean e.g., increasing or decreasing the value for QoS parameters such as GFBR or MFBR.

NOTE 3: After adjusting QoS parameters in QoS rules and QoS profile by SMF, it is possible that the SMF may trigger UP path adjustment to reselect a UPF which can meet the updated QoS parameters for the UE and for the RAN.

6.X.2	Architectural assumption

The solution is based on clauses 4.2.18 and 5.51 of TS 23.501 [y1] as a baseline to enhance the QoS adjustment to meet the energy efficiency and/or energy saving for the UE.

Furthermore, the existing QoS model is re-used as specified in clause 5.7 of TS 23.501 [y1].

6.X.3	QoS adjustment for the UE

In clause 5.7 of TS 23.501 [y1], the UE is provided with QoS rules via PDU Session establishment/modification procedure as specified in TS 23.502 [y2] to perform the classification and marking UL traffic based on the received QoS rules from the SMF. Based on the network architecture in Fig. 4.2.18-2 of TS 23.501 [y1], to achieve energy saving for the UE, the SMF may update the QoS rules or QoS profile or Alternative QoS profiles based on the PCC rules updated from PCF which has interacted with EIF. interact with EIF to obtain the energy related information (e.g., energy consumption of UE in a PDU Session, and/or a QoS flow granularity to determine how to update and/or generate the QoS rules for the UE.

In the current specification of clauses 5.51.2 to 5.51.4 of TS 23.501 [y1], they describe how the energy consumption value is derived for a UE in different granularities. However, those existing clauses do not specify whether the SMF can determine to update QoS rules based on the results after the interaction with EIF or the SMF obtains updated policy from the PCF interacting with EIF.

To achieve the QoS adjustment for the UE, the SMF may decide to perform to update QoS rules to the UE, or to update QoS profile to the RAN, and/or to perform the alternative QoS profile to the UE and RAN after the SMF interacts with EIF or the SMF receives the update policy from PCF.

Furthermore, the SMF may subscribe to the EIF for different granularities of the energy consumption value. Based on the changes obtained from EIF, the SMF may decide to update the QoS configurations to the UE and/or the RAN, and may further update the N4 rules to the UPF.

6.X.4	Procedures for QoS rules, QoS profiles and N4 rules update to the UE/RAN/UPF

6.X.4.1	QoS configurations to the UE/RAN/UPF

The PDU Session establishment procedure or modification procedure in clause 4.3.2 and clause 4.3.3 of TS 23.502 [y2] can be re-used.enhanced to add the service operations between SMF and EIF.

SMF may be triggered to perform PDU Session modification procedure to update the QoS rules or QoS profile to the UE and to the RAN as mentioned by step 1b in clause 4.3.3 of TS 23.502 [y2], PCF initiated SM policy Association Modification procedure in clause 4.16.5.2 to notify the policies changes after PCF interacts with EIF. At the same time, the SMF may also update the N4 rules to the UPF based on the updated QoS rules and QoS profiles

Editor’s note: How SMF is triggered to do QoS adjustment is FFS.

After interaction with EIF, the SMF may decide to update the QoS rules in PDU Session accept message to the UE and QoS profile to the RAN. At the same time, the SMF may also update the N4 rules for the UPF based on the Updated QoS rules

6.X.4.2	Alternative QoS profiles

The value of Alternative QoS profile can be also adjusted or updated based on the energy attributes/characteristics of RAN (e.g., whether the RAN is using renewable energy or not).

After interaction with EIF and with OAM, the SMF may get the energy attributes for RAN (e.g., whether the RAN is using renewable energy or not) and the energy related information for UE, then the SMF may decide to allocate or update a prioritized list of Alternative QoS Profile(s) to the NG-RAN.

Furthermore, the existing Notification control procedure needs to be enhanced as follows:

-	e.g., when the NG-RAN switches to use the renewable energy based on the availability information of the renewable energy from the OAM, the RAN may send a notification towards SMF for informing “the renewable energy- is applied/used” to notify the SMF, and the SMF may decide to update Alternative QoS profile based on the rules from PCF or local configuration to the RAN and at the same time, the SMF may also trigger PDU Session modification procedure to the UE based on the updated Alternative QoS profile.

NOTE : The usage of Alternative QoS profile is the same as specified in TS 23.501 [y1].

Editor’s note: How the Alternative QoS profile is updated to NG-RAN based on the status of NG-RAN (e.g., whether the RAN is using renewable energy or not) is FFS.

Editor’s note: Whether and how RAN renewal energy status information is known at the SMF/PCF is FFS.

6.X.5	Impacts on existing services, entities and interfaces

OAM:

-	Needs to provide the energy attributes of RANs to 5GC NFs (e.g., SMF)

RAN:

-	Needs to know its current energy source (e.g. renewable, non-renewable) state.

-	Needs to notify SMF the energy usage status

SMF:

-	Needs to provide the energy related QoS profile based on policies from PCF or local configuration and optionally the energy related Alternative QoS Profile to the NG-RAN.

-	Needs to provide the energy related QoS rules based on policies from PCF or local configuration to the UE and update the N4 rules to the UPF

PCF:

-	Needs to generate PCC rules to SMF for generating the energy related QoS profile, QoS rules and optionally the energy related Alternative QoS profiles after interacting with EIF

EIF:

-	needs to interact with SMF and provides the energy related information (e.g., energy consumption for the UE in different granularities)

-	if subscribed by SMF, the EIF needs to periodically or event triggered based to provide the energy related information to the SMF

-	needs to provide the notification to PCF to determine the policies to SMF for QoS rules, QoS profile or Alternative QoS profile

* * * End of Change * * * *