3GPP TSG-SA2 Meeting #168	S2-2504374

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025		    was 2897 was 0543

# Source:	KPN N.V.

# Title:	New SID on System Enhancement for Proximity-based Services in 5GS - Phase 4

# Document for:	Approval

# Agenda Item:	30.6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on System Enhancement for Proximity-based Services in 5GS - Phase 4

Acronym:	 FS_5G_ProSe_Ph4_ARC

Unique identifier:	TBD

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| AIPN-SAE | SA1 | 320022 | Requirements for evolution of the 3GPP system architecture |
| SMARTER | SA1 | 720005 | New Services and Markets Technology Enablers |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 900007 | Proximity based Services in 5GS | Rel-17 Stage 2 5G_ProSe work item. |
| 970016 | Proximity-based Services in 5GS Phase 2 | Rel-18 Stage 2 5G_ProSe_Ph2 work item. |
| 1040030 | Proximity-based Services in 5GS Phase 3 | Rel-19 Stage 2 5G_ProSe_Ph3 work item. |
| 941002 | NR sidelink relay enhancements | Rel-18 RAN work item on NR sidelink relay enhancements |
| 1040114 | NR sidelink multi-hop relay | Rel-19 RAN work item on NR multi-hop sidelink relay |

# 3	Justification

The 5GS has been enhanced to support Proximity Services in the past releases as specified in TS 23.304, based on the service requirements defined in TS 22.278, TS 22.261 and TS 22.115.

However, some requirements about Proximity Services captured in TS 22.278 and TS 22.261 such as multi-hop support for Layer-2 UE-to-UE Relay or multi-path communications via different UE-to-Network Relays remain to be addressed. Possible enhancements to existing discovery procedures such as MANET multicast routing for Layer-3 IP Type UE-to-UE multi-hop can also be beneficial.

# 4	Objective

The following objectives are considered for this study:

- WT-1: Enhance ProSe to support multi-hop over NR PC5 reference point for Layer-2 UE-to-UE Relay.

NOTE 1: This WT is subject to RAN WGs coordination.

- WT-2: Enhance ProSe to support MANET multicast for Layer-3 IP Type UE-to-UE multi-hop Relay.

- WT-3: Support Multi-path communications via different UE-to-Network Relays. The scope of this WT is limited to the study of the below combinations in this release:

  - Combination of Layer-2 UE-to-Network Relay and Layer-2 UE-to-Network Relay

  - Combination of Layer-2 UE-to-Network Relay and Layer-3 UE-to-Network Relay with N3IWF

  - Combination of Layer-3 UE-to-Network Relays w/o N3IWF and Layer-3 UE-to-Network Relays w/o N3IWF

  - Combination of Layer-3 UE-to-Network Relays w/o N3IWF and Layer-3 UE-to-Network Relay with N3IWF

NOTE 1: The study considers 2 UE-to-Network Relays only.

NOTE 2: 	Combination of Layer-2 UE-to-Network Relay and Layer-2 UE-to-Network Relay is subject to RAN WGs coordination.

- WT-4: Support Multi-path communications for Layer-2 Multi-hop Relay.

NOTE 1: This WT is subject to RAN WGs coordination.

## TU estimates and dependencies

| Work Task ID | TU Estimate | TU Estimate | RAN Dependency | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
|  | (Study) | (Normative) | (Yes/No/Maybe) |  |
| WT-1 | 1 | 0.5 | Yes |  |
| WT-2 | 1 | 0.5 | No |  |
| WT-3 | 1 | 0.5 | Yes |  |
| WT-4 | 1 | 0.5 | Yes |  |
| Totals | 4 | 2 |  |  |

Total TU estimates for the study phase: 4

Total TU estimates for the normative phase: 2

Total TU estimates:4 + 2 = 6

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.304 | Updates to support 5G_ProSe_Ph4 features |  |  |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Security aspects are considered by SA3.

Charging aspects are considered by SA5.

RAN aspects are considered by RAN WGs.

# 9	Supporting Individual Members

Supporting IM name

KPN N.V.

FirstNet

AT&T

Softil

Motorola Solutions

SyncTechno Inc

Suomen Erillisverkot

A.S.T.R.I.D. SA/NV

Netherlands Police

Nkom

Erillisverkot

Public Safety Communication Europe (PSCE)

BDBOS

NIST