3GPP TSG-WG2 Meeting #168	S2-2503105

Goteborg, Sweden, April 07 – 11, 2025

Source:	KDDI, SK Telecom

Title:	Discussion on IMS resiliency study

Document for:	Discussion

Agenda Item:	30.6

Work Item / Release:	T.B.D / Rel-20

Abstract: This paper provides background for IMS resiliency study.

1. Introduction

The contribution provides background and analyses for IMS resiliency study.

2. Discussion

At the SA#107 and CT#107 meetings, a draft IMS resiliency study item and discussion paper (SP-250245/SP-250246/CP-250210/CP-250243) were discussed, which address the following work tasks:

-	WT#1: Study whether and how the UDM provides single authentication between UEs and the 5GC and the IMS during the registration procedure.

-	WT2: Study whether and how 5GC notify the IMS status indicating failure or recovery conditions to UEs to avoid signalling storms and to mitigate IMS failures.

-	WT3: Study whether and how the 5GC in coordination with the UE configure the signalling frequency and timing of signalling based on the operator’s policy to mitigate negative impacts on the 5GC and/or the IMS.

Based on the discussion, SA #107 and CT #107 concluded the guidance and recommendation as follows:

For SA #107 (SP-250418)

- This topic is added to the list of potential Rel-20 5GA topics (See SP-250246 for reference)

- Based on CT WGs feedback, moderated discussion in SA2 can start after SA2#168 (April) meeting, if needed

For CT #107 (CT_107_meeting_report_v110)

- TSG CT recommendation was to have a contribution in CT working groups which impacts only to CT related issues.

Based on the guidance and discussion in SA#107, we update the discussion paper (SP-250245) to generalize each work task and to remove solution related description.

2.1	Background

From July 2 to July 4, 2022, KDDI experienced a significant communication failure. Failures caused by maintenance work cascaded and nationwide area was affected. The number of affected customers was approximately 22.78 million for Voice (VoLTE) and 7.65 million for Data (4G/5G). The impact extended widely across industries using smartphones and IoT, affecting logistics, automotive, government services, banking, and transportation, thereby influencing essential living infrastructure (see clause 2.2.for detail).

Most MNOs apply the IMS framework for their voice services and will still need to continue operating IMS even in the 6G era. However, large-scale communication failures related to or affected by IMS have occasionally occurred on not few operators’ mobile network in the past few years.

Based on the lessons learned from the significant failure and aiming to achieve stable voice services with IMS, we provide issues and potential solutions for IMS resiliency.

2.2	Overview of KDDI Communication Failure and Response

The communication failure and the response progressed through the following three phases:

Phase 1: Congestion at IMS

The routers were incorrectly configured during maintenance of the nationwide transport network routers at the NC in Tama. The SIP Registration request was abandoned, rapidly triggering numerous retransmissions in network and Tama’s VoLTEs Nodes became congested. The VoLTEs Nodes at other sites became congested because distributed processing was performed via the nationwide transport network.

NOTE:	During this failure, the VoLTEs Nodes were congested, and as a result, the signals from the VoLTEs Nodes were not sent to the HSS. However, if the VoLTEs Nodes had been able to send signals to the HSS, there is a high possibility that the HSS would have received an excessive number of signals and become congested.

Phase 2: Congestion at PCRF

The congestion in the VoLTEs nationwide led to an increase in the retransmission of SIP Registration requests from the UE to the PCRF, resulting in congestion in the PCRF as well. Due to congestion in the PCRF, the PCRF returned error responses to session requests from the PGW. As a result, the UEs that received these error responses retransmitted signals to the PCRF, leading to further and continued congestion in the PCRF. As a result, it became difficult to process SIP Registration and establish voice communication nationwide. Additionally, some UEs are believed to have affected data communication as well.

Phase 3: IMS disconnection (recovery)

In order to reduce congestion on PCRF, flow rate control was implemented, but congestion was not eliminated. Some IMS nodes continued to send excessive signals, so the overloaded status in PCRF continued. The VoLTEs Nodes nodes (6 out of 18 nationwide) were isolated, thus eliminating retransmission from devices.

Source: The July 2 Communication Failure and Our Response

https://www.kddi.com/english/important-news/20220729_01/

Observation 1: The cause of signaling on the core network side.

The SIP Registration request was abandoned, rapidly triggering numerous retransmissions in network and S/I-CSCF became congested. The S/I-CSCFs at other sites became congested because distributed processing was performed via the nationwide transport network.

The congestion in the S/I-CSCFs nationwide led to an increase in the retransmission of SIP Registration requests from the UE to PCRF, resulting in congestion in the PCRF as well. Due to congestion in the PCRF, the PCRF returned error responses to IMS session requests transmitted from the UE via the PGW. As a result, the UEs that received these error responses retransmitted signals to the PCRF, leading to further and continued congestion in the PCRF.

Observation 2: The cause of signaling on the UE side.

Due to a misconfiguration of the core router during maintenance work, while the UL signals pass through, the DL signals fail to reach the UE in 50% of the cases. As a result, the UE determines that the SIP Registration process has not been completed and failed, and the SIP Registration request were repeatedly retransmitted.

For 5GS/EPS deployment, NAS level congestion control is achieved by providing a Back-off Timer and/or cause values to UEs. However, UEs need to be switched OFF/ON after receiving a Back-off Timer IE from a network (incl, IMS). So, the network using a Back off Timer does not solve the issue of the manual OFF/ON caused by the variety of UE implementations.

Observation 3: The issues for 3GPP standards

When a UE subscribes to IMS service, it sends a USIM authentication request to 5GC first, then to the IMS second. This means the UDM must process the same authentication twice, causes extra signaling.

For current 3GPP standards, the IMS has no response or returns a response with error code which does not contain the appropriate reason for the failures. If the IMS and/or EPC/5GC providing MMTEL service is under failure condition, UEs may not be able to receive SIP messages and may initiate re-attach/re-registration procedure to the EPC/5GC and/or IMS.

The flexibility allowed by 3GPP standards for vendors to decide on signaling behavior after receiving cause values can lead to inconsistent UE behavior, some of which a mobile operator does not expect.

Proposal 1: Based on observation 1, 2 and 3 above, 3GPP needs to address following issues:

-	#1: Whether and how to mitigate authentication signalling between UEs and the 5GC and the IMS during the registration procedure.

-	#2: Whether and how the UE is aware of the IMS status indicating failure or recovery conditions to avoid signaling storms and to mitigate IMS failure.

-	#3: Whether and how the 5GC in coordination with the UE configure the signaling frequency and timing of signaling based on the operator’s policy to mitigate negative impacts on the 5GC and/or the IMS.

3. Proposal

Source companies of this contribution propose to kick-off a moderated discussion to determine which working group(s) should lead the work tasks as follows and to identify what SA2 should address:

-	WT#1: Study whether and how to mitigate authentication signalling between UEs and the 5GC and the IMS during the registration procedure.

-	WT#2: Study whether and how the UE is aware of the IMS status indicating failure or recovery conditions to avoid signaling storms and to mitigate IMS failure.

-	WT#3: Study whether and how the 5GC in coordination with the UE configure the signaling frequency and timing of signaling based on the operator’s policy to mitigate negative impacts on the 5GC and/or the IMS.

NOTE: The discussion should be based on the feedback from CT WG.