SA WG2 Meeting #168                                                                                    S2-2503671

Goteborg, Sweden, 07 – 11April 2025

# Source:	vivo

# Title:	New SID: Study on Architecture support of Ambient power-enabled Internet of Things Phase2

# Document for:	Discussion and Approval

# Agenda Item:	30.6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Architecture support of Ambient power-enabled Internet of Things Phase2

Acronym: FS_AmbientIoT_Ph2

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

In R19 FS_AmbientIoT study conclusion phase, the UE reader is postponed. On one hand. the UE reader can provide efficient and flexible coverage supplement to the RAN reader coverage for Ambient IoT and reduce the deployment cost of the AIoT feature; on the other hand, the support of UE reader can open up the market in personal IoT network, e.g. the smart phone can work as the UE reader, to control the personal and residential AIoT devices. Furthermore, as shown in RP-250802, there was general consensus at RAN side on the support of device 1 in D2T2 in R20 Ambient IoT.

Hence, from the architecture perspective, Ambient IoT services needs to support UE reader.

4	Objective

This SID studies the architecture support of ambient power-enabled IoT devices, based on the services requirements defined in 3GPP SA1 and RAN WGs.

The objectives include:

- WT#1 Support of UE reader:

  - WT#1.1  Study architecture enhancement for UE reader.

  - WT#1.2  Study the management of capability, subscription, authorization, registration and connection for a UE to work as a UE reader.

  - WT#1.3  Study how the support AIoT service operation request towards UE reader and how the resource allocation for UE readers to perform the Ambinet IoT service operation.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 |  |  | YES |  |
| WT#1.1 |  |  |  |  |
| WT#1.2 |  |  |  |  |
| WT#1.3 |  |  |  |  |

Total TU estimates for the study phase: XX TUs

Total TU estimates for the normative phase: XX TUs

Total TU estimates:  XX TUs

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

6	Work item Rapporteur(s)

7	Work item leadership

SA2

8	Aspects that involve other WGs

SA3 for the Security aspects,

SA5 for the Charging aspects,

RAN WGs for the RAN related issues,

9	Supporting Individual Members

Supporting IM name