3GPP TSG-SA2 Meeting #168	S2-2503739

# Goteborg, Sweden, 07 - 11 April 2025

# Title:	[Draft] LS Reply on Optimizations to UPF event subscriptions and reporting

# Response to:	C4-250455 LS on Optimizations to UPF event subscriptions and reporting

# Release:	Rel-19

# Work Item:	FS_PAIDC_UPF

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person:

Name:	Belen Pancorbo

Tel. Number:

E-mail Address:	Belen.Pancorbo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks CT4 for sharing the Technical Report for the Study on Protocol for AI Data Collection from UPF.

SA2 has analysed solutions #2, #4 and #5 from TR 29.889 and would like to inform about our conclusions on that analysis.

According to the descriptions, solutions #2, #4 and #5 provide a solution for several use cases where the operator needs to collect frequent User Plan measurements for PDU sessions or users with specific characteristics (e.g. specific subscriber category such as VIP or gold/silver/bronze users). Those solutions focus on the enhancement of current UPF event exposure functionality to allow grouping of subscriptions/notifications targeting specific set of UEs/PDUs having specific characteristics.

Solutions #2 and #5 propose to allow grouping subscriptions/notifications for UPF event exposure for sessions identified by a marker and to enhance the role of the PCF with the additional function of marking sessions according to specific criteria.

For solution #2 the criteria is based on adding new subscription information provisioned per UE/S-NSSAI/DNN in UDR Policy Data, which would require the provisioning of such new information by the operator.

Solution #5 instead allows the dynamic provisioning of such criteria as new Application Data in UDR (new resource) and defines a new NEF API to provision such dynamic criteria in UDR. It is required also new procedures in PCF to obtain such data from UDR Application Data and use it as input to decide whether and how to mark PDU sessions towards the SMF/UPF.

The marking process implies additional interactions over N7 just for the purpose of providing the PDU session marking from the PCF to the SMF and over N4 from the SMF to the UPF. Those interactions also involve the transfer of new additional information including such marking beyond of PCC and sessions rules provided to the SMF. In addition, solution #5 proposes dynamic conditions as part of the criteria, e.g. the location of the UEs, which would imply the additional monitoring of such conditions in PCF and additional signalling over N7 to update the marking of the related PDU sessions when those dynamic conditions change. It is considered the aspects above deviate from the current architecture using the PCC framework for a different purpose than the one related with Policy and Charging.

In addition, the current analytic framework, according to clause 6.1.3. from TS 23.288, just allows a few targets for analytics reporting as a specific list of UEs, i.e. a list of SUPIs, group of UEs, i.e. a list of Internal-Group-Ids, or any UE (i.e. all UEs). For the generation of the corresponding analytic for a target, the NWDAF may need to collect data from different NFs e.g. UPF, SMF, AMF, about the related targets.

However, Solutions #2 and #5 lack the analysis on how the NWDAF, upon a request for analytic for a specific target determines that data should be collected from the UPF for sessions identified by a specific marker.

Last, solutions #2 and #5 are only focused on how the NWDAF should collect data from the UPF about the specific set of UEs/PDU sessions having specific characteristics but it is missed the analysis on how the NWDAF would be able to collect data from other source NFs, as e.g. the SMF or AMF, for those UEs/PDU sessions for the generation of the analytic.

Solution #4 proposes instead using 5G VN groups to classify UEs/PDU sessions. This solution assumes that a 5G VN group needs to be created and associated to the UEs with certain characteristics, following clause 5.29.2 in TS 23.501. It is assumed those 5G VN groups will be defined with no data associated to 5G VN Group communication (i.e. setting 5G VN group communication indication to False). Then, when a consumer of the analytics wants to request some analytic from the NWDAF for such set of UEs it provides the external group Id identifying the 5G VN group as target for the analytic reporting. Upon this request the NWDAF triggers data collection from the UPF (also from other NFs when needed) by including the corresponding internal Group Id as target of the UPF event exposure subscription. In addition, the SMF provides the internal group IDs identifying the 5G VN groups associated to the UE to the UPF during PFCP session establishment/modification. This solution also proposes an enhancement of the UPF event exposure to allow the NWDAF to request a bundle subscription and notification for the sessions associated with an internal group Id(s).

From SA2’s perspective, the enhancement of the UPF event exposure allowing subscriptions for internal group Ids together with providing the list of internal group Ids from the SMF to UPF during PFCP establishment, is considered aligned with the current architecture.

However, using 5G VN groups solely for dynamic group management and UE classification, decoupled from its original purpose related with private communication for 5G LAN-type service is considered at least questionable mainly due to the following implications:

- A 5G VN group is mandatorily defined with an S-NSSAI/DNN, and this S-NSSAI/DNN is get associated to the UEs belonging to that 5G VN group. That is needed for 5G VN group private communication but may be unnecessary or incorrect when used for grouping UEs with specific characteristics.

- When a 5G VN group is created and UEs associated to it, the UDM sends individual notifications per UE/PDU session to the SMF/AMF including the new internal group Id and 5G VN group Data (even if it is only the associated S-NSSAI/DNN). This results in additional signalling in the NW, which could become significant in case of having large groups with many members or a big number of groups. In addition, since new S-NSSAIs are associated to the UEs, it may imply the sending of additional UE Configuration Update from the AMF to notify the involved UEs about updates in Configured and/or Allowed NSSAI.

- The definition of 5G VN group is considered primarily oriented to use shared data feature, but in this case, there is no common data among the group members.

As a summary, it is considered that Solution #4 builds on existing mechanisms and concepts however, it introduces complexities not aligned with the intended use cases for 5G LAN services. Solutions #2 and #5, on the other hand, involve even more complex changes involving new procedures, APIs and additional signalling that deviate from the existing PCC and analytic framework.

2. Actions:

To CT4.

ACTION: 	SA2 kindly requests CT4 to take this information into account when concluding in the final solution for the normative work.

3. Dates of Next SA2 Meetings:

SA2#169	May 19 – 23, 2025	Fukuoka, Japan

SA2#170	August 25 – 29, 2025	Göteborg, Sweden