3GPP TSG-SA2 Meeting #168	S2-2503745

07 - 11 April 2025, Goteborg, SE	(revision of S2-25xxxxx)

Source:	CATT, CSCN

Title:	FS_5GSAT_Ph4_ARC Key issues for WT#2_2

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper is to propose new key issue for FS_5GSAT_Ph4_ARC.

1 Discussion

The Work Task#2 of the FS_5GSAT_Ph4_ARC SID is:

-	WT-2: UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available.

-	WT-2.1 Support of UE-SAT-UE communication for roaming case considering the requirements for lawful interception.

NOTE 6:	The scenario will be restricted to two roaming UEs served by same VPLMN and HPLMN.

NOTE 7:	GSMA should be consulted with regard to the roaming requirement for UE-satellite-UE communication, and then further clarification may be needed to specify the detailed roaming scenario..

-	WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked.

NOTE 9: No UE impact is expected for WT-2.

Based on the WT, we have to study how to support UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO.

2 Proposal

It is proposed to agree the new key issues for WT#2 into TR 23.700-19 on FS_5GSAT_Ph4_ARC as followed.

* First change *

5.X	Key Issue #Y: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

5.X.1	Description

In Rel-18, it only supports UE-satellite-UE communication for 5G-LAN services under GEO satellite constellation. The enhancement of UE-SAT-UE communication under NGSO constellation for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type is also needed.

To support UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, the following aspects need to be further studied:

-	UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

NOTE 1:	Whether existing mechanism can be reused to support this KI needs to be checked.

NOTE 2:	No UE impact is expected for this KI.

* End of change *