3GPP TSG-SA WG2 Meeting #168	S2-2503815

# Goteborg, Sweden, April 07 – 11, 2025               	       (revision of S2-250xxxx)

Source: 	OPPO

Title: 	Key Issue for WT2, WT#3 & WT#4: QoS Mechanism

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#2,3,4 on QoS mechanism for sensing services.

1. Discussion

The WT-2, WT-3, and WT-4 requires end-to-end QoS mechanism for Integrated Sensing and Communication services, where sensing accuracy, latency, and reliability directly impact application performance. SA1 has defined 5G wireless sensing performance requirements at clause 6 of TS 22.137 with detailed example of Performance requirements (e.g.accuracy, latency, refreshing rate, false alarm ratio etc.). This Key Issue focuses on ​sensing-specific QoS requirements with 3GPP QoS frameworks to ensure service guarantees across heterogeneous sensing entities and data flows.

The issues to be studied in this KI would include:

- ​Sensing QoS Parameter Definition:

  - How to derive ​sensing-specific QoS parameters (e.g., detection probability ≤10⁻³, spatial resolution ≤0.1m) from SA1 use case requirements (e.g., V2X collision avoidance, UAV restricted zone monitoring).

- ​QoS Mapping to Transport Mechanisms:

  - How to map sensing QoS parameters (e.g., update rate ≥10Hz) to 3GPP QoS attributes (e.g., 5QI for prioritized packet delay budget) for different data types: e.g. Raw sensing data, Intermediate results, Final sensing results (strict integrity requirements).

- ​QoS-Driven Entity Selection:

  - How to extend WT-3 sensing entity discovery mechanisms to evaluate QoS capabilities.

  - How to prioritize SF nodes based on capability criterias.

- ​QoS-Based Authorization:

  - How to integrate QoS validation into WT-2 service authorization.

- ​Dynamic QoS Adaptation:

  - How to trigger QoS parameter adjustments (e.g., reducing sensing resolution during network congestion) while maintaining minimum service levels.

Thus, it is proposed to agree the following changes to 3GPP TR 23.700-14.

2. Proposal

Start of Changes (all new text below)

## 5.1.x	Key Issue #x: QoS Mechanism

### 5.1.x.1	Description

This Key Issue addresses the need to define QoS mechanisms for integrated sensing and communication services, ensuring alignment with sensing accuracy, latency, and reliability requirements across diverse applications.

The study aims to investigate:

- How to define sensing-specific QoS parameters (e.g., sensing accuracy, update rate, detection probability) based on SA1 performance requirements?

- Whether and how to map sensing QoS requirements to 3GPP data transport QoS attributes (e.g., latency, reliability, throughput) for the transmission of different data types (e.g. 3GPP sensing data, processed sensing data and sensing result).

NOTE: Definition on format of 3GPP sensing data, processed sensing data and sensing result needs to coordinate with RAN WGs.

- Whether it is necessary and how to ensure the selection of sensing entities (transmitters, receivers, SF nodes) can fulfil the sensing service performance requirements?

- How to provide the sensing QoS service requirements to the sensing entities?

- Whether and how to enforce authorization to validate sensing requests against available QoS capabilities (e.g., rejecting requests exceeding network QoS capacity)?

- Whether and how to adjust sensing QoS parameters or resource allocation dynamically?

End of Changes