3GPP TSG-WG SA2 Meeting #168	S2-2504019

Goteborg, Sweden, April 7 - 11, 2025	(revision of S2-2503195)

Source:	Ericsson

Title:	Clause 5.X: ADM Discovery and Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR proposes ADM discovery and selection mechanism.

1. Introduction

In general, for each feature, when a new NF is introduced, the NF discovery and selection is described in clause 6.3 of TS 23.501 or in a new clause of the specification of the feature.

[Proposal-1] Introduce clause 5.X for ADM discovery and selection.

When the AIOTF needs to retrieve or update the device profile data, the retrieval or update operation is performed in the ADM. In this case, the AIOTF needs to perform ADM discovery and selection.

When the AIoT Device ID information indicates individual AIoT device(s), the NEF may further query ADM to get the last serving AIOTF information as an optimization, assuming the AIoT device(s) are likely to be served by the last known AIOTF if they stay in the same area. In this case, the NEF needs to perform ADM discovery and selection.

When the ADM discovery and selection is performed, the AIOTF or the NEF query NRF or check local configuration.

[Proposal-2] Capture the description above in clause 5.X.

Regarding the factors to be considered for ADM selection, it hasn’t been discussed in SA2. It hasn’t been discussed whether the network identifier granularity is sufficient or there are needs of having permanent device ID range granularity. Therefore, it is proposed to have an EN for the factors to be considered for ADM selection.

[Proposal-3] Factors for ADM selection are FFS.

In clause 7.5.1, the service operations provided by the AF is listed in the table below:

Table 7.5.1-1: NF services provided by ADM

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF |
|  | Update | Request/Response | AIOTF |

The NEF is missing as an example consumer for the Query service operation.

[Proposal-4] Update the table to include the NEF as an example consumer.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.2.0:

* * * Start of Change * * * *

## 5.X	ADM Discovery and Selection

The ADM discovery and selection function is supported by the AIOTF to select an ADM instance to retrieve the device profile data or update the last known AIOTF for the AIoT device. The AIOTF may also discover and select an ADM to retrieve AF authorization data. Similarly, the NEF uses the ADM discovery and selection function to select an ADM to obtain the last known AIOTF for the AIoT device.

When the ADM discovery is performed, the AIOTF or the NEF utilize the NRF to discover the ADM instance(s) unless the ADM information is available by other means, e.g., locally configured. The AIOTF or the NEF  select an ADM instance based on the available ADM instances (obtained from the NRF or locally configured).

One or more of the following factors may be considered for the ADM discovery and selection:

- The AIoT device permanent ID.

- The AF ID.

Editor’s Note: 	The further investigation of factors for the ADM selection may be needed and other factors are FFS.

* * * Next Change * * * *

### 7.5.1	General

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5.1-1: NF services provided by ADM

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |

* * * End of Change * * * *