3GPP TSG-SA2 Meeting #168	S2-2504121

# Göteborg, Sweden, April 07-11, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0550 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the 5G ProSe Multi-hop UE-to-UE Relay Discovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, FirstNet, Qualcomm, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 6.3.2.6.2 the procedure description is not aligned with the description of the parameters in clause 5.8.6.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligning procedure description in 6.3.2.6.2 with the parameter description in 5.8.6.2. Additionally, some spelling errors were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Important information missing in the procedure description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

#### 6.3.2.6	5G ProSe Multi-hop UE-to-UE Relay Discovery

##### 6.3.2.6.1	General

The 5G ProSe Multi-hop UE-to-UE Relay discovery may use one of the two different types of discovery operations, i.e. discovery for IP PDU type and discovery for non-IP PDU type, depends on the RSC for the discovery.

5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type only needs to discover the presence of the 5G ProSe Multi-hop UE-to-UE Relay. The discovery of the target 5G ProSe End UEs are performed via DNS queries after establshing a link with the 5G ProSe Multi-hop UE-to-UE Relay(s). This procedure is also used by Multi-hop UE-to-UE Relay(s) among themselves in order to join/create MANET network. The details of the procedure are defined in clause 6.3.2.6.2.

##### 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP (Ethernet or Unstructured) PDU type is for the discovery of a target 5G ProSe End UE and the corresponding routes. The procedures for 5G ProSe Multi-hop UE-to-UE Relay discovery Model A/B for non-IP PDUs (e.g. Ethernet or Unstructured) are defined in clause 6.3.2.6.3.

##### 6.3.2.6.2	Procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type

Depicted in figure 6.3.2.6.2-1 is the procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type, with Model A.

Figure 6.3.2.6.2-1: 5G ProSe Multi-hop UE-to-UE Relay Discovery of IP PDU type with Model A

1.	The 5G ProSe Multi-hop UE-to-UE Relay is configured to operate IP PDU type of discovery based on the indication associated with the RSC, as defined in defined in clause 5.1.5.1.

2.	The 5G ProSe Multi-hop UE-to-UE Relay sends a Multi-hop UE-to-UE Relay Discovery Announcement message. This message contains the Type of Discovery Message, User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay and the RSC, as defined in clause 5.8.6.2.

The 5G ProSe Multi-hop UE-to-UE Relay self generates a Source Layer-2 ID and decides a Destiantion Layer-2 ID as defined in clause 5.8.6.2.

The 5G ProSe End UE(s) or other 5G ProSe Multi-hop UE-to-UE Relay(s) in proximity monitoring the same RSC may receive the announcement message and process it accordingly, e.g. initiates a Layer-2 Link establishment with the announcing 5G ProSe Multi-hop UE-to-UE Relay.

Depicted in figure 6.3.2.6.2-2 is the procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type, with Model B. A 5G ProSe Multi-hop UE-to-UE Relay may also use the procedure, i.e. send the Multi-hop UE-to-UE Relay Discovery Solicitation (taking the role of the 5G ProSe End UE (Discoverer)), if it needs to discover other 5G ProSe Multi-hop UE-to-UE Relays.

Figure 6.3.2.6.2-2: 5G ProSe Multi-hop UE-to-UE Relay Discovery of IP PDU type with Model B

1.	The 5G ProSe End UE is configured to operate IP PDU type of discovery based on the indication associated with the RSC, as defined in defined in clause 5.1.5.1.

2.	The 5G ProSe End UE sends a Multi-hop UE-to-UE Relay Discovery Solicitation message. This message contains the Type of Discovery Message, RSC and (optional) User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay, as defined in clause 5.8.6.2.

The User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay is only included if the 5G ProSe End UE wants to find a specific target relay.

The 5G ProSe End UE self generates a Source Layer-2 ID and decides a Destination Layer-2 ID as defined in clause 5.8.6.2.

3.	The 5G ProSe Multi-hop UE-to-UE Relay-1 may send a Multi-hop UE-to-UE Relay Discovery Response message, if the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.5.1. If User Info ID is included in the solicitation message, 5G ProSe Multi-hop UE-to-UE Relay-1 only sends the response message when it matches its own User Info ID. This response message contains the User Info of discoverer, User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay, the RSC and type of Discovery message, as defined in clause 5.8.6.2.

The 5G ProSe Multi-hop UE-to-UE Relay self generates a Source Layer-2 ID and decides a Destination Layer-2 ID as defined in clause 5.8.6.2.

The 5G ProSe Multi-hop UE-to-UE Relay-1 in proximity may decide not to respond if the criteria are not met, or due to UE local configurations.

>>>> End of Changes <<<<