3GPP TSG SA WG 2 Meeting SA2#168	                                                                  TDoc S2-2504337

Gothenburg, Sweden, 2025-04-07 – 2025-04-11

Title:	LS reply on support for multiple non-3GPP device identifiers

Response to:	LS C1-250822 on LS on support for multiple non-3GPP device identifiers from CT1

Release:	Rel-19

Work Item:	UIA_ARC

Source:	SA2

To:	CT1

Cc:	CT3

Contact person:	Magnus Hallenstål

magnus.l.hallenstal@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR on TS 23.501, S2-2504253, CR# 6208

CR on TS 23.502, S2-2504336, CR# 5383

CR on TS 23.503, S2-2504477, CR# 1551

# 1	Overall description

SA2 has discussed the questions in LS on support for multiple non-3GPP device identifiers from CT1 and have arrived a the following conclusion:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

SA2 answer:

SA2 recognise the possibility that allowing only a single non-3GPP Device Identity in a PDU SESSION MODIFICATION REQUEST may increase the signalling overhead. To rectify this issue, SA2 has agreed the attached CRs, which allows the PDU SESSION MODIFICATION REQUEST to optionally have multiple non-3GPP Device Identifiers

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

SA2 answer:

If any non-3PP Device Identifier in the PDU session modification request is not available in the UDR, the SMF will reject the PDU SESSION MODIFICATION REQUEST, without sending any non-3GPP Device Identifier. See attached CRs.

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

SA2 answer:

SMF will not send any non-3GPP Device identifier in the PDU SESSION MODIFICATION COMMAND message in the scenario where one or more non-3GPP Device Identifier is/are removed from UDR. See attached CRs.

# 2	Actions

To CT1

ACTION: Based on the answers above, SA2 asks CT1 to take any needed action

# 3	Dates of next TSG SA WG 2 meetings

SA2#169		2025-05-19 – 2025-05-23 Fukuoka City, Japan

SA2#170		2025-08-25 – 2025-08-29 Gothenburg, Sweden