3GPP TSG-WG SA2 Meeting #168	S2-2504421

Goteborg, SE Apr 07th – 11th, 2025

Source:	Huawei, HiSilicon, CMCC, SK Telecom, ZTE

Title:	New use case for WT2: user plane performance optimization with assistance of NWDAF

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: It is proposed to add use case in user plane performance optimization with assistance of NWDAF.

1. Introduction/Discussion

Knowing the traffic pattern in advance is beneficial for 5GC to synchronize its network status with the application traffic, which can optimize user plane performance, e.g. better QoS control .

Considering that the analytic capability of NWDAF can generate the statistic or prediction of the traffic pattern without relying on special protocols. It is proposed to add a use case for user plane performance optimization with assistance of NWDAF as described above.

2. Text Proposal

# * * * * First change (ALL NEW TEXT) * * * *

5.1.x	Use Case #X:  user plane performance optimization with assistance of NWDAF

5.1.x.1	Description

It is beneficial for the network to know about the traffic pattern in advance in order to optimize user plane performance, e.g., better QoS control. For instance, the network can generate MDBV, Periodicity for TSC service based on the traffic pattern as described in TS 23.501[1].

Currently, the above mechanism can only be applicable for services with special protocols, e.g. TSN. For other kinds of applications, most of them rely on transport layer protocol to mitigate congestion, e.g. TCP or QUIC as described in RFC 793 [2] and RFC 9000 [3]. Then, the application behaviour (e.g., packet sending rate) is related to the network status, (e.g., the E2E delay and packet loss, etc). Hence, it is hard to give a precise traffic pattern in advance to the network from the AF/AS.

The analytic capability of NWDAF can generate the statistic or prediction of the traffic pattern for 5GC to do user plane performance optimization, e.g., assistance to QoS control management, in advance.

NOTE 1:  Solutions for this use case should aim at avoiding extensive signalling.

NOTE 2:  Solutions will reuse existing policy and QoS control as much as possible.

NOTE 3:  Solutions shall not include any recommendations by the NWDAF.

# * * * * Next change  * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	IETF RFC 793: "TRANSMISSION CONTROL PROTOCOL(rfc793.txt.pdf)"

[3]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport" (rfc-editor.org/rfc/rfc9000.txt)

# * * * * End of change * * * *