SA WG2 Meeting ##169	S2-2504502

19 - 23 May, 2025, Fukuoka, Japan

3GPP TSG-CT WG1 Meeting #154	C1-252562

# Wuhan, China, 07-11 April 2025

# Title:	LS on clarification for multi-hop UE-to-UE relay discovery using model B

# Response to:	-

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	CT1

To:	SA2

Cc:	-

Contact Person:

Name:	Sunghoon Kim

E-mail Address:	sunghoon@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR0732 against TS 24.554

1. Overall Description:

For 5G ProSe multi-hop UE-to-UE relay discovery procedure using model B, CT1 has identified the following issue:

According to the current stage-2 requirement, as described in the clause 6.3.2.6.2 of TS 23.304, the user Info ID of the 5G ProSe multi-hop UE-to-UE relay UE is optional in the discovery solicitation message. Without this information, the receiver (i.e., 5G ProSe multi-hop UE-to-UE relay UE) cannot identify the target 5G ProSe multi-hop UE-to-UE relay UE and cannot determine whether to complete the discovery procedure, thus the receiver will continue to send the discovery solicitation message indefinitely.

Based on the above analysis, CT1 has concluded to include the user Info ID of the 5G ProSe multi-hop UE-to-UE relay UE as a mandatory information element in the discover solicitation message.

2. Actions:

To SA WG2

ACTION: 	CT1 asks SA WG2 to take the above information into account and consider updating their specification if deemed necessary.

3. Date of Next CT1 Meetings:

CT1#155                           19th May- 23rd  May 2025.............................Bratislava, Slovakia

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India