SA WG2 Meeting ##169	S2-2504503

19 - 23 May, 2025, Fukuoka, Japan

3GPP TSG-CT WG3 Meeting #140	C3-251602

# Wuhan, CN; 7 – 11 April 2025

# Title:	LS Reply on Optimizations to UPF event subscriptions and reporting

# Release:	Rel-19

# Work Item:	FS_PAIDC-UPF

Source:	CT3

To:	CT4

Cc:	SA2

Contact Person:

Name:	Susana Fernández

Tel. Number:

E-mail Address:	susana. fernandez@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

CT3 thanks CT4 for sharing the Technical Report for the Study on Protocol for AI Data Collection from UPF.

CT3 has studied solutions #2, #4 and #5 in TR 29.889 proposed to enable an NF service consumer (e.g. NWDAF) to create a bulk subscription targeting a specific set of UEs/PDU sessions instead of having a UPF event subscription per UE/PDU session. CT3 would like to inform about our conclusions of that analysis.

Solutions #2 and #5 are introducing additional functionality not associated with the existing roles for NEF, PCF and UDR with the purpose to mitigate the signalling in the UPF. This extra functionality requires the support of a new API and new resources (solution #5), extra logic and traffic for the provisioning and update of markers as part of the Nudr_DataManagement and Npcf_SMPolicyControl APIs, dynamic validity conditions control handling (location, roaming) and the support of the related error scenarios. The PCC framework is being used with a purpose that is not related to the support of policy and charging control in the 5GC network (support of PCC and session rules) for which it is specified. The acceptance of these solutions would open the door to future solutions to mitigate extra processing load in the SMF and AMF for similar scenarios.

From CT3 point of view, the final solution to reduce the extra signalling and processing load in the UPF should not impose additional signalling and processing load in other entities of the 5GC not related to the functionality associated to the degradation, as it is the case of NEF, UDR, PCF and SMF.

CT3 considers that solution #4 does not have any impact in CT3 procedures and the analysis of that solution goes beyond CT3 remit. Thus no comment is provided.

2. Actions:

To CT4.

ACTION: 	CT3 kindly requests CT4 to take this information into account when concluding on the final solution for the normative work.

3. Dates of Next CT3 Meetings:

CT3#141				May 19-23, 2025 				Bratislava, SK

CT3#142				Aug 25-29, 2025 				Gothenburg, SW