SA WG2 Meeting ##169	S2-2504509

19 - 23 May, 2025, Fukuoka, Japan

3GPP TSG-CT WG4 Meeting #128	C4-251477

# Wuhan, P.R. China; 07th – 11th April 2025

# Title:	LS on Clarification on SCP data collection

# Response to:	--

# Release:	Rel-19

# Work Item:	AIML_CN

Source:	CT4

To:	SA2

Cc:	--

Contact Person:

Name:	Roya Rezagah

Tel. Number:

E-mail Address:	roya.rezagah@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	--

1. Overall Description:

TS 23.502 clause 5.2.29.2 defines the event(s) supported by Nscp_EventExposure. In this clause one event is defined, which is "Service Signalling characteristics". Furthermore, TS 23.288 clause 6.22.2 defines the list of information which may be reported to NWDAF as the consumer via SCP Event Exposure service. CT4 would appreciate SA2 insights on a few questions related to data collection from SCP:

Q1) Regarding the "UE related Context Data collection" specified in Table 6.22.2-1 of TS 23.288:

Requiring the SCP to provide information with UE granularity means that the NFs in the network would be required to be configured to use the custom header 3gpp-Sbi-Correlation-Info which is optional to support and may be subject to security/regulatory requirements (see TS 29.500). Furthermore, different NFs may use different UE identities, e.g. SUPI, GPSI, etc, for the same UE within this header Also the NFs of earlier releases that do not support this header do not provide UE information to the SCP, which would impact the accuracy of the statistics.

Considering these limitations, CT4 would like to ask SA2 to confirm whether the information with UE granularity is required to be obtained by the SCP for the event exposure purpose.

Q2) According to TS 23.288 clause 6.22.2, the SCP is required to distinguish between different types of communications and signallings and provide information, for example, to collect the data of "Number of redundant signalling of NF" and "A posterior Request type of NF (0..max)" in Table 6.22.2-1, and "SCP Signalling statistics" in Table 6.22.2-3 which are depicted bellow. Currently there is no mechanism to enable SCP to differentiate between different types of communication SA2 refers to, e.g. whether the signalling is redundant or an "A posterior Request".

Therefore, CT4 would like to ask SA2 to consider reverting the requirements on "A posterior Request type of NF (0..max)" and "Number of redundant signalling of NF", unless SA2 could provide clarification on how these requirements can be implemented.

Table 6.22.2-1: UE related Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| UE ID |  | Identifies a single UE. |
| Signalling feature data |  | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures (NOTE 1). |
| [...] |  |  |
| > Request type and number from NF (0..max) | NF | Request type received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc., as well as the number of requests received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. (NOTE 2) (NOTE 3). |
| >> Time duration from receiving request from NF to response to NF | NF | Time duration between the request from NF and response to NF. |
| >> Time duration of transactions related to SCP ingress interface | SCP | Time duration between the received request from consumer NF and response forwarded by SCP to consumer NF (NOTE 4). |
| [...] |  |  |
| >> Number of redundant signalling of NF | NF | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted multiple times (NOTE 2) (NOTE 3). |
| >> A posterior Request type of NF (0..max) | NF | A posterior Request types triggered from NF, for NF Service request (NOTE 3) (NOTE 6). |
| [...] |  |  |
| [...] NOTE 3: The word "NF" refers here to SCP as well. As a centric routing/proxy entity, SCP might be involved with some signalling that may not be visible to the NF sending the SCP's ingress traffic, such a signalling could be associated with actions such as selection & discovery, retransmissions, re-directions, re-selection, throttling, etc. [...] |  |  |

Table 6.22.2-3: NF Specific Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF ID |  | NF instance ID |
| > Usage information of UE IP address resources | SMF | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. This parameter is valid for SMF only. |
| > Load information of connected UPFs | SMF | Load information of connected UPFs such as using PFCP Load Control Information. This parameter is valid for SMF only. |
| SCP Signalling statistics | SCP | The number of different types of signalling received and sent by SCP during a target time period, etc. This parameter is valid for SCP only. |

Q3) CT4 would also appreciate SA2 to clarify what "different types of signalling" corresponds to for "SCP Signalling statistics".

Q4) "Heart-beat related information", "NF load status information", "Capacity and priority information" which are listed in TS 23.288 Table 6.22.2-2, may be provided by NRF. If required, SCP may get this information from NRF. Therefore, CT4 would like to ask for clarification on why the SCP is also required to provide this information.

CT4 kindly asks SA2 to provide clarification or answer to the above questions.

2. Actions:

To SA WG2 group.

ACTION: 	CT4 kindly asks SA2 to consider the above questions and provide clarification or answer to for questions.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm