3GPP TSG-SA WG2 Meeting #169	S2-2504599

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

Source:	OPPO

Title:	New Solution for KI#1: Discovery, selection and authorization of data collection UE.

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution on how to discover, select and authorize data collection UE.

- Discussion

SA2 has discussed and approved a R20 FS_AIML_CN_Ph2 SID which is captured in SP-250413. The aim of this study work is to investigate and identify potential architecture and system level enhancements to support AI/ML enhancements. Specifically, the objectives include:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

The following requirements have been defined in TR 23.700-04:

The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

Determination of the UEs used for data collection is very important for MNO to manage the UE-side data collection. In some scenarios, operators or AF may deploy dedicated UEs for data collection, even in that case, implementing a selection mechanism is critical to optimize efficiency, ensure data quality, and minimize operational overhead as stated below:

- Not all UEs have equal power reserves or processing capabilities. Selecting UEs with sufficient battery life or computational resources prevents premature device shutdowns or performance degradation.

- Operators may need data from UEs in specific environments (e.g., urban hotspots, rural dead zones) or experiencing particular issues (e.g., interference, handover failures). A selection mechanism ensures only relevant UEs are polled.

- Different AI/ML models require different data:

- Beam Management: Select UEs with multi-antenna capabilities.

- Positioning Accuracy: Prioritize UEs equipped with high-precision sensors (e.g., GPS, IMU).

- Channel State Feedback: Target UEs in diverse channel conditions (e.g., cell-edge vs. cell-center).

Observation 1: A UE selection mechanism ensures operators collect high-value data with minimal overhead, aligning with specific AI/ML goals while maintaining network performance and cost efficiency. Without such a mechanism, data quality, relevance, and operational sustainability would be compromised.

To obtain measurement data for AI/ML model training, the network may select one or multiple UEs for RRC measurements. And then the network collects data from these UEs. However, not all the UEs capable for data collection are trusted by the network to peform data collection. Restricting data collection to network-trusted and authorized UEs is critical for ensuring security, reliability, and compliance; otherwise, the following problems are identified:

- Untrusted UEs could inject false or manipulated data into the network, poisoning AI/ML models and degrading network performance (e.g., falsifying beam measurements to disrupt beamforming).

- Unauthorized UEs might intercept sensitive data (e.g., user locations, network configurations) during collection, exposing vulnerabilities.

- Trusted UEs are validated to ensure their sensors/hardware meet accuracy standards. Untrusted UEs may provide noisy or inconsistent data.

- Authorized UEs are selected based on predefined criteria (e.g., geographic distribution, mobility states), ensuring balanced datasets. Unauthorized UEs could skew data (e.g., over-representing static devices).

- Untrusted UEs might flood the network with unnecessary data, congesting the user plane (UP) or control plane (CP).

Observation 2: Authorizing only trusted UEs for data collection safeguards security, privacy, and data quality, ensuring AI/ML models are trained on reliable inputs.

Additionally, requiring explicit verification from a UE (or its user) before enabling data collection is crucial for ethical, legal, and operational reasons:

- Collected data (e.g., location, mobility patterns, device usage) may reveal personal information. Consent ensures compliance with privacy laws

- Explicit consent is mandatory under many jurisdictions for collecting, processing, or sharing user/device data.

- Users have the right to know what data is collected, how it will be used, and who can access it.

- Mandatory consent fosters trust between users and operators, reducing backlash or reputational harm.

- Data collection (e.g., frequent measurements) drains battery and consumes bandwidth. Consent allows users to avoid unintended resource costs.

Observation 3: Mandating UE availability verification aligns with ethical AI practices, legal obligations, and user-centric service design. It balances the network’s need for high-quality data with individual rights to privacy and control, ensuring sustainable and responsible AI/ML deployments.

Proposal: It is proposed to develop a solution of discovery, selection and authorization of data collection UE.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X | x |  |  |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: Discovery, selection and authorization of data collection UE

### 6.X.1	High level principles

This solution is for Key Issue#1 on discovery, selection and authorization of data collection UE.

Before the standardized data collection transfer process, 5GC network needs to determine which are the UEs that can collect the required data and that may perform measurement to collect the data based on requirements.

The UEs selected for data collection have to be qualified to perform the measurement to ensure security, privacy, and data quality while maintaining network performance and cost efficiency.

The UEs selected for data collection have to be available to perform the measurments to ensure reasonable resource consumption.

### 6.X.2	Description

The selection of data collection UE is performed by the core network element responsible for data collection, i.e. Data Collection Function (DCF). Upon receiving a service request from a data collection consumer, DCF first identifies candidate UEs for data collection before triggering the data collection process.

The selection involves two key steps:

- Selection of candidate data collection UE(s): determined based on pre-configured information, authorization, and UE context (e.g. device capabilities, location, and connectivity status).

- Determination of data collection UE: Final determination based on interaction with candidate data collection UE(s), which may involve verifying real-time availability, resource capacity, or network conditions.

The Selection of candidate data collection UE(s) are based on the following parameters:

- Candidate UE ID list locally preconfigured at DCF

- Authurised UE provided by UDM

- Candidate UE ID list provided by AMF

- UE context information obtained from AMF, including:

  - Vendor ID/Chipset ID generated from PEI

  - UE capability: e.g. UE data collection capability

  - NR RedCap Indication

  - UE Data Collection UP/DP connection information

  - Access Type

  - RM State

  - User location information: Registration Area，TAI of last Registration，User Location Information

  - Mobility Restrictions

After determining the candidate data collection UEs, the Data Collection Function (DCF) can dynamically interact with these candidate UEs to acquire relevant parameters and identify those that meet the requirements of the data collection consumer.

DCF first establishes connections (CP or UP) with each candidate data collection UE. It then requests the following information from each candidate UE:

- UE mobility: Static, low-speed, high-speed, or mobility rate.

- UE capabilities: Supported functionalities.

- UE current availability: Workload, battery level, power-saving mode status.

- UE current location: Geographic or network-based position.

- UE environment: Indoor or outdoor.

- User authorization: Whether the user permits the UE to perform data collection.

For authorization of UE for Data Collection, UE Data Collection Subscription data are stored and managed by UDM. The UE Data Collection Subscription data includes the following parameters:

- Purpose of data collection:

  - Model training: e.g. CSI, beam management, positioning

  - Data analytic

- PLMN

- Location: Cell ID, gNB, TAI List

- Time Period

- Device Type

- UE Availability verification indication

UE Data Collection Subscription data can be updated based on UE status and willingness of the UE user for data collection. The update can be triggered by UE or DCF.

### 6.X.3	Procedures

##### 6.X.3.1      Discover and selection of data collection UE

Figure 6.X.3.1-1: Discover and selection of data collection UE

- DCF receives a data collection service request, which may include candidate data collection UE list or a target data collection area

- a) If candidate data collection UE list is preconfigured or obtained from step 1, for each UE DCF requests UE context information (defined in 6.X.2) from its serving AMF

b) If no candidate data collection UE list is available, DCF requests candidate data collection UE list from the AMF who’s serving area matches the target data collection area, the request includes the following data collection requirements:

  - Vendor ID/Chipset ID generated from PEI

  - UE capability: e.g. UE data collection capability

  - NR RedCap Indication

  - UE Data Collection UP/DP connection information

  - Access Type

  - RM State

  - User location information: Registration Area，TAI of last Registration，User Location Information

  - Mobility Restrictions

AMF determines the candidate data collection UE(s) based on the above requirements and UE contexts, and responds to the DCF with the UE IDs matching the requirements.

- DCF determines the potential candidate data collection UE(s) based on information received from AMF

- DCF sends the dcata collection UE authorization request to the UDM, which includes UE ID(s) and Data collection purpose. UDM checks for each UE if it is authurised to perform data collection based on its subscription information, and responds to DCF with the following information:

  - UE ID

  - Whether UE is authurised to provide data collection service

  - additional information if the UE is authorised: authurised data collection purpose, authurised PLMN, authurised Location, authurised Time Period, authurised Device Type, UE Availability verification indication

- For each authurised data collection UE, if UE Availability verification indication is set, the DCF needs to verify UE availability. If DCF determines to interact with UE over UP connection, and there is no available UP connection, the UP connection is established

- DCF retrieves UE data collection availability information from the UE, which includes: UE mobility, UE capabilities, UE current availability, UE current location, UE environment, User authorization.

- DCF determines data collection UE(s) based on UE data collection availability information.

##### 6.X.3.2      Update of UE data collection authurization information

Figure 6.X.3.2-1: Update of UE data collection authurization information triggered by DCF

- If UE Availability verification indication is set, before triggering the UE to perform data collection, the DCF needs to verify UE availability. DCF retrieves UE data collection availability information from the UE, which includes: UE mobility, UE capabilities, UE current availability, UE current location, UE environment, User authorization, UE Availability verification indication.

- DCF sends UE data collection availability information received from UE to the UDM using Nudm_SDM_ModifySubscription service operation request.

- UDM stores the information and responds to DCF using Nudm_SDM_ModifySubscription service operation response.

Figure 6.X.3.2-2: Update of UE data collection authurization information triggered by DCF

- Based on UE own status (e.g., battery level, CPU load, storage, mobility status) and user consent, the UE determines a change in its suitability for data collection and sends a data collection UE subscription information update request to the UDM, including the following information: UE mobility, UE capabilities, UE current availability, UE current location, UE environment, User authorization, UE Availability verification indication.

- UDM stores the information and responds to UE.

The interaction between the UE and UDM in a UE-triggered data collection UE authorization update procedure can be implemented in the following ways:

  - As a container carried by the NAS (Non-Access Stratum) transport.

  - Reusing the UE-triggered location privacy setting procedure defined in TS 23.273 (Section 6.12.1, Steps 1–3) with the following extensions:

    - Step 1: include following information in the message: UE mobility, UE capabilities, UE current availability, UE current location, UE environment, User authorization, UE Availability verification indication

    - Step 2: include include following information in Nudm_ParameterProvision_Update: UE mobility, UE capabilities, UE current availability, UE current location, UE environment, User authorization,  UE Availability verification indication.

### 6.X.4	Impacts on services, entities and interfaces

DCF:

  - Support data collection UE selection

  - Support data collection UE availability verification

  - Support data collection UE authurization

AMF:

  - Support data collection UE selection

UE:

  - Support data collection UE availability verification

UDM:

  - Support data collection UE authurization

  - Support data collection UE availability management

* * * End of Change * * * *