SA WG2 Meeting #169	S2-250XXXX

# Fukuoka, Japan, 19-23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | XXXX | rev | --- | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.503 – TEI20_ DAMUP – Deferred AM and UE Policies in the 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, Verizon UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_DAMUP |  |  |  |  |  | Date: |  |  | 2024-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR updates 23.503 along the lines of TEI20 DAMUP WID (Deferred AM and UE Policies in the 5GC), as follows: - Add for AM policy the mechanisms of policy activation/deactivation in a deferred mode and policy revalidation time. - Add for UE policy the mechanisms of policy revalidation time. NOTE: UE policy (specifically URSP) already has a “built-in” mechanism of activation/deactivation via the time window attribute of the RSD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementing in 23.503 policy activation/deactivation in a deferred mode and policy revalidation time for AM and UE policy (URSP only). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AM will lack the ability to use the mechanisms of policy activation/deactivation in a deferred mode and policy revalidation time. UE policy will lack the ability to use the mechanism of policy revalidation time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.X(New), 6.1.2.Y(New), 6.5, 6.6.7, 6.1.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 6.1.2.X	Policy activation/deactivation in a deferred mode

The interaction between AMF and PCF related to access and mobility policy can be also performed in a deferred mode. An access and mobility policy may have either a single deferred activation time, or a single deferred deactivation time or both.

An AM policy with only a deferred activation time shall be inactive until that time. An AM policy with only a deferred deactivation time shall be active until that time. When the policy activation time occurs prior to the policy deactivation time, the policy is inactive until the activation and remains active until the deactivation time occurs. When the policy deactivation time occurs prior to the policy activation time, the policy is initially active until the deactivation time, then remains inactive until the activation time, and then becomes active again. An inactive AM, that has not been activated yet, is still considered to be installed, and may be removed by the PCF. When modifying a dynamic AM policy by setting, modifying or clearing its deferred activation and/or deactivation time or by changing any other attribute of an AM policy having a deferred activation and/or deactivation time, the PCF shall provide all information of that AM policy.

* * * * Start of 2nd Change * * * *

#### 6.1.2.Y	Policy Revalidation time

The (H-)PCF may instruct the AMF/V-PCF to contact it at a later time, so that the (H-)PCF will be able then to re-evaluate whether a new AM or UE policy decision has to be made. This is done via the policy control request trigger mechanism. Specifically, the (H-)PCF sets a revalidation time and provides it to the AMF/V-PCF along with Revalidation Timeout Policy Control Request Trigger.

The SMF shall start the timer based on the revalidation time and shall trigger an AM or UE policy request towards the PCF before the indicated revalidation time. Upon receiving such a policy request the (H-)PCF will re-evaluate the AM or UE policy and may or may not provide modified AM or UE policy to the AMF/V-PCF.

Prior to expiration of revalidation time the (H-)PCF can update or delete the current revalidation time at the AMF/V-PCF.

* * * * Start of 3rd Change * * * *

## 6.5	Access and mobility related policy information

To enable the enforcement in the 5GC system of the access and mobility policy decisions made by the PCF for the control as described in clause 6.1.2.1, the 5GC system may provide the Access and mobility related policy information from the PCF to the AMF.

Table 6.5-1 lists the AMF access and mobility related policy information.

Table 6.5-1: Access and mobility related policy information

| Information name | Description | Category | PCF permitted to modify in a UE context in the AMF | Scope |
| --- | --- | --- | --- | --- |
| Aggregate maximum bit rate | This part defines the aggregate maximum bit rate |  |  |  |
| UE-AMBR | This defines the UE-AMBR value that applies for a UE | Conditional (NOTE 5) | Yes | UE context |
| List of UE-Slice-MBR | This defines the List of UE-Slice-MBR (UL/DL) that each applies to the network slice of the UE. | Conditional (NOTE 8) | Yes | UE context |
| Service Area Restrictions | This part defines the service area restrictions |  |  |  |
| List of allowed TAIs. | List of allowed TAIs (NOTE 3) (NOTE 4). | Conditional (NOTE 1) | Yes | UE context |
| List of non-allowed TAIs. | List of non-allowed TAIs (NOTE 3). | Conditional (NOTE 1) | Yes | UE context |
| Maximum number of allowed TAIs | The maximum number of allowed TAIs. (NOTE 4) | Conditional (NOTE 1) | Yes | UE context |
| RFSP Index | This part defines the RFSP index related information |  |  |  |
| RFSP Index for Allowed NSSAI | Defines the RFSP Index associated with Allowed NSSAI that applies for a UE | Conditional (NOTE 2) | Yes | UE context |
| RFSP Index for Target NSSAI | Defines the RFSP Index associated with Target NSSAI that applies for a UE | Conditional (NOTE 2) | Yes | UE context |
| RFSP Index in Use Validity Time | Defines the time by which the RFSP Index will be used in MME after 5GS to EPS mobility. | Conditional (NOTE 2, NOTE 11) | Yes | UE context |
| 5G access stratum time distribution | This part defines the 5G access stratum time distribution |  |  |  |
| 5G access stratum time distribution indication | Defines if 5G access stratum time distribution via Uu reference point is enabled or disabled | Conditional (NOTE 9) | Yes | UE context |
| Uu interface time synchronization error budget | Indicates the Uu Time Synchronization error budget for 5G access stratum time distribution | Conditional (NOTE 10) | Yes | UE context |
| Clock quality detail level | Defines which clock quality information (clock quality metrics or acceptable/not acceptable indication) to report to the UE as defined in clause 5.27.1.12 of TS 23.501 [2] | Conditional (NOTE 9) | Yes | UE context |
| Clock quality acceptance criteria | Indicates the acceptable criteria as defined in clause 5.27.1.12 of TS 23.501 [2] | Conditional (NOTE 9) | Yes | UE context |
| SMF selection management | This part defines the SMF selection management instructions |  |  |  |
| DNN replacement of unsupported DNNs | Defines if a UE requested unsupported DNN is requested for replacement by PCF | Conditional (NOTE 6) | Yes | UE context |
| List of S-NSSAIs | Defines the list of S-NSSAIs containing DNN candidates for replacement by PCF | Conditional (NOTE 6) (NOTE 7) | Yes | UE context |
| Per S-NSSAI: List of DNNs | Defines UE requested DNN candidates for replacement by PCF | Conditional (NOTE 6) | Yes | UE context |
| Slice replacement management | Defines slice replacement management |  | Yes | UE context |
| S-NSSAI availability information | Defines the S-NSSAI availability and/or alternative S-NSSAI for S-NSSAI | Conditional (NOTE 12) | Yes | UE context |
| Slice Related Restrictions | Defines network policies for Slices subject to network control |  |  |  |
| List of S-NSSAIs | Defines the List of S-NSSAIs that are on demand | Conditional (NOTE 13) | No | UE context |
| Per S-NSSAI: Deregistration Inactivity Timer value | Defines the S-NSSAI deregistration inactivity timer value before removing the S-NSSAI from the Allowed Slices | (NOTE 14) | No | UE context |
| Charging related information | Defines information related to Charging |  |  |  |
| Charging information | Defines the containing CHF address and optionally the associated CHF instance ID and CHF set ID | Conditional (NOTE 15) | Yes | UE context |
| Deferred activation/deactivation time |  |  |  |  |
| Deferred activation time | Defines the time when an AM policy becomes active. (The AM policy shall be inactive until that time). | Optional | Yes | UE context |
| Deferred deactivation time | Defines the time when an AM policy becomes inactive. (The AM policy shall be active until that time). | Optional | Yes | UE context |
| Revalidation time limit |  |  |  |  |
| Revalidation time limit | Defines the time period within which the AMF shall send the PCF an AM policy re-evaluation request. | Optional | Yes | UE context |

The list of allowed TAIs indicates the TAIs where the UE is allowed to be registered, see clause 5.3.4 of TS 23.501 [2] for the description on how AMF uses this information.

The list of non-allowed TAIs indicates the TAIs where the UE is not allowed to be registered, see clause 5.3.4 of TS 23.501 [2] for the description on how AMF uses this information.

The Maximum number of allowed TAs indicates the maximum number of allowed Tracking Areas, the list of TAI is defined in the AMF and not explicitly provided by the PCF.

The RFSP Index for Allowed NSSAI and RFSP Index for Target NSSAI defines the RFSP Index for radio resource management functionality.

RFSP Index in Use Validity Time defines the time for which the RFSP Index in use will be used in MME after 5GS to EPS mobility as specified in clause 5.17.2.2 of TS 23.501 [2].

The UE-AMBR limits the aggregated bit rate across all Non-GBR QoS Flows of a UE in the serving network.

The list of UE-Slice-MBR defines the list of authorized UE-Slice-MBR allocated for a UE, how it is enforced is described in clause 5.7.1.10 of TS 23.501 [2].

The DNN replacement of unsupported DNNs indicates that the AMF shall contact the PCF for replacement of an unsupported DNN requested by the UE.

The List of S-NSSAIs defines the S-NSSAIs, valid in the serving network, of the Allowed NSSAI that contain DNN candidates for replacement by PCF.

The List of DNNs defines the DNN candidates for which the AMF shall contact the PCF for replacement if such a DNN is requested by a UE.

The 5G access stratum time distribution indicates the 5G access stratum time distribution parameters to be indicated to the NG-RAN via AMF.

The S-NSSAI availability information indicates whether the S-NSSAI is not available or is available, and/or an alternative S-NSSAI that the S-NSSAI can be replaced with.

The Charging information includes CHF address(es) and if available, the associated CHF instance ID(s) and/or CHF set ID(s). This is described in detail in clause 6.3.11 of TS 23.501 [2] and in clause 6.1.1.4.

The Deferred activation time includes the time when an AM policy becomes active.

The Deferred deactivation time includes the time when an AM policy becomes inactive.

The Revalidation time limit defines the time period within which the AMF shall send the PCF an AM policy re-evaluation request.

* * * * Start of 4th Change * * * *

### 6.6.7	UE Policy Association supplementary information

To enable the (H-)PCF to provide the V-PCF/AMF information that is related to UE Policy, but is to be consumed by the AMF itself (e.g. address information of CHF), the 5GC system may provide the UE Policy Association supplementary information from the (H-)PCF to the V-PCF/AMF.

Table 6.6.7-1 lists the UE Policy Association supplementary information.

Table 6.6.7-1: UE Policy Association supplementary information

| Information name | Description | Category | PCF permitted to modify in a UE context in the AMF/V-PCF | Scope |
| --- | --- | --- | --- | --- |
| Charging information | Information related to charging |  |  |  |
| Charging information | Defines the CHF address and optionally the associated CHF instance ID and CHF set ID. | Conditional (NOTE 1) | Yes | UE context |
| Revalidation time limit |  |  |  |  |
| Revalidation time limit | Defines the time period within which the AMF/V-PCF shall send the (H-)PCF a UE policy re-evaluation request. | Optional | Yes | UE context |
| NOTE 1: Shall be included If the home operator policies indicate that the same CHF is selected by the PCF for the UE and the AMF, otherwise optional. |  |  |  |  |

The Charging information includes CHF address(es) and, if available, the associated CHF instance ID(s) and/or CHF set ID(s). This is described in detail in clause 6.3.11 of TS 23.501 [2] and in clause 6.1.1.4.

The Revalidation time limit defines the time period within which the AMF/V-PCF shall send the (H-)PCF a UE policy re-evaluation request.

* * * * Start of 5th Change * * * *

#### 6.1.2.5	Policy Control Request Triggers relevant for AMF

The Policy Control Request Triggers relevant for AMF are listed in the tables below and define the conditions when the AMF shall interact again with PCF after the AM Policy Association Establishment or UE Policy Association Establishment.

The PCF provides Policy Control Request Triggers to the AMF indicating a specific UE (i.e. SUPI or PEI) in the Policy Association establishment and modification procedures defined in the TS 23.502 [3]. The Policy Control Request Triggers are transferred from the old AMF to the new AMF when the AMF changes.

The Policy Control Request Triggers are not applicable any longer at termination of the AM Policy Association or termination of UE Policy Association.

Table 6.1.2.5-1: Policy Control Request Triggers relevant for AMF and 3GPP access type

| Policy Control Request Trigger | Description | Condition for reporting |
| --- | --- | --- |
| Location change (tracking area) | The tracking area of the UE has changed. | PCF (AM Policy Association, UE Policy Association) |
| Change of UE presence in Presence Reporting Area | The UE is entering/leaving a Presence Reporting Area. | PCF (AM Policy Association, UE Policy Association) |
| Service Area restriction change | The subscribed service area restriction information has changed. | PCF (AM Policy Association) |
| RFSP index change | The subscribed RFSP index has changed. | PCF (AM Policy Association) |
| Change of the Allowed NSSAI | The Allowed NSSAI has changed. | PCF (AM Policy Association) |
| Generation of Target NSSAI | The Target NSSAI has been generated. | PCF (AM Policy Association) |
| Change of Partially Allowed NSSAI | The Partially Allowed NSSAI has changed. | PCF (AM Policy Association) |
| Change of S-NSSAI(s) rejected partially in the RA | The S-NSSAI(s) rejected partially in the RA has changed. | PCF (AM Policy Association) |
| Change of rejected S-NSSAI(s) for the RA | The rejected S-NSSAI(s) for the RA has changed. | PCF (AM Policy Association) |
| Change of Pending NSSAI | The Pending NSSAI has changed. | PCF (AM Policy Association) |
| UE-Slice-MBR change | The subscribed UE-Slice-MBR has changed. | PCF (AM Policy Association) |
| PLMN change | The UE has moved to another operators' domain. | PCF (UE Policy Association) |
| Slice replacement management | The AMF cannot determine the Alternative S-NSSAI for an S-NSSAI or, the AMF has determined/changed the Alternative S-NSSAI for an S-NSSAI after received the Alternative S-NSSAI from OAM or network slice availability notification from NSSF. | PCF (AM Policy Association) |
| Connectivity state changes | The connectivity state of UE is changed. | PCF (UE Policy Association) |
| NWDAF info change | The NWDAF instance IDs used for the UE or associated Analytics IDs used for the UE and available in the AMF have changed. | PCF (AM Policy Association) |
| Satellite backhaul category change | Satellite backhaul category changes between different types of satellite backhaul, or between satellite backhaul and non-satellite backhaul. | PCF (UE Policy Association) |
| LBO Information change | LBO Information (i.e. DNN(s) and/or S-NSSAI(s) that are allowed for LBO in VPLMN in SMF Selection Data) has changed. | PCF (UE Policy Association) |

Table 6.1.2.5-1a: Policy Control Request Triggers relevant for AMF and both 3GPP and Non 3GPP access type

| Policy Control Request Trigger | Description | Condition for reporting |
| --- | --- | --- |
| Access Type change (NOTE 1) (NOTE 2) | The Access Type or RAT Type or both Access Type and RAT Type changed, added, or removed. | PCF (AM Policy Association, UE Policy Association) |
| UE-AMBR change | The subscribed UE-AMBR has changed. | PCF (AM Policy Association) |
| SMF selection management | UE request for an unsupported DNN or UE request for a DNN within the list of DNN candidates for replacement per S-NSSAI. | PCF (AM Policy Association) |
| Configured NSSAI change | The Configured NSSAI has changed. | PCF (UE Policy Association) |
| Revalidation Timeout | Revalidation timer has timed out. AMF requests evaluation of AM or UE policy or V-PCF requests evaluation of UE policy. | PCF (AM Policy Association, UE Policy Association) |
| NOTE 1: RAT Type change is applicable only for AM Policy. NOTE 2: Corresponding triggers are defined for this PCRT at stage 3. |  |  |

Table 6.1.2.5-2: Policy Control Request Triggers relevant for AMF and Non 3GPP access type

| Policy Control Request Trigger | Description | Condition for reporting |
| --- | --- | --- |
| wrong non-3GPP access | UE has connected to a wrong non-3GPP access that does not match its subscribed S-NSSAI(s). | Always report |

NOTE 1:	In the following description of the Policy Control Request Triggers relevant for AMF, the term trigger is used instead of Policy Control Request Trigger where appropriate.

If the Location change trigger are armed, the AMF shall activate the relevant procedure which reports any changes in location as explained in clause 5.6.11 of TS 23.501 [2] by subscribing with the Npcf_AMPolicyAssociation service or Npcf_UEPolicyAssociation service. The reporting is requested to the level indicated by the trigger (i.e. Tracking Area). The AMF reports that the Location change trigger was met and the Tracking Area identifier.

If the Change of UE presence in Presence Reporting Area trigger is armed, i.e. the PCF subscribed to reporting change of UE presence in a Presence Reporting Area, including a list of PRA ids. In addition, for "UE-dedicated Presence Reporting Area" a short list of TAs and/or NG-RAN nodes and/or cells identifiers is included. Then, the AMF shall activate the relevant procedure which reports any Change of UE presence in Area of Interest as explained in clause 5.6.11 of TS 23.501 [2]. The reporting is requested for the specific condition when target UE moved into a specified PRA. The AMF reports the PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s) to the PCF.

The Service Area restriction change trigger and the RFSP index change trigger shall trigger the AMF to interact with the PCF for all changes in the Service Area restriction or RFSP index data received in AMF from UDM. The reporting includes that the trigger is met and the subscribed Service Area restriction or the subscribed RFSP index provided to AMF by UDM, as described in clause 6.1.2.1.

The Change of the Allowed NSSAI trigger shall trigger the AMF to interact with the PCF if the Allowed NSSAI has been changed. The reporting includes that the trigger is met and the new Allowed NSSAI. The PCF may update RFSP index and/or SMF selection management related policy information (described in clause 6.5) in the AMF based on the Allowed NSSAI.

The Generation of a Target NSSAI trigger shall trigger the AMF to interact with the PCF. The reporting includes that the trigger is met and the generated Target NSSAI. The PCF may generate RFSP index associated with the Target NSSAI.

The Change of the Partially Allowed NSSAI trigger shall trigger the AMF to interact with the PCF if the Partially Allowed NSSAI has been changed. The reporting includes that the trigger is met and the new Partially Allowed NSSAI. The PCF may update RFSP index related policy information (described in clause 6.5) in the AMF based on the Partially Allowed NSSAI.

The Change of the S-NSSAI(s) rejected partially in the RA shall trigger the AMF to interact with the PCF if the S-NSSAI(s) rejected partially in the RA has been changed. The reporting includes that the trigger is met and the new S-NSSAI(s) rejected partially in the RA. The PCF may update RFSP index related policy information (described in clause 6.5) in the AMF based on the S-NSSAI(s) rejected partially in the RA.

The Change of the rejected S-NSSAI(s) for the RA shall trigger the AMF to interact with the PCF if the rejected S-NSSAI(s) for the RA has been changed. The reporting includes that the trigger is met and the new rejected S-NSSAI(s) in the RA. The PCF may update RFSP index related policy information (described in clause 6.5) in the AMF based on the rejected S-NSSAI(s) for the RA.

The Change of the Pending NSSAI trigger shall trigger the AMF to interact with the PCF if the Pending NSSAI has been changed. The reporting includes that the trigger is met and the new Pending NSSAI. The PCF may update RFSP index related policy information (described in clause 6.5) in the AMF based on the Pending NSSAI.

If the Configured NSSAI change trigger is armed, the AMF shall report the Configured NSSAI and mapping of each S-NSSAI of the Configured NSSAI to corresponding HPLMN S-NSSAI values as defined in clause 5.15.4.1.1 of TS 23.501 [2] to the PCF. The V-PCF sends the HPLMN S-NSSAI to the H-PCF after mapping the S-NSSAI of the VPLMN into the S-NSSAI of the HPLMN as described in clause 4.15.6.7 of TS 23.502 [3]. The H-PCF may take this into account to update UE Policy as defined in clause 6.1.2.2. When the UE is connected to a non-3GPP access, the PCF may take this into account to update UE Policy e.g. ANDSP as defined in clause 6.1.2.2.

The UE-AMBR change trigger shall trigger the AMF to interact with the PCF for all changes in the subscribed UE-AMBR data received in AMF from UDM. The reporting includes that the trigger is met and the subscribed UE-AMBR provided to AMF by UDM, as described in clause 6.1.2.1.

The Slice-UE-MBR change trigger shall trigger the AMF to interact with the PCF for all changes in the Subscribed UE-Slice-MBR for each subscribed S-NSSAI in the NSSAI with a Subscribed UE-Slice-MBR received at the AMF from UDM. The reporting includes that the trigger is met, as described in clause 6.1.2.1.

If the PLMN change trigger is armed, the AMF shall report it to the PCF to trigger the update of V2X service authorization parameters to the UE as defined in clause 6.2.2 of TS 23.287 [28], to trigger the update of ProSe authorization parameters to the UE as defined in clause 6.2.2 of TS 23.304 [34], to trigger the update of A2X service authorization parameters to the UE as defined in clause 6.3.2.2 of TS 23.256 [43] and to trigger the update of Ranging/Sidelink Positioning authorization parameters to the UE as defined in clause 6.2.2 of TS 23.586 [41]. The reporting includes the event with the serving PLMN ID.

If the SMF selection management trigger is set, then the AMF shall contact the PCF when the AMF detects that the UE requested an unsupported DNN and the PCF indicated DNN replacement of unsupported DNNs in the Access and mobility related policy information (see clause 6.5). The PCF shall select a DNN and provide the selected DNN to the AMF.

If the SMF selection management trigger is set, then the AMF shall contact the PCF when the UE requested a DNN within the list of DNN candidates for replacement for the S-NSSAI indicated in the Access and mobility related policy information (see clause 6.5). The PCF shall select the DNN and provide the selected DNN to the AMF.

If the slice replacement management trigger is set, the AMF shall contact the PCF:

-	in Case i) when AMF cannot determine the Alternative S-NSSAI for the S-NSSAI(s), e.g. NSSF doesn't provide Alternative S-NSSAI and there is no Alternative S-NSSAI in the AMF local configuration. The reporting includes that the trigger is met, the S-NSSAI(s) that requires slice replacement, as described in clause 6.1.2.1.

-	in Case ii) if the AMF can determine the Alternative S-NSSAI for the S-NSSAI(s) when network slice replacement applies for some S-NSSAIs, or if the AMF determines to stop the network slice replacement for some S-NSSAI(s) after receiving the Alternative S-NSSAI from OAM or network slice availability notification from NSSF. The reporting includes the trigger is met and S-NSSAI(s) that requires slice replacement and the corresponding Alternative S-NSSAI. If no corresponding Alternative S-NSSAI is provided, the PCF is informed no Alternative S-NSSAI is mapped to the replaced S-NSSAI(s) any longer.

NOTE 2:	The Alternative S-NSSAI does not have to be a Subscribed S-NSSAIs.

NOTE 3:	AMF receiving network slice replacement notification from the PCF is not a trigger for the AMF to update the PCF with the mapping information between Alternative S-NSSAI and replaced S-NSSAI.

If the Connectivity state changes trigger is set, then the AMF shall notify the PCF when the UE connectivity state is changed e.g. from IDLE to CONNECTED. The AMF then reset the trigger.

The NWDAF info change trigger shall trigger the AMF to interact with the PCF when the list of NWDAF Instance IDs used for the UE or associated Analytics IDs used for the UE at the AMF are changed in the AMF.

If the Satellite backhaul category change trigger is armed, the AMF shall report the Satellite backhaul category to indicate the change between different types of satellite backhaul, or the change between satellite backhaul and non-satellite backhaul (as specified in clause 5.43.4 of TS 23.501 [2]) to the PCF. The PCF may take this into account to update UE Policy as defined in clause 6.1.2.2.

The AMF indicates a PCRT corresponding to wrong non-3GPP access when the UE has connected to a non-3GPP access that is not supporting the configured NSSAI. The AMF also indicates whether it is for untrusted or trusted non-3GPP access. This triggers the PCF to update the relevant policies on the UE e.g. WLANSP or Non-3GPP access network selection information.

If the LBO Information change is armed, the AMF shall report the LBO Information (i.e. DNN(s) and/or S-NSSAI(s) that are allowed in VPLMN for LBO roaming in SMF Selection Data) to the PCF when there is change in the LBO Information.

If the Access Type change trigger is met, the AMF reports the changed, the added or the removed Access Type and/or the changed RAT Type to the PCF.

If the (H-)PCF had previously provided the AMF or the V-PCF Revalidation timer, and it has timed out the AMF requests evaluation of AM or UE policy or V-PCF requests evaluation of UE policy.

* * * * End of Changes * * * *