SA WG2 Meeting #169	S2-2504783

19 - 23 May, 2025, Fukuoka, Japan

Title:	[draft] Reply LS on Questions on stage 2 requirements for AIML_CN

Response to:	C4-250630 LS on Questions on stage 2 requirements for AIML_CN

Release:	Rel-19

Work Item:	AIML_CN

Source:	vivo (to be SA2)

To:	CT4

Cc:	CT3

Contact person:	Vivian Chong

vivian.chong@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2504784

1	Overall description

SA2 thanks CT4 for the LS C4-250630 on Questions on stage 2 requirements for AIML_CN.

After discussion, SA2 provides answers for the 4 Questions:

Q1: Given that the metrics can be reported by OAM APIs, is it necessary for the NRF APIs to also support reporting the same metrics, and if so, can SA2 explain why?

A1: Yes, the information outlined in Table 6.22.2-5 is crucial for detecting and predicting signalling storms caused by NF. Given that different vendors may implement OAM differently, it's possible that OAM might lack these capabilities. The CT4's work on standardizing the NRF will be beneficial in implementing the capability to detect and predict signalling storms caused by NF.

Q2: If the answer to Q1 is yes, this required functionality does not fit with the current scope of the Nnrf_NFManagement_NFStatusSubscribe service operation. CT4 kindly asks SA2 to clarify which service operation is suitable for this purpose?

A2: For the SCP, a new service called Nscp_EventExposure has been added to support information reporting to the NWDAF for similar cases. SA2 proposes to introduce a new event exposure service for NRF.

Q3: Accordingly, should NF metrics related to signalling storm information be reported per Service Name (i.e. with the NF instance aggregating metrics of all NF service instances supporting the same service name) or per NF service instance?

A3: Per NF service instance. The information detailed in Table 6.22.2-1 is gathered to detect or predict signaling storms caused by UE. The UE may repeatedly send NAS signaling from a specific AMF through AUSF to UDM. Therefore, the metrics of type and number of requests per NF service instance will help NWDAF to identify abnormal from the different AMF or AUSF, enabling NWDAF to reconstruct the entire signaling path to identify the UE.

Q4: Does “Service Type” refer to “service name”? If the answer is yes, should SCP metrics related to signalling storm information be reported per "Service Name" or "Service instance"? If the answer is no, what does "Service Type" refer to?

A4: Yes. As outlined in the above A3, SCP metrics should be reported per NF service instance. This approach allows NWDAF to collect more granular data, facilitating the detection or prediction of signaling storms caused by a UE or a specific NF.

Besides the above answers, SA2 has approved S2-2504784 in the attachment for clarification on the aspects above.

2	Actions

To CT WG4:

ACTION: 	SA2 kindly asks 3GPP CT4 to take the above information into consideration

3	Dates of next TSG SA WG 3 meetings

SA2#170				        25-29 Aug 2025				     Goteborg, Sweden

SA2#171				        13-17 Oct 2025				     Wuhan, China