3GPP TSG-SA2 Meeting #169	S2-2504893

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Energy Consumption information collection from SMF

Document for:	Discussion

Agenda Item:	19.4.2

Work Item / Release:	EnergySys

Abstract: UPF Event Exposure can be used for information collection from SMF for Energy Consumption with no impact on PCC Rules.

# 1. Introduction

Energy Consumption information collection from SMF has been discussed in SA2 #168. This data Collection is not related to Policy or Charging Control however the solutions discussed impacted the rules that SMF installs in UPF and that SMF determines mostly from PCC Rules from PCF or other Local Policies.

UPF Event Exposure service can be used for Data Collection from UPF, for example to collect information of data volumes. This UPF service can be subscribed via SMF and SMF subscribes to UPF on behalf of the consumer using SBI. UPF reports the information directly to the consumer. Analysis follows for how this alternative solution can solve the Energy Consumption Use Case

# 2. Discussion

According to TS 23.501 clause 5.51.2.2.2	(“Energy Consumption information collection from SMF”)

“The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1. The SMF receives the data volume of the required granularities from the PSA UPF. The SMF then sends the collected data volume, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. And the information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2.“

This solution has some issues. The procedure assumes that,

-   SMF provides the IDs of the gNB and UPF(s) in the data path, and for that SMF maps cell ID to gNB ID, (see TS 23.502 clause 4.29.2, “The SMF can derive gNB ID(s) according to the AMF provided ULI as described in clause 4.3.2”). But this mapping is not needed for other purposes. Furthermore, in scenarios with I-SMF, SMF may not receive ULI when user changes Cell (and maybe gNB). In addition, SMF does not receive the I-UPF ID, but I-UPF IP address as part of the UP tunnelling information.

-   SMF correlates the EIF subscription request with the PCC Rules in order to install rules in UPF with right precedence for reporting and respecting the rules for policy control and charging needs. This correlation is not straight forward.

-   SMF receives and processed all the data reports from UPF in order to send then these data reports to their actual destination which is EIF.

It is worth to discuss whether, for Energy Consumption, UPF Event Exposure service can be used instead of URR/UR Reports over N4 and have a more efficient and simpler solution (e.g. data volume reports are sent directly to EIF by UPF) that also solves the issues listed above.

The Energy Consumption information collection from SMF requirements that can be derived from above information and descriptions in clause 5.51.2.2 are:

1)  In the request by EIF,

- The PDU Session(s) of interest are identified by UE ID, and S-NSSAI and DNN, and

- The Target traffic is identified by a Packet Filter or application identifier.

2)  The report(s) to EIF include,

a) The UL/DL Data Volume measured for the target traffic and the time interval when this has been measured

b) The UE ID (ie. SUPI) and traffic Application Identifier

c) The ID of the gNB ID and UPFs (UPF PSA and optionally I-UPF(s)) that compose target traffic data path.

3) In addition, Node level energy consumption is collected PLMN-wide at a configurable starting time with interval T such that the Energy Consumption information reported to the EIF is time-aligned along the system.

Analysis follows for how a UPF Event Exposure based solution for the Energy Consumption Use Case would look like:

With baseline SMF and UPF Event Exposure services, a cNF can subscribe to UPF User Data Usage Measures event and request data volume according to 1) and receive reports according to 2)(a&b). The SMF needs to be configured to provide the UE ID (ie. SUPI) as part of the PFCP Session information sent to UPF so UPF can include it in the reports. UPF includes the Application identifier for the service data flow if available. However,

- UPF is not aware of gNB or other UPFs in the data path and can not include this information in the reports (neither is SMF aware in all cases as described above)

- Also UPF does not take measurements in specific preconfigured periods for User Data Usage Measures event.

Therefore, to comply with the listed requirements using UPF Event Exposure service, the SMF/UPF event exposure services would need to be enhanced. The proposed solution enhances these services as follows:

-  The SMF Event Exposure subscription to UPF event User Data Usage Measures includes an indication that the report is required with data path. It also includes an indication that the system preconfigured measurement intervals shall be used for the measurements.

-  Also the subscription to UPF event User Data Usage Measures includes indication that the report is required with data path. It also includes an indication that the system preconfigured measurement intervals shall be used for the measurements:

-  If data path is required, the UPF considers, if available, the data Path information of the PFCP Session. Mutliple reports are sent if the data path changes during the measurement period.

-  If indication of system preconfigured intervals is received, UPF considers it for the measurements. This way, the UPF measurements are taken on same intervals in the whole system to guarantee they are in aligned.

In addition:

-  The PFCP Session information can include as part of a PFCP Session establishment or update the data path information (the ID or address (depending on the information available in SMF) of the (I-)UPFs that compose the data path of traffic steered through this UPF PSA).

-  The AMF event exposure service is extended to provide gNB ID as part of user location information Then, EIF can get the information from AMF directly (how EIF splits data volumes between gNBs if it changes during the measurement period is left to EIF implementation).

With these updates, the following procedure can be used for Energy Consumption information collection instead of the procedure in TS 23.502 clause 4.29.2:

As a prerequisite, EIF subscribes to the energy consumption per node and total data volume per node from OAM.

1.	The Consumer NF (i.e. trusted AF or 5GC NF) subscribes to the energy consumption event with the EIF by using Neif_EventExposure_Subscribe service for the required granularities (UE, PDU Session and/or QoS flow) as described in clause 5.51.2.3 of TS 23.501 [2].

If the Consumer is untrusted AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF IDs and AMF ID for the required granularity.

3. The EIF invokes the Namf_EventExposure_Subscribe request to the AMF to request the information of the user location in terms of gNB.

4.	The EIF invokes the Nsmf_EventExposure_Subscribe request to the SMF to request the information as defined in clause 5.51.2 of TS 23.501 [2] by subscribing the UPF User Data Usage Measures event and requesting volume measurements. Data path reporting is requested. An indication that configurable starting time with interval T should be used for the measurements is also included.

NOTE 1:	The SMF is assumed to provide the data path in the PFCP session when this solution is deployed in the network.

5. The SMF invokes the Nupf_EventExposure_Subscribe request to the UPF for User data Usage Measures requesting the PSA UPF to report data volume optionally with an event filter for the target traffic (packet filter or applicaton ID). SMF indicates that Per Data path in reporting is requested. The subscription also indicates that configurable starting time with interval T should be used for the measurements.

6. The UPF provides the data volume information for energy consumption calculation (see 5.51.2 of TS 23.501 [2]) to EIF by invoking Nupf_EventExposure_Notify service operation for the required granularity. The gNB Id information has been obtained by EIF from AMF directly.

NOTE 2:	When the data path changes for an ongoing subscription, SMF updates the PFCP session which triggers UPF to close the measurement report and start a new one. The UPF reporting include the data path information.

NOTE 3:	It is left for EIF implementation how the reported data volume is split between gNBs when gNB changes in the middle of the measurement period.

NOTE 4:	It order to avoid simulatenous reporting at end of the period causes signalling storms, the mechanisms specified in 4.15.4.5.1 to differ delay tolerant reporting should be used.

7.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

# 3. Proposal

Specify a procedure for Data Collection for Energy Consumption using UPF Event Exposure service instead of URR/UR Reports over N4. It is more efficient and simpler (e.g. data volume reports are sent directly to EIF by UPF) and solves the issues identified.

2 CRs (S2-2504894 and S2-2504895) are submitted to update 23.501 and 23.502 according to the procedure described above.