3GPP TSG-WG SA2 Meeting#169	S2-2504924

# Fukuoka, JP, 19 - 231 May 2025												(revision of S2-xxxxxxx)

| R-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0736 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter PLMN GMLC communication via NEF removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During the retrieval of the MWAB-UE location between GMLC in the MWAB broadcasted PLMN and GMLC in the MWAB-UE HPLMN, the option of sending request via NEF is based on the assumption that the GMLC can act as external AF NF (e.g. supporting N33 interface towards NEF including configuration related to external AF authorization mechanism). This is a new path of communication between GMLCs and a totally new behaviour for 5GC internal NF acting as external AF. This creates extra burden/logic on the GMLC as it is only a 5GC internal NF today. With so many options already specified to retrieve the MWAB-UE location, it’s unnecessary to specify this additional option considering the complexity introduced to the GMLC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the NEF related options in corresponding clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Introduce unnecessary additional functionality in the GMLC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.19.2, 5.19.3, 6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.19.2	Obtaining location information for the MWAB

There are multiple options for an LMF to obtain the location information and velocity of the MWAB(s):

-	The LMF can derive the location and velocity of the MWAB by using NRPPa or by requesting the GMLC in MWAB Broadcasted PLMN to derive the location of the MWAB-UE using the UE-ID of the MWAB-UE. The GMLC in MWAB Broadcasted PLMN requests the location of the MWAB-UE by requesting the GMLC in MWAB-UE HPLMN which is responsible to trigger MT-LR procedure as specified in clause 6.1.1 or clause 6.1..

-	As the timing of the location estimations for the Target UE and MWAB(s) is important for the quality of the location estimation of the Target UE, the LMF needs to reduce the timing offset of the positioning measurements, i.e. the positioning of the Target UE and MWAB can be scheduled with using the same scheduled location time and compensate for the potential time difference of the positioning measurements, e.g. taking velocity of MWAB into account.

# * * * * Next change * * * *

### 5.19.3	Privacy check for MWAB

If the positioning of the MWAB is performed for the location estimation of a Target UE different from a MWAB-UE), the privacy check in clause 5.4 is skipped for the MWAB-UE. When the LMF requests the GMLC in MWAB Broadcasted PLMN to derive the location of the MWAB-UE, the LMF includes in the request to the GMLC a MWAB indication indicating the location of MWAB is requested to determine the location of a Target UE in the location request. The GMLC in the MWAB-UE HPLMN receives the MWAB indication from the GMLC of the Broadcasted PLMN. Based on the MWAB indication, the GMLC in MWAB-UE HPLMN skips the privacy check.

If the positioning of the MWAB-UE is performed for the location estimation of the MWAB-UE itself the UE privacy check procedure in clause 5.4 is performed.

# * * * * Next change * * * *

### 6.1.5	5GC-MT-LR procedure involving MWAB

Figure 6.1.5-1 illustrates the network positioning requested by the LCS clients when MWAB(s) is involved.

In this scenario, it is assumed that the target UE may be identified using an SUPI or GPSI. The procedure follows the functionality in clause 5.19 and the 5G-MT-LR procedures in clauses 6.1.1 and 6.1.2. It is further assumed that:

-	The NG-RAN in figure 6.1.5-1 is a MWAB-gNB.

-	When the MWAB-gNB is integrated to the 5GS, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a MWAB-gNB is mobile and its UE ID (GPSI) of the MWAB-UE via a TRP information exchange towards the MWAB-gNB with the Cell ID of the TRP. The LMF may also performs NRPPa TRP information exchange procedure before the positioning procedure for a target UE, e.g. to determine the position capability of NG-RAN.

-	The LMF that performs the location estimation of the MWAB (MWAB-UE) can be different (e.g. even in different PLMN) than the LMF that performs the location estimation of the target UE (not shown in figure 6.1.5-1).

Figure 6.1.5-1: 5GC-MT-LR procedure involving MWAB

NOTE 1:	In this procedure one MWAB is assumed, however multiple MWABs could be involved (in which case the steps of location of MWAB are repeated for each involved MWAB).

1.	The location services client sends a request to the (H)GMLC for a location for the target UE identified by an GPSI or an SUPI. The UE may or may not be served by a MWAB.

2.	If the UE was roaming in the Broadcasted PLMN, the (H)GMLC of the target UE requests the Broadcasted PLMN GMLC to provide the UE location by a Ngmlc_Location_ProvideLocation_Request. Otherwise the GMLC of the Broadcasted PLMN and the (H)GMLC of the target UE are the same.

3.	The GMLC in MWAB Broadcasted PLMN invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF serving the target UE to request the current location of the target UE. The location procedure may be also triggered at the AMF for a 5GC-NI-LR as at step 1 in clause 6.10.1, or an event report sent by the UE for a deferred 5GC-MT-LR for periodic or triggered location as at step 25 in clause 6.3.1.

4.	The 5GC-MT-LR procedure as specified in clause 6.1.1 step 5-8 or clause 6.1.2 step 6-12 are performed with the difference that the LMF selection also consider LMF capability supporting MWAB involvement. If the AMF is aware that the target UE is served by a MWAB, it would select the LMF that can support the MWAB handling.

NOTE 1:	The steps 5 to 12 may happen as part of step 4.

5.	LMF derives if any MWAB(s) is involved in the positioning of the target UE based on the cell-ID used for positioning measurements in the step 4. The AMF serving the target UE may indicate that the serving cell is an MWAB (if applicable). When the MWAB-gNB was integrated as a TRP, the LMF may determine from in a TRP information exchange procedure that the cell-ID belongs to the MWAB and/or the UE-ID (GPSI) associated with MWAB-UE. The LMF, when the OAM provides a new Cell Id to the LMF, performs a TRP information exchange as specified in TS 38.455 [15] for the new Cell ID. When the MWAB-gNB is integrated in the 5GS, the Cell ID(s) of the MWAB is (are)provided to the LMF. If the LMF selects any cell for UE positioning measurements, and the LMF determines it needs to perform a TRP information exchange with the cell, it shall perform TRP Information Exchange procedure as specified in TS 38.455 [15].As the MWAB can be mobile the LMF may need to determine an updated location of the MWAB by either performing steps 6-8 (option 1) or performing steps 9-11 (option 2) if option 1 is not feasible.

NOTE 2:	It is assumed that for the case of MWAB mobility, the OAM can timely update the Cell Id of the MWAB to the LMF so as to enable it to perform a proper TRP information exchange. Until the Cell Id of the MWAB is provided to LMF by OAM, the Cell of the MWAB is not used for location procedures.

6.	[Conditional] The LMF initiates a NRPPa TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MWAB(s) by directing the TRP information exchange messages to the MWAB-gNB associated with the Cell ID(s), for example, by setting the NRPPa TRP Information Type Item IE to "mobile TRP location info".

7.	[Conditional] The MWAB-UE may initiate a 5GC-MO-LR procedure (as defined in clause 6.2) to get its location information if requested by MWAB-gNB based on implementation. The MWAB-UE's location info is used by the co-located MWAB-gNB to determine the updated location for its TRP.

8.	[Conditional] The MWAB-gNB report its updated TRP location, e.g. TRP's geo-coordinate, velocity and the time for obtaining them, to the LMF.

9.	[Conditional] If the LMF invokes the Ngmlc_Location_ProvideLocation_Request service operation to the GMLC in MWAB Broadcasted PLMN by providing the identifier of MWAB-UE, a MWAB Indication indicating the location request is for a MWAB-UE and a scheduled location time. The LMF determines the identifier of MWAB-UE based on the Cell ID of MWAB-gNB and the received the GPSI of the MWAB-UE in a TRP information exchange when the MWAB-gNB was integrated as a TRP. LMF may receive the scheduled location time in step 4. Otherwise, LMF may generate the scheduled location time, e.g. based on response time.

10.	[Conditional] The GMLC in BH PLMN determines the AMF serving the MWAB-UE and steps 5-11 in clause 6.1.1 or steps 4-23 in clause 6.1.2 is performed. The privacy check in clause 5.4 is skipped for the MWAB based on the MWAB Indication received at step 9. The scheduled location time received in step 9 via the LMF is also provided to the MWAB-UE when sending a location request to the MWAB-UE. When the MWAB-UE is roaming, the GMLC of the UE may further retrieve the location information of the MWAB-UE via the GMLC in MWAB-UE Broadcasted PLMN which is responsible to retrieve the location information of the MWAB-UE via the GMLC in BH PLMN, based on the information retrieved from UDM of the MWAB-UE (i.e. the serving AMF address and the GMLC address serving the MWAB-UE in BH PLMN).

11.	[Conditional] The GMLC in MWAB Broadcasted PLMN sends the location service response including the MWAB-UE's location to the LMF of the broadcasted PLMN. When the MWAB-UE is roaming, the GMLC in MWAB Broadcasted PLMN receives the location response from the GMLC in MWAB-UE HPLMN.

12.	[Conditional] The LMF performs one of the positioning procedures with the target UE described in clauses 6.11.1 and 6.11.2. To reduce the timing offset of the positioning measurements, the UE positioning may be scheduled with the same scheduled location time as the MWAB-UE positioning in step 10. LMF sends the same time with the scheduled location time in step 9 to the target UE in clause 6.11.1. If Network Assisted procedure is used, the MWAB-gNB may provide the MWAB-UE updated location and velocity information and the time obtained them to the LMF via NRPPa message as defined in TS 38.455 [15].

13.	The LMF uses the received location and velocity of the MWAB when estimating the location of the target UE together with the measurements reports in step 4 or optionally the updated measurement reports in step 12.

14.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the target UE.

15.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC to return the current location of the target UE.

16.	If the target UE was roaming in the Broadcasted PLMN, the GMLC of the Broadcasted PLMN responds to the (H)GMLC of the UE to provide the target UE location in a Ngmlc_Location_ProvideLocation_Response. Otherwise the GMLC of the Broadcasted PLMN and the (H)GMLC of the UE are the same.

17.	The (H)GMLC of the target UE sends the location service response to the location services client.

# * * * * End of changes * * * *