3GPP SA WG2#169	S2-2505041

Fukuoka, Japan, 19-23 May, 2025

Source:	Tejas Networks Limited

Title:	Enhancements to A-IoT Command Procedure

Spec:	3GPP TS 23.369

Agenda item:	19.14.2

Work Item / Release: AmbientIoT-ARC / Rel-19

Document for:	Discussion and decision

Abstract of the contribution: This contribution is to discuss the enhancement of the AIoT Command Procedure for enabling more efficient operation.

1. Introduction

3GPP TS 23.369 defines two AIOTF services viz., the Inventory service and Command service. Inventory service is used to discover the list of operational AIoT devices that are present in the network and obtain their identification (i.e., AIoT device permanent IDs). Once the list of operational devices is obtained by AIOTF, Command procedure may be used to perform a device-specific operation such as read, write and disable. However, these procedures need to be enhanced to improve the usage of radio resources and enable error handling. To better understand the need for these enhancements, let us consider the following scenario illustrated in Figure 1., wherein an AIOTF wishes to alter the configuration of two specific devices namely, Device 1 and Device2 for an Inventory cum follow-on command procedure.

Figure 1: Example scenario for Inventory procedure with a follow-on command.

The following set of actions occur within the network for processing this request. The AIOTF sends an Inventory Request with a Follow-onCommand as per the procedure described in clause 6.2.3 of 3GPP TS 23.369.

- As per this procedure, the AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Device Permanent Identifier, and assistance information to the selected NG-RAN node.

- On receiving the Inventory Request message, the RAN readers associated with the NG-RAN transmit a paging message that includes the AIoT Device Permanent Identifier. If the AIoT Device Permanent Identifier in the paging message matches that of Device 1 and Device 2, they respond to the NG-RAN paging request and send an Inventory Report (NAS message) including their device IDs to the AIOTF.

- On receiving the Inventory Report from these two devices, the AIOTF can proceed to send out a Command Request with a “write” operation.

We can make the following observation from the above scenario:

[Observation-1]]: In the case where the Inventory procedure is successfully completed and the “Command Requests” to both Device1 and Device 2 have been sent, the AIOTF will no longer have any commands to be sent to other devices in the network. However, since the devices and the RAN is unaware of this, the devices continue to wait for a follow-up command. In this case, the devices remain busy and unable to respond to other requests until the NG-RAN triggering time is completed for each device. To resolve this issue, we make the following proposals

[Proposal1]: Include information for maximum waiting time for “Command Response” in the Command Assistance Information provided to NG-RAN. The NG-RAN can then release the NGAP A-IoT device IDs at the end of the time provided.

* * * * First Change * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command) .

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Optionally, approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

e) Optionally, maximum wait time interval for the command response.

Editor's note:    Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e., in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b) and c), AIoT Device responses aggregation by using bullet d).

* * * * End of Changes * * * *

[Proposal2]: Send a “Command Release” message to the NG-RAN to release the remaining A-IoT Device NGAP ID contexts. This will signal to the NG-RAN that no further “Command Requests” will be sent relating to the present Correlation ID.

Figure 2: Proposed Command Release Message to indicate end of Procedure.

Note 1: The format of the “Command Release” message will have to be decided by RAN3.

Note 2: The manner in which NG-RAN then uses this information to inform the devices is to be coordinated by RAN2

[Proposal3]: Send a “Last Command” Indication  in the “Command Request” message sent by the AIOTF to last AIoT device to indicate the end of the Inventory cum follow-on command procedure for the present Correlation Id.

Note 3: The format of the “Last Command” indication will have to be decided in co-ordination with RAN3.

Note 4: The manner in which NG-RAN then uses this information to inform the devices is to be coordinated by RAN2