3GPP TSG-WG SA2 Meeting #169	S2-2505146

19th – 23rd May 2025, Fukuoka, Japan	(revision of 25XXXXX)

Source:	Lenovo, Nokia

Title:	Discussion: Way forward for Transport Level Marking at I-UPF

Document for:	Discussion

Agenda Item: 	19.3.2

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract of the contribution: This paper proposes to discuss the issue of DSCP value marking at the I-UPF.

- Background

During the SA2#168 meeting, an LS from CT4 was received (C4-250629/S2-2502815) asking about how transport level marking should be performed by I-UPF, and the original texts from the LS are copied below:

CT4 discussed the following SA2 requirements in clause 5.8.2.7 of 3GPP TS 23.501 for the Transport Level Marking on N3/N9.

5.8.2.7    PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF.

For QoS Flows that are configured for PDU Set QoS handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values. When an I-SMF/I-UPF is inserted in the PDU Session, the I-SMF may instruct the I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet.

CT4 would like to ask if the highlighted requirement in yellow implies a new requirement for N4 interface specific for XRM_Ph2?

If so, how can the I-SMF learn from the anchor SMF to instruct the I-UPF for a QoS flow to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet?

More generally, could SA2 clarify how transport level marking should be performed by I-UPF?

This discussion paper lists all the available options based on companies’ inputs in SA2#168 meeting and propose a way to address how the I-SMF/I-UPF behave regarding the Transport Level Marking.

2. Discussion

2.1 Review of the progress made during the SA2#168 meeting

During the discussion in the SA2#168 meeting, certain consensus aspects have been concluded as below:

Consensus 1: As of Rel-18, the I-UPF does not check PDU Set Importance (PSI) values.

Based on Consensus 1, the I-UPF would not be able to apply any PSI-based DSCP marking even it was provisioned with a PSI-DSCP mapping list, and hence the I-SMF does not provide a PSI-DSCP mapping list to the I-UPF.

Consensus 2: DSCP value marking at I-UPF can be either based on

- I-SMF provided configuration over N4 as a specific DSCP value or

- Received Transport Level Marking value from the incoming N9 packets and set that same value for the outgoing N3 packets.

In legacy releases, Transport Level Marking is supported on a per QoS flow basis, as specified in the TS 23.501 [1], i.e., for each QoS flow, a transport level packet marking value is determined by the SMF and provided to the UPF, based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level. In Release 19, as part of XRM_Ph2 work, a finer transport level packet marking is introduced for the QoS Flows that are configured for PDU set QoS handling, i.e., the SMF may additionally consider the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a mapping of transport level packet marking values (DSCP) with PSIs for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values.

Since the PSI value is only readable by the PSA-UPF, to ensure the consistency of the transport level packet marking values along the transport network when I-UPF is inserted, it is expected that the I-UPF does not apply new transport level packet marking value(s), and simply uses/follows the DSCP value(s) set by the PSA-UPF, which is received at N9, for the outgoing N3 packets.

2.2 I-SMF/I-UPF behaviour analysis for general case

I-SMF/I-UPF insertion in the PDU session is neither limited to the XRM case nor to downlink. In legacy Transport Level Marking at I-UPF when not considering the XRM traffic, the I-UPF can always choose to 1) either apply the new transport level packet marking value based on the provided configuration by the I-SMF via N4, or 2) keep consistent DSCP marking based on received value, i.e., marking of the outgoing N3 packets based on the value received at incoming N9 packets. The latter is dictated by the fact that “Type of Service” and “Traffic Class” are mandatory IP header fields and cannot be empty/absent. Thus, an I-UPF shall always set some value to the corresponding fields, even nothing is received over N4 from I-SMF, and the DSCP value is accordingly always set by the I-UPF.

Observation 1: Transport Level Packet Marking by I-SMF/I-UPF is not limited to XRM traffic only but applies to all traffic types and for both uplink and downlink.

Therefore, the problem under consideration is not XRM-specific, and a general solution that applies to all the traffic types and scenarios should be derived.

Proposal 1: A general solution should be adopted on how the I-SMF/I-UPF apply the Transport Level Packet Marking capturing both existing cases of consensus 2 (i.e., I-SMF controlled new Transport level marking value or I-UPF marking the outgoing transport marking value based on the incoming marking value).

In our view, the I-UPF transport level marking behaviour is controlled by the I-SMF configuration. Identically to the anchor SMF when determining to apply transport level packet marking (based on its own decision, without any outside trigger or indication), I-SMF can determine whether to configure the I-UPF to apply a specific transport level packet marking value (DSCP value), or not (allowing UPF to mark based on the DSCP value of the incoming packets).

Proposal 2: I-SMF can determine on how the I-UPF should perform the transport level packet marking, based on its local configuration.

Legacy behaviour, as specified in TS 29.244 [2], dictates that the IE Transport Level Marking is included in the FAR sent from SMF to UPF. Presence of this IE is Conditional and is provided by the SMF to configure the UP function to mark the IP header with the SMF-provided DSCP marking. This logic is appropriate since the PSA-UPF is the first network entity at the user plane on N6 and N9 interface.

For the I-SMF and I-UPF case, an implicit approach is assumed. That is, if Transport Level Marking IE is present in the FAR, then I-UPF uses the value provided by the I-SMF to mark the outgoing packets. Otherwise (Transport Level Marking IE is not present in the FAR), I-UPF marks the outgoing packets based on the received DSCP value in the incoming N9 packets. This approach ensures backwards compatibility, as there is no need for an explicit new indication between the I-SMF and the I-UPF.

Figure 1 TS 29.244 excerpt

Proposal 3: Clarify with a Note in specs that the UPF applies the DSCP value marking of the outgoing packets based on the incoming packets, if no Transport Level Marking is included in the N4 rule from SMF.

2.3 I-SMF and I-UPF behaviour analysis for XRM traffic

When it comes to XRM traffic, different options have been proposed to address the Transport Level Marking at I-UPF from different companies during the SA2#168 meeting, which are listed in the Table 2.3-1 below:

Table 2.3-1 Options for Triggering PSI-based Transport Level Marking at I-UPF

| Option | N16a | N4 | I-UPF Behaviour |
| --- | --- | --- | --- |
| Option 1 | A new Indication or explicit information saying that the PSI-DSCP based mapping is applied for the QoS flow, from Anchor SMF to I-SMF | New indication+Transport Level marking (legacy in FAR) together | - If new indication and Transport Level Marking are both included, then I-UPF applies the first 3 bits of DSCP according to the Transport Level marking value received via N4, and the last 3 bits according to the in-coming N9 packets. |
| Option 2 | Same as Option 1 | A new indication overriding any Transport Level marking (legacy in FAR) FFS whether the indication is used with Transport Level Marking Value together or not | - If new indication is included in N4 and received at I-UPF, then I-UPF shall take the DSCP value received in the outer header of the incoming N9 downlink packets and set that value as the DSCP of the outer header of the outgoing N3 packets. - If no new indication is included in N4 and received at I-UPF, then I-UPF acts according to the I-SMF configuration, e.g., applies the DSCP as provided by I-SMF. |
| Option 3 | Nothing new | Lack of legacy Transport Level Marking value is used implicitly as trigger for I-UPF to use incoming DSCP value from N9 packets | - If Transport Level Marking value is included in N4 and received at I-UPF, then I-UPF applies it accordingly. - If no Transport Level Marking is included in N4, then I-UPF shall take the DSCP value received in the outer header of the incoming N9 downlink packets and set accordingly the value as the DSCP of the outer header of the outgoing N3 packets. |
| Option 4 | Indication that anchor SMF/UPF applies PSI-based DSCP marking | If PSI-based DSCP marking indication received, the I-SMF shall not set any legacy Transport Level Marking value. | - Same as Option 3 |

Options 1 and 2 differentiate the case that XRM traffic is being transmitted on the user plane, and PSI-based DSCP marking is applied, i.e., the I-UPF will apply the outgoing N9 packets DSCP marking according to the N3 incoming packets marking value only when it is XRM traffic. However, there are three potential drawbacks for Option 1 and Option 2:

Drawback 1: The insertion of I-SMF and I-UPF shall not be limited to XRM traffic, and for legacy case, there can also be the case at I-UPF to keep the DSCP value marking consistency, i.e., directly use the value from N9 to the N3.

Drawback 2: For the XRM case, there is no need for a specific indication transmitted from anchor SMF to the I-SMF to notify that the QoS Flows are for XRM traffic. The reason is that the existing mechanism has already supported the I-SMF to know that the current QoS Flows are related to PDU set based handling or related to the XRM traffic, e.g., by receiving the PDU set QoS parameters, or the DL PDU Set Information Marking Support Indication from the anchor SMF, or receiving the PDU Set based Handling Support Indication from the NG-RAN via AMF.

Take PDU set QoS parameters as an example, as specified in the clause 4.3.2.2 of TS 23.502 [3]:

Step 13: H-SMF to V-SMF: Nsmf_PDUSession_Create Response (QoS Rule(s), QoS Flow level QoS parameters if needed for the QoS Flow(s) associated with the QoS rule(s), PCO including session level information that the V-SMF is not expected to understand, selected PDU Session Type and SSC mode, Reliable Data Service Support, H-CN Tunnel Info, QFI(s), QoS profile(s), Session-AMBR, Reflective QoS Timer (if available), information needed by V-SMF in the case of EPS interworking such as the PDN Connection Type, User Plane Policy Enforcement, [ECS Address Configuration Information for the serving PLMN]).

The H-SMF to V-SMF in step 13 can be replaced by anchor SMF and I-SMF, based on the below statement specified in TS 23.502 clause 4.23.5.1:

If the service area of the selected SMF does not include the current UE location and the UE does not request for a MA PDU Session, the AMF selects an I-SMF that serves the area where UE camps. The UE requested PDU Session Establishment procedure for Home-routed Roaming defined in clause 4.3.2.2.2 is used to establish the PDU Session. Compared to the procedure defined in clause 4.3.2.2.2, the V-SMF and V-UPF are replaced by I-SMF and I-UPF and H-SMF and H-UPF are replaced by SMF and UPF(PSA) respectively.

Therefore, the PDU set QoS parameters can be received by the I-SMF from the anchor SMF to indicate the that the QoS Flows are related to PDU set based handling. Apart from the PDU set QoS parameters received at I-SMF from anchor SMF, I-SMF can also know that the QoS Flows are related to PDU set based handling via 1) a DL PDU Set Information Marking Support Indication (if no PDU set QoS parameters), or 2) PDU set based handling support indication from NG-RAN (from NG-RAN to AMF and then I-SMF).

Observation 2: The I-SMF can be aware of the QoS Flows that are related to PDU set based handling or XRM traffic based on the existing mechanism.

Drawback 3: For XRM traffic, the need for providing the N4 indication that the PSI-based transport level packet marking is applied for the affected QoS Flow (As per S2-2504229), is not well justified. As long as the I-SMF knows that the QoS Flows are related to PDU set based handling or XRM traffic (as we listed above in Observation 2), it will not include the Transport Level Marking in the FAR sent to I-UPF. Then, I-UPF will always apply the “copy-paste” of the DSCP value from the incoming N9 packets to the outgoing N3 packets, either it is a legacy DSCP value on a per QoS Flow basis, or the PSI based DSCP value. The I-UPF behaviour logic of keeping the DSCP marking consistent across N3/N9 should be irrelevant to whether the PDU set based handling, and the PSI-DSCP mapping is applied for the QoS flow or not.

Proposal 4: For XRM traffic, there is no need for the I-SMF to be explicitly informed by the anchor SMF on whether PSI-DSCP mapping has been applied. The I-SMF knows the QoS Flows which are related to XRM traffic (i.e., PDU set based Handling), and shall not include the Transport Level Marking in the N4 sent to the I-UPF; then, the I-UPF will take the DSCP value from the incoming N9 packets and apply it into the outgoing N3 packets. Therefore, option 3 is preferred.

Alternatively, if the I-SMF needs to be able to identify whether PSI-based DSCP marking is applied by the anchor SMF/UPF, a simple indication from the anchor SMF would suffice as per Option 4.

3. Summary

Consensus 1: As of Rel-18, the I-UPF does not check PDU Set Importance (PSI) values.

Consensus 2: DSCP value marking at I-UPF can be either based on

- I-SMF provided configuration over N4 as a specific DSCP value or

- Received Transport Level Marking value from the incoming N9 packets and set that same value for the outgoing N3 packets.

Observation 1: Transport Level Packet Marking by I-SMF/I-UPF is not limited to XRM traffic only but applies to all traffic types and both uplink and downlink.

Proposal 1: A general solution should be adopted on how the I-SMF/I-UPF apply the Transport Level Packet Marking capturing both existing cases of consensus 2 (i.e., I-SMF controlled new Transport level marking value or I-UPF marking the outgoing transport marking value based on the incoming marking value).

Proposal 2: I-SMF can determine on how the I-UPF should perform the transport level packet marking, based on its local configuration.

Proposal 3: Clarify with a Note in specs that the I-UPF applies the DSCP value marking of the outgoing packets based on the incoming packets, if no Transport Level Marking is included in the N4 rule from I-SMF.

Observation 2: The I-SMF can be aware of the QoS Flows that are related to PDU set based handling or XRM traffic based on the existing mechanism.

Proposal 4: For XRM traffic, there is no need for the I-SMF to be explicitly informed by the anchor SMF on whether PSI-DSCP mapping has been applied. The I-SMF knows the QoS Flows which are related to XRM traffic (i.e., PDU set based Handling), and shall not include the Transport Level Marking in the N4 sent to the I-UPF; then, the I-UPF will take the DSCP value from the incoming N9 packets and apply it into the outgoing N3 packets. Therefore, option 3 is preferred.

Proposal 5: It is proposed to agree the corresponding 23.501 CR (S2-2505147) and the LS reply (S2-2505145).

4. Reference

[1] 3GPP TS 23.501.

[2] 3GPP TS 29.244.

[3] 3GPP TS 23.502.