SA WG2 Meeting #169	S2-2505170

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-25xxxxx)

Source:	ZTE, Nokia, CATT, China Mobile

Title:	pCR to TS 23.369: Update Protocol Stacks

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper updates AIoT protocol stacks, and clarifies that the NGAP protocol is used to transport the AIoT Reader Control messages and removes related ENs.

1. Discussion

In clause 4.6 of TS 23.369, there are following ENs regarding protocol stack and AIoT message transportation:

Editor's note:	The protocol stacks between RAN and 5GC in the following clauses need to align with RAN specifications.

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Based on the LS from RAN3 (S2-2504523/R3-252481), RAN3 has reached the following working agreement regarding how to transport AIOTF information.

- Previous working assumption: “Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP” is changed to an agreement.

- Existing NGAP Interface management procedures will be enhanced to support A-IoT.

Based on RAN3 agreement, it is proposed to remove the ENs and add the description that the AIoT messages will be included in the NGAP protocol.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

4.6.1	General

This clause specifies the protocol stacks between entities used for AIoT. The protocol stacks include the following:

-	Protocol stacks between AIoT Device and AFs, including the protocol stacks among AIoT Device, NG-RAN, AMF, AIOTF, NEF and AFs.

4.6.2	Protocol Stack between AIoT Device and AF

4.6.2.1	General

The general protocol stack between an AIoT Device, NG-RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN is specified in TS 38.300 [5].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.2.1-1: Protocol Stack Between AIoT Device and AF

The NG-RAN may communicate to AIOTF via different connectivities, see clause 4.2.2.1. The NG-RAN protocol stack remains the same and is transparent to the different connectivities. The protocol stacks and routing of information between an NG-RAN and the AIOTF is defined in other clauses depending upon the architecture used:

-	Direct Connectivity: When the NG-RAN communicates with AIOTF directly, the protocol stack is specified in clause 4.6.2.2.

-	Indirect Connectivity: When the NG-RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

The AIoT NAS protocol supports the inventory response and command related signalling.

4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Connectivity option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN transparently.

The AIoT messages between the NG-RAN and the AIOTF are included in a container within the NGAP protocol.

4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN and AMF transparently.

The AIoT messages between the NG-RAN and the AIOTF are included in a container within the NGAP protocol between the NG-RAN and the AMF, and transferred through AIOTF3 reference point between the AMF and the AIOTF.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

* * * * End of Changes * * * *