SA WG2 Meeting #169	S2-2505236

May 19 – 23, 2025, Fukuoka, Japan

# Title:	Discussion on OAM based solution to support NTN regenerative payload

# Source:	CATT, CSCN, ZTE, vivo

# Document for:	Discussion

Agenda Item:	19.1.2

Abstract:	This paper presents a discussion on OAM based solution to support NTN regenerative payload related to LS R3-250895 from RAN3 and S5-250842 from SA5.

- Background and benefits of OAM based solution for NTN regenerative payload

In the scenarios of regenerative payload for satellite access (i.e. the gNB is onboard the satellite), the Supported TAI list of the gNB will change as the gNB moves along with the NGSO satellite. This would cause some problems e.g. for paging, as the AMF may not be able to tell which gNB on satellite is serving the UE when the downlink data/ signalling is to be delivered.

RAN WG3 has discussed this issue for several meetings and agreed on an OAM based solution as briefly described in the LS R3-245845/S2-2411306, which is further confirmed and clarified in the LS R3-250895/S2-2504578, as a response to S2-2413030/R3-250023 and S5-247311/R3-250030.

The following clarifications are provided in LS R3-250895/S2-2504578:

“	-	In general, satellite orbital movement is periodic and predictable; therefore, any resulting change to node parameters will result in the same information being frequently exchanged over network interfaces at regular intervals.

- In the solution agreed by RAN3, both AMF and gNB share the same OAM configuration on supported TAI list of the on-board gNB, this is considered as an optimization.

- NG Setup procedure including the supported TAs is always expected to take place according to TS38.413.

- The solution agreed by RAN3 avoids the need for periodic and frequent RAN Configuration Update procedures. Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation.

- With respect to the frequency of updates by OAM, RAN3 believes this is in SA5 scope.”

The reply LS to S2-2413030/R3-250023 from SA5, i.e. S5-250842/S2-2504580, provides further clarifications on the frequency of updates by OAM, i.e. how the configuration of supported TAI list of the gNB is done by OAM while avoiding frequent configuration update as the gNB/satellite moves, as follows:

“AMF is pre-configured with multiple subsequent time windows instances within which the NTN gNB ID and supported TAI list are specified. In other words, the pre-configured ‘timetable’ indicates which TAIs the gNB will support at what time and avoids the OAM configuration for each time as the satellite moves.”

One thing needs to be pointed out is that the OAM solution is optional (“as an optimization”), i.e. the operator can choose to use it or not. And the gNB onboard can initiate NG Setup and RAN Configuration Update procedures to update the supported TAI list as per existing spec in TS 38.413. But:

1)	There is no description in TS 38.413 that the gNB shall/should/may initiate RAN Configuration Update procedures whenever the supported TAI list changes as the gNB/satellite moves.

2)	There is no agreement reached in RAN3 to enhance the existing NG Setup and RAN Configuration Update message, e.g. to add timetable information in the Supported TAI list IE.

Thus, the benefits of the OAM solution agreed in RAN3 and SA5 are not only “avoids the need for periodic and frequent RAN Configuration Update procedures” as stated in the LS, but mainly, improving the paging performance (e.g. shorten the paging time) and lowering the possibility of paging failure (e.g. UE unreachable due to prolonged paging time) in the scenarios of NTN regenerative payload. Considering that satellite communication is often used in remote/wild area and for emergency communication, the improved paging performance and reduced possibility of paging failure can make a difference.

- Clarifications to some questions on OAM based solution

In SA2#168 meeting, some questions are raised in S2-2503231 on OAM based solution. So clarifications are provided as below to address these questions.

1) “Frequency of updates by OAM is not clarified and therefore the signalling optimisation is unproven”

Frequency of updates by OAM is clarified in S5-250842/S2-2504580 as described above.

Regarding the signalling optimisation, according to ESA - Low Earth orbit, a LEO satellite can move at a speed of around 7.8km per second and take approximately 90 minutes to circle Earth. Though the speed and period of different LEO satellites may vary due to different orbit altitudes, the frequency for updating the supported TAI list via NGAP signalling can be high (e.g. 1 ~ several seconds), especially when the planned TA sizes are small. A detailed analysis on the signalling optimisation can be found in S2-2503268. Also, as clarified above, the main benefits of the OAM solution agreed in RAN3 and SA5 is to improve the paging performance and lower the possibility of paging failure (given that the NGAP signalling enhancement is not agreed in RAN3), thereby meeting the requirements of many (if not all) operators.

2) “Coordination across the signalling based and OAM solutions”

The gNB needs not to know whether the AMF is “correctly configured” via OAM. Based on OAM configuration and/or implementation of gNB (e.g. configuring a timetable of Supported TAI list, or determining the Supported TAIs based on the ephemeris and not performing RAN configuration update due to satellite movement), the gNB can decide not to initiate the RAN Configuration Update procedure when it crosses the TA geographic boundaries. And if the gNB initiates the RAN Configuration Update procedure and the Supported TAI list is indicated, the AMF can simply ignore the Supported TAI list based on OAM configuration, which is up to implementation. OAM configuration should ensure the alignment of the supported TAI list configurations in the onboard gNB and the AMF.

3) “Whether the requirements violate “plug n’ play” principles”

The gNB onboard is configured with the timetable of supported TAI list according to its ephemeris, which can be done before being connected to the network. The configuration of the AMF can be updated before the gNB is connected to the network or put into use. On the hand, current AMF configuration already contains some information which needs to be updated before/when a new gNB for satellite access is mounted, e.g. mappedCellIdInfoList and satelliteCoverageInfoList, see TS 28.541.

In multi-vendor case, i.e. gNB and AMF are provided by different vendors, the PLMN/network level OAM controls/coordinates the Domain level or Network Element level OAM to ensure that the AMF and gNB share the same OAM configuration. Figure 5.2-1 of TS 28.104 gives an example of such a coordination, though it is for the MDA MnS producer.

RAN domain MDA MnS producer

MDA MnS

3GPP cross-domain MDA MnS consumer

3GPP cross-domain MDA MnS producer (domain MDA MnS consumer)

CN domain MDA MnS producer

gNB

MDA MnS

MDA MnS

NWDAF

Other 5GC NF

Nnf

Nnwdaf

MDA MnS

MDA MnS

MnS

MnS

Nwdaf

RAN domain

CN domain

MnS

MDA MnS

Figure 5.2-1 of TS 28.104

In roaming cases, the gNB and AMF belong to the same serving PLMN and the configuration can be aligned as above.

4) “Whether the requirements change functional split between RAN and CN”

Currently the CN has some knowledge of geographic area covered by the gNB based on the TAIs/(mapped) Cell IDs reported by the gNB, to facilitate e.g. paging. But if paging based on mapping between the ULI and Cell IDs/TAIs supported by the gNB is not considered as having knowledge of geographic area, here the way of OAM configuration is actually the same, as it is just adding time information for the supported TAIs by the gNB.

- Conclusions

Base on the above discussion, the following conclusions on OAM based solution to support NTN regenerative payload are proposed:

-	The OAM based solution agreed in RAN3 and SA5 can achieve the goal of informing AMF of the Supported TAI list of the gNB onboard the satellite, thus improving paging performance while avoiding periodic and frequent RAN Configuration Update procedures between the gNB onboard and the AMF.

-	The specification in SA2 should be updated to align with the OAM based solution agreed in RAN3 and SA5, as proposed in the relevant CR(s).