3GPP TSG-SA WG2 Meeting #169	S2-2505282

Fukuoka, Japan, May 19 – 23, 2025

Source:	CATT

Title:	KI#1#2 new solution to improve energy consumption calculation and energy saving control

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#1 and KI#2 to improve accuracy of energy consumption calculation and effectiveness of energy saving control.

1. Introduction

A solution is proposed on how to enhance the determination of energy related information in the 5GS to improve the accuracy and effectiveness of energy saving in the network, to address KI#1 and also related to KI#2.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

6.x	Solution #X: Enhancements on energy consumption calculation and energy saving control

6.x.0	High-level solution principles

This solution proposes to modify the Energy Consumption (EC) calculation formulas, and that based on the statistics of calculated EC, the network sets an EC threshold per required granularity for taking actions (e.g. policy control) for energy saving. The EC calculation formula parameters and the EC threshold value per required granularity can be adjusted based on the results of energy saving actions taken by the network.

6.x.1	Description

This solution addresses Key Issue #1 and #2.

In Annex T of TS 23.501 [2], examples on how EIF calculates the energy consumption for required granularities are provided, wherein the energy consumption over a time interval T at a UPF (EUPF) or a gNB (EgNB) is used to deduce the energy consumption per specific UE, S-NSSAI, PDU Session or QoS flow.

However, considerable energy is consumed even when there is no UE traffic transmitted in the node, as well as for control plane signalling processing when there is UE traffic transmitted. To take these into account and make the calculation of energy consumption for the above granularities more accurate, it is proposed to modify the formulas, i.e. replacing the node-level energy consumption EgNB and EUPF in the formulas by E'gNB and E'UPF as follows:

E'gNB=w11*EgNB-w12*EgNB,0

E'UPF=w21*EUPF-w22*EUPF,0

Where:

- EgNB is the node-level energy consumption of gNB when there is UE traffic transmitted via the gNB;

- EgNB,0 is the node-level energy consumption of gNB when there is no UE traffic transmitted via the gNB;

- EUPF is the node-level energy consumption of UPF when there is UE traffic transmitted via the UPF;

- EUPF,0 is the node-level energy consumption of UPF when there is no UE traffic transmitted via the UPF;

- w11, w12, w21 and w22 are weight factors and their values are in the range of [0, 1]. w11 and w21 are set taking into account of energy consumption for control plane signalling processing in the node.

NOTE:	The values of w11, w12, w21 and  w22 are up to implementation and operator policy.

Based on the statistics of calculated EC per required granularity (e.g. EC per UE calculated by the EIF) for the AoI and/or target time period (e.g. 8 AM -10 AM everyday) in the past, the operator can set an EC threshold per required granularity where:

-	the threshold value is set considering the statistical distribution of EC per required granularity in the AoI and/or target time period, e.g. when the required granularity is per UE, for 95% of the UEs in the AoI and/or target time period, each UE consumes less network energy than this value; or,

-	the threshold value is calculated based on the average and variance of EC per required granularity in the AoI and/or target time period.

The EC threshold per required granularity can be configured in the network (e.g. PCF). Then based on the EC threshold per required granularity and the notifications from the EIF, the PCF can decide whether or not to take actions for energy saving control per required granularity, e.g. performing policy control on the UEs of which the energy consumption goes beyond the EC threshold per UE in the AoI and/or target time period.

After actions are taken to save network energy consumption, the EIF collects energy related information and calculates the total EC for the AoI and/or target time period, and compares the EC with that before the actions are taken.

Based on the EC comparison result, the EC calculation formulas can be optimized, e.g. by adjusting the values of w11, w12, w21 and/or w12 in the formulas, or the EC threshold value per required granularity can be modified, so that the EC calculation per required granularity and determination of energy saving control can be more accurate and effective.

6.x.2	Procedures

Figure 6.x.2-1: Policy control based on notifications from the EIF

1.	The PCF subscribe to notifications of EC per required granularity for the AoI and/or target time period from the EIF.

2.	The EIF collects energy related information and calculates EC per required granularity using the formula as described in clause 6.x.1.

3.	The EIF sends notifications of EC per required granularity to the PCF.

4.	Based on the EC threshold per required granularity and the notifications from the EIF, the PCF decides to perform policy control for energy saving. The EC threshold value is configured in the PCF and can be set based on the statistics of calculated EC per required granularity.

5.	The PCF may send a notification to the EIF indicating that actions are taken based on the EC information.

6.	The EIF continues to collect energy related information and calculates the total EC for the AoI and/or target time period, and compares the EC as triggered by the notification from the PCF in step 5 or based on configuration. Based on the comparison result, the EC calculation may be modified, e.g. by adjusting the parameter values of w11, w12, w21 and/or w12.

7.	The EC threshold value configured in the PCF can be updated based on the statistics of newly calculated EC per required granularity.

6.x.3	Impacts on services, entities and interfaces

EIF:

-	Calculates energy consumption per required granularity using new formula and adjusts the formula parameters based on the comparison of the EC information before and after network energy saving actions are taken.

-	Sends energy related notifications to the PCF.

PCF:

-	Subscribes to notifications of EC per required granularity for the AoI and/or target time period from the EIF.

-	Performs policy control based on the configured EC threshold per required granularity and the notifications from the EIF. The EC threshold value can be set based on the statistics of calculated EC per required granularity.

* * * End of Change * * * *