SA WG2 Meeting #169	S2-2505313

Fukuoka, Japan, 19-23 May, 2025

# Title:	Correction on device filtering information

# Source:	InterDigital Inc.

# Document for:	Approval

Agenda Item:	19.14.2

Abstract:	The pCR proposes a correction to remove the ambiguity of device filtering information matching process.

1. Discussion

In “Caluse 5.8 Filtering Information”, there is the following NOTE:

NOTE:	The bitstring match can start at any position in the corresponding component

In our understanding, it is intended to allow a variety of  matching cases illustrated in the figures below.

Case 1: full match

Case 2: partial match at the beginning of the component

Case 3: partial match in the middle of the component

Case 4: partial match at the end of the component

However, the NOTE can be misinterpreted as only part of the bitstring needs to be matched, as illustrated in figures below. And we believe this case should not be considered as a match.

To remove this ambiguity, one (better) solution is to always align the beginning of the component with the beginning of the bitstring when the matching is performed. This will also reduce the computational complexity in the device.

If matching at any starting point in the component is still desired, at least it should be clarified that the full bitstring needs to be matched. The proposed change in this pCR follows this approach.

2. Proposal

It is proposed to adopt the proposed change in TS 23.369.

* * * * First Change (All new text) * * * *

## 5.8	Filtering Information

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier as defined in clause 5.7.

The filtering information includes a list of filtering element information. The filtering element information includes:

-	Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A corresponding bitstring which is used to compare with the indicated component. The length of the bitstring should be equal or less than the length of the indicated component.

NOTE:	The bitstring match can start at any position in the corresponding component but the full bitstring should be matched.

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the bitstring in a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

* * * * End of Change * * * *