3GPP TSG-SA WG2 Meeting #169	S2-2505590

# Fukuoka, Japan, May 19 – 23, 2025	(revision of S2-2504788)

Source: 	vivo

Title: 	KI #3, New Sol: AMF (re-)selection based on energy related information

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys / Rel-20

Abstract of the contribution: This contribution is trying to provide a sol for KI#X on the AMF selection aspect.

- Introduction

This solution applies to WT#3 on the AMF selection aspects based on energy related information.

SP-250417 … WT#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information

This solution includes the following sub-aspects:

- Registration with AMF re-allocation considering energy saving

2	Proposal

It is proposed to capture the following solution in TR 23.700-67.

First change (all new text)

6.x	Solution #X: AMF (re-)selection based on energy related information

6.x.0	High-level solution principles

This solution proposes to take energy related information into account for AMF (re-)selection.

6.x.1	Description

This is a solution related to KI#3.

This solution leverages some existing procedure for AMF selection/reselection based on energy related information including following parts:

-	When the initial AMF receives the UE initial message or uplink NAS transport message as in clause 4.2.2.2.3 of TS 23.502[3], the initial AMF may need to re-route the message to another AMF according to the energy related information in its NF profile, for example, according to the Schedule of EnergySaving state, the initial AMF knows it is about to enter into EnergySaving state as defined in TS 28.310[b], then the initial AMF may trigger the AMF re-allocation to re-route the UE message to a suitable Target AMF.

-	During the AMF re-allocation, AMF reuse the NF discovery procedure as in TS 23.502[3] and takes the energy related information in the NF profile from the NRF into account for the Target AMF selection. Optionally, the initial AMF may indicate the NRF the energy requirement parameters for Target AMF discovery, then the NRF considers it for target AMF querying and feedback

6.X.2	Procedures

6.X.2.1	AMF re-allocation considering energy saving

Figure 6.x.2.1-1: AMF re-allocation procedure considering energy related information

1.	Initial AMF receives a UE initial message or Uplink NAS transport message.

2.	If the initial AMF detects that it is not satisfied to serve the UE based on the energy related information, e.g., the initial AMF knows it is about to enter into EnergySaving state as defined in TS 28.310[b] soon, the initial AMF triggers the AMF re-allocation as in the subsequent steps.

3.	The initial AMF queries from the NRF for Target AMF discovery as in clause 4.17.4 of TS 23.502[3], AMF takes the energy related information in the NF profile from the NRF into account for the Target AMF selection.

Optionally, during the querying, the initial AMF may indicate the NRF that energy requirement parameters for Target AMF querying/discovery is needed, the NRF considers it for target AMF discovery and feedback.

4.	Initial AMF reroutes the UE registration request or Uplink NAS transport message to the Target AMF by Namf_Communication_N1MessageNotify message directly, or sends a NGAP Reroute NAS Request message with Target AMF info to the (R)AN, the (R)AN reroutes the UE initial NAS message to Target AMF as in clause 4.2.2.2.3 of TS23.502[3].

6.X.3	Impacts on existing services, entities and interfaces

AMF:

-	AMF may redirect UE initial NAS message by considering the energy related information.

-	AMF may indicate the NRF to consider that energy saving or efficiency requirement for Target AMF querying is needed.

NRF:

-	takes the energy related information in NF profile into account for Target AMF querying.

-	when receives the indication that energy saving or efficiency for Target AMF query is needed from initial AMF, the NRF considers it for target AMF discovery and feedback.

Second change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository Service for Policy Data, Application Data and Structured Data for Exposure; Stage 3".

[b]	3GPP TS 28.310 " Energy efficiency of 5G ".

End of Changes