3GPP TSG-WG SA2 Meeting #169	S2-2505602

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Improve AIoT NF Services

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: address the open issues to support Ambient IoT services on the NFs

1. Introduction/Discussion

1.1	Issues for Number of Service Operation

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

Editor’s note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

There are two ways to design the AIoT Command service operations, either 1) Single Command service operation + Command Type service parameter, or 2) Three separate Read, Write or Disable command service operation.

Both ways work technically. However, way 2) shows benefits in specification documentation, it enables a more structural and clear way to show the differences (e.g., Command specific parameters, Information to be used for resource allocation) between the Read, Write or Disable command service operations.

If way 1) continues, then it is suggested to have a more structural and clear documentation manner to show the differences for different command operations.

1.2	Issues on AMF transport service to support BS Reader information reporting

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect path.

Editor's note:	AIoT messages is FFS and coordination with RAN3 is needed.

Editor’s note:	Further functions on AMF is FFS.

RAN3 would like to inform SA2 and SA5 about the following RAN3 progress related to Architecture and OAM aspects:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information...

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

- The gNB always sends the A-IoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

- “Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

LS from RAN3 (R3-250905) states that

RAN3 would like to inform SA2 and SA5 about the following RAN3 further progress:

General Aspects:

- Previous working assumption: “Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP” is changed to an agreement.

- Existing NGAP Interface management procedures will be enhanced to support A-IoT.

- RAN3 agreed to define a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. The detailed coding of the new “A-IoT Area” is FFS.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

On Inventory:

- Within all Inventory related NGAP messages:

  - The Correlation Identifier is included inside the AIOTF information container.

  - The AIOTF Identifier is included outside the AIOTF information container.

  - Working assumption: The Correlation identifier is also included outside the AIOTF information container.

On Command after Inventory:

- Introduce a “follow-on command indication” in the AIOTF information container in the Inventory Request message.

- Working assumption: The NGAP Command Request procedure concerns a single device only.

- Within all Command related NGAP messages:

  - The AIOTF Identifier is included outside the AIOTF information container.

  - Working assumption: The Correlation identifier is included inside and outside the AIOTF information container.

- RAN3 agreed to introduce per-session per-device device context (“RAN NGAP Device ID”).

  - Upon receiving the “follow-on command indication”, the gNB allocates for each device a “RAN NGAP Device ID” and provides it in the AIOTF information container in the Inventory Report message.

  - The “RAN NGAP Device ID” is included in the AIOTF information container of each Command related NGAP message.

LS from RAN3 (R3-252481) further states that:

Observation 1: Container in NGAP is used to transport any AIOTF information. In case of Indirect connectivity between AIOTF and AIoT RAN via an AMF, the AMF performs transparent relay of the AIOTF information in the NGAP Container between AIOTF and AIoT RAN, thus the AIOTF information transferred in the NGAP Container is transparent to the AMF, i.e., the AMF will not interpret any AIOTF information in the NGAP Container.

In case of Indirect connectivity between AIOTF and AIoT RAN via an AMF, NGAP over the N2 reference point between the AIOT RAN and AMF supports Ambient IoT services including delivery of inventory/command messages. Thus, AIOTF information in the NGAP Container includes at least inventory/command messages, which RAN3 specifies the following Container IE:

-	Inventory Request Transfer, Inventory Response Transfer, Inventory Failure Transfer, Command Request Transfer, Command Response Transfer, Command Failure Transfer

-	Inventory Report Transfer

According to RAN3 LS, AIOTF can obtain the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration. RAN3 signalling for the A-IoT RAN information is FFS. However, even though RAN3 defines the signalling to transfer A-IoT RAN information from AIoT RAN to AIOTF, this A-IoT RAN information will be transported in the NGAP Container, thus to align with RAN3 agreement.

Observation 2: Even though RAN3 defines the signalling to transfer A-IoT RAN information from AIoT RAN to AIOTF, this A-IoT RAN information will be transported in the NGAP Container.

Proposal 1: AMF is enhanced to perform transparent relay of any AIOTF information in the NGAP Container between AIOTF and AIoT RAN. AIOTF information includes Container IE defined for inventory/command messages. Whether to support relaying of A-IoT RAN information needs RAN3 progress. If supported, SA2 can align with RAN3, this EN can be removed now.

Currently, the message transfer service for N1N2Message and NonUEN2Message doesn’t include the Container in the message transfer response, instead, the container is included in the message transfer notification.

Proposal 2: Move the Container IE from message delivery response service operation to message notify service operation.

1.3	Issues on notify service operation

Editor’s note:	It is FFS whether that fail to discovery any AIoT device is normal operation result or failure case.

Failure case indicates that the inventory or command procedure cannot be executed normally due to unexpected error, e.g., resource allocation failure. If the procedure completes normally but there is no matched target AIoT device, this should be a normal operation result.

Editor’s note:	It is FFS how to make AF be aware of the report is finished in case there are multiple notify for the same AIoT operation.

Editor's note:	It is FFS how to make AIOTF be aware of the report is finished in case there are multiple notify for the same operation.

When the AIoT RAN detects the completion of the procedure e.g., that there is no further Ambient IoT Device performing random access, collected AIoT devices has exceeded the expected number of AIoT devices, the allocated resources have used up etc., the AIoT RAN can inform the AIOTF of the completion indication of the procedure. This completion indication can be sent along with the last inventory report or sent using a dedicated message signalling.

Meanwhile, the AIOTF can also detect the completion of the procedure, e.g., all target Ambient IoT Device(s) have been inventoried etc. In this case, the AIOTF can either inform the AIoT RAN of the completion indication or disregard the further reports from the AIoT RAN.

When the AIOTF knows the completion of the procedure, it can instruct to the consumers that the procedure is completed or this is the last notification.

Proposal: Notify service operation is used to carry the completion indication to indicate that the procedure is finished or to carry the last report indication.

1.4	Issues on AMF service operation

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

Editor's note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

TR conclusion 1: When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request.

The Correlation ID is used by AIOTF to correlate the results in one or more reports to a given operation request. This Correlation ID shall be part of the Inventory Report or Inventory Response or Command Response.

In LS from RAN3 (R3-252481), it also concludes that 1) The Correlation Identifier is included inside the AIOTF information container.

Proposal 1: No need to add Correlation ID as input parameters of the Namf_AIoT_ notify.

TR conclusion 2: UL traffic is routed based on an AIoT correlation identifier (i.e. AIOT-RAN identifies the appropriate AMF to which to forward UL messages base on the association between AIoT correction identifier and the AMF).

The Correlation ID is used by AIoT-RAN to correlate the NGAP association with AMF to route the UL traffic back. Meanwhile, inventory request, response, failure is paired class 1 NGAP messages. Same for command request, response and failure. If the NGAP association between AIoT-RAN and AMF is used to transfer NGAP Command messages for multiple devices in parallel, then the AMF and AIoT-RAN need to identify different paired NGAP command procedure messages. A device transaction ID is required for this purpose.

In LS from RAN3 (R3-252481), RAN3 agreed to introduce per-session per-device device context (“RAN NGAP Device ID”). The “RAN NGAP Device ID” is included in the AIOTF information container of each Command related NGAP message. Similarly, the “CN NGAP Device ID” is also needed from CN PoV. Then the RAN NGAP Device ID and the CN NGAP Device ID can be used to indicate the device transaction ID for a paired NGAP command message over the NGAP association between AIoT-RAN and AMF.

TR conclusion 3: The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

The AIOTF identifier is used by AMF to route the responses related to a request back to the AIOTF that triggers the request.

Same for LS from RAN3 (R3-252481), it indicates that 1) Working assumption: The NGAP Command Request procedure concerns a single device only. 2) The AIOTF Identifier is included outside the AIOTF information container. 3) Working assumption: The Correlation identifier is included inside and outside the AIOTF information container. 4) RAN3 agreed to introduce per-session per-device device context (“RAN NGAP Device ID”).

Proposal 2: add both the Correlation identifier and AIOTF identifier as required input parameter for Namf_AIoT_ Messagedelivery service operations.

Proposal 3: add device transaction identifier (“RAN NGAP Device ID” and “CN NGAP Device ID”) as required input parameter for Namf_AIoT_Messagedelivery service operations in case it used to transfer AIoT command messages.

1.5	Others

Introduce the general description for network function Ambient IoT services.

In Message Delivery AMF service operation, the AIOTF needs to indicate to the AMF the message type, so the AMF can construct the correct NGAP message header (Inventory or Command) sent to the AIoT RAN.

Remove the EN about AF authorization, as concluded in clause 5.6.

For NEF AIoT services, this needs to align with the AIOTF AIoT services.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

4.5.7	AMF

The AMF performs the following functions when the NG-RAN and the AIOTF communicate indirectly via an AMF:

-	Relaying signalling for AIoT service (including e.g., inventory/command messages) between NG-RAN and AIOTF transparently.

-	Providing transparent transport for AIoT NAS messages between AIoT Device and AIOTF.

# * * * * Second change * * * *

7	Network Functions Services

7.1	General

The following clauses provide for each involved NF the NF services it exposes through its service-based interfaces for AIoT Services.

# * * * * Third change * * * *

7.3	AMF services

7.3.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.3.1-1. The Namf_AIoT AMF service is used when the NG-RAN and the AIoTF communicate indirectly via an AMF.

Table 7.3.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data towards NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT Transfer Container (Inventory Request Transfer, Command Request Transfer) to deliver to NG-RAN.

2)	NG-RAN ID.

3) 	AIoT NGAP Message Type ("Inventory" or "Command").

4) 	AIOTF Identifier and Correlation Identifier, this is to allow identifying the association between NG-RAN and AMF.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT Transfer Container (Inventory Response Transfer, Inventory Failure Transfer, Command Response Transfer, Command Failure Transfer, Inventory Report Transfer) received from NG-RAN.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *